3GPP TSG-SA WG2 Meeting #143E 	S2-2101433

Elbonia, February 24 – March 09, 2021

Source:	Huawei, HiSilicon

Title:	Interworking with MBMS over E-UTRAN

Document for:	Agreement

Agenda Item: 	8.9.2

Work Item / Release:	5MBS /Rel-17

Abstract of the contribution: This contribution proposes to add description on Interworking with MBMS over E-UTRAN into the new TS.

- Discussion

This contribution proposes text in clause 6.8 based on the conclusion in TR 23.757.

- Proposal

It is proposed to add description on interworking with MBMS over E-UTRAN in clause 6.7 of the TS 23.247:

**************************** 1st Change ***************************

6.8 Interworking with MBMS over E-UTRAN for public safety services

In order to minimize the interruption of services, upon mobility from NR/5GC to E-UTRAN/EPC, the following applies:

-	If the same multicast service is provided via eMBMS in target E-UTRAN, the session context for multicast service transferring is not handover to E-UTRAN during mobility from 5GS to EPS, i.e. the EPS bearer context associated with the MBS session is not transferred to EPS network. UE releases the related EPS bearer(s) and the associated MBS session context locally. After handover, the UE is connected to the target E-UTRAN, the UE starts to receive the service via eMBMS.

-	If the same multicast service is not provided via eMBMS in target E-UTRAN, during handover from 5GS to EPS procedure, the 5GC shared MBS traffic delivery method is switched to Individual MBS traffic delivery over EPS. The unicast QoS flow(s) corresponding to the multicast QoS flow(s) of the MBS session are mapped to EPS bearer(s).

In order to minimize the interruption of services, upon mobility from E-UTRAN/EPC to NR/5GC, the following applies:

-	Before EPS to 5GS mobility, the application may trigger the switching the service receiving from eMBMS to Individual MBS traffic delivery over EPS. The AF provides the MBS Session ID (i.e. the TMGI or multicast IP address) as part of service information to PCF to trigger EPS bearer resource allocation for the service. Based on the received MBS Session ID, the SMF+PGW-C link the established EPS bearer(s) with the indicated MBS session.

-	If the UE receives the service via the Individual MBS traffic delivery over EPS, the Individual MBS traffic delivery over EPS is switched to 5GC Individual MBS traffic delivery method during handover from EPS to 5GS procedure. After handover, the SMF+PGW-C switches the 5GC Individual MBS traffic delivery method to 5GC shared MBS traffic delivery method if the target NG-RAN supports 5G MBS.

-	If the UE receives the service via eMBMS in source E-UTRAN, after handover from EPS to 5GS, the UE may join the 5MBS Session directly without reporting the UE is out of eMBMS service to AF.

********************************* End Change ****************************