SA WG2 Meeting #143E (e-meeting)	S2-2101434

February 24 – March 09, 2020, Elbonia	(revision of S2-2100591)

Source:	CATT

Title:	Establishment of a PDU Session associated with multicast session(s)

Document for:	Approval

Agenda Item:	8.9.2

Work Item / Release:	5MBS / Rel-17

Abstract of the contribution: This contribution proposes the procedures for establishment of a PDU Session associated with multicast session(s).

- Introduction

As described in the conclusion of FS_5MBS, "The AMF shall select an SMF that supports 5MBS for multicast session join during PDU session establishment, which is used for sending join (i.e. handling of join requests for 5MBS and/or fallback to individual delivery)." Accordingly, this contribution proposes the procedures for establishment of a PDU Session associated with multicast session(s) to the TS.

# 2. Proposal

It is proposed to include the following changes in TS 23.247.

******************** 1st Change (all new text)********************

7.2.x	Establishment of a PDU Session that can be associated with multicast session(s)

A PDU Session associated with multicast session(s) is established using the procedures as specified in TS 23.502 [W] clause 4.3.2.2 with the following differences:

- In step 1, in the NAS Message to the AMF, the UE includes an indication of establishing a PDU Session associated with multicast session(s);

Editor's note:	Whether an explicit indication of establishing a PDU Session associated with multicast session(s) is required is FFS. For example, an S-NSSAI or DNN supporting multicast services may be used instead.

- In step 2, based on the indication of establishing a PDU Session associated with multicast session(s), the AMF selects an SMF capable of handling multicast sessions based on locally configured data or a corresponding SMF capability stored in the NRF. For indirect discovery, the AMF requests the SCP to select an SMF capable of handling multicast sessions.

******************** End of Change ********************