3GPP TSG-WG SA2 Meeting #143E e-meeting  	S2-2101556

Elbonia, 24 Feb – 09 Mar, 2021	(revision of S2-21xxxxx)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 2545 | rev | 1 | Current version: | 16.7.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | NGAP UE-TNLA-binding update |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16, 5GS_Ph1; |  |  |  |  |  | Date: |  |  | 2021-02-18 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | When UE is in CM-CONNECTED State, if the AMF serving the UE is failure, and a downlink signalling is triggered, e.g. a PDU Session modification is triggered, the SMF re-select a new AMF from the AMF set. The new AMF send the N2 SM message to RAN. The N2 SM will be sent via a new TNL between the RAN and the new AMF. However, when the SMF triggers the DL N2 SM message, the RAN may have not detected the AMF failure yet. Hence, the NGAP association of the UE is still bound to a TNL toward the old AMF. It is unclear how the RAN change the NGAP UE-TLNA binding in this case: Should the RAN update the NGAP UE-TNLA binding with the new TNL association toward the new AMF when it receives a new DL message via a new TLN association? Or should the new AMF first request RAN to release the old NGAP UE-TNLA binding before the new AMF can send the N2 SM message to RAN(then how to identify the old TNL association )? Thus it is proposed that for the above case, the RAN update the NGAP UE-TNLA binding with the new TNL association when it receives a new DL message via a new TLN association. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The NGAP UE-TNLA binding is updated with the new UE-TNLA when RAN receives a DL message via a new TNLA. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It is impossible to update the NGAP UE-TNLA binding toward a new AMF. The RAN sends response to old unavailable AMF. Thus the AMF failure can not be handled. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.7.2.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

4.2.7.2.4	NGAP UE-TNLA-binding update procedure

At any time the AMF may decide to re-bind the NGAP UE association to a new TNL association:

-	by sending a UE-specific NGAP message on a new TNL association in case:

-	AMF responds to the 5G-AN node initiated NGAP message (i.e. triangular redirection) as described in clause 4.2.7.2.1, 4.2.7.2.2, and 4.2.7.2.3, or

- 	AMF initiated UE-specific NGAP message needs to be sent to 5G-AN node.

-	by sending a UE-specific NGAP UE-TNLA binding release message to 5G-AN and the 5G-AN node updates the NGAP UE-TNLA binding with the new TNL association.

# * * * * End of changes * * * *