SA WG2 Meeting #143E	S2-2102070

24 February - 9 March 2021, Elbonia

Source:	Nokia, Nokia Shanghai Bell, Ericsson

Title:	KI #3A: Conclusion update

Document for:	Discussion/Approval

Agenda Item:	8.5

Work Item / Release:	FS_IIoT / Rel-17

Abstract of the contribution: This contribution proposes to resolve ENs in KI#3A.

# 1		Discussion

# 2		Proposal

The following change is proposed for TR 23.700-20.

# * * * * First change * * * *

## 8.5	Key Issue #3A: Exposure of deterministic QoS

The following is taken as the basis for the way forward:

Take solution#5 as basis for KI#3A:

-	The AF provides traffic related description and QoS requirement:

-	UE related Identification used to determine target UE PDU Session, AF Identification, a Traffic Description, a 5GS delay, Bandwidth, which are used to identify the target traffic and related QoS requirement.

-	Flow Direction, Burst Arrival Time at UE (uplink) or UPF (downlink), Burst Size, Burst Periodicity, and a Timing Domain, which are used for efficient scheduling in RAN for Ethernet PDU session.

-	For ETH PDU Sessions, in order to reuse hold and forward functionality in the DS-TT and NW-TT, Qbv parameters can be derived by NEF/PCF based on AF request (with no impact to nodes other than NEF/PCF) and provided to NW-TT/DS-TT. It is assumed that Rel-16 hold and forward functionality in DS-TT and NW-TT is re-used.

Editor's note:	Whether a requirement exists that hold and forward functionality is needed for VIAPA services needs to be confirmed by SA WG1.

Editor's note:	Need for Jitter measurement is FFS.