- SA WG2 Meeting #S2-143E	S2-21000AA

- 24 February - 09 March, 2021, Electronic meeting

Source:	vivo Mobile Communications Ltd

Title:	TS 23.304: Functional Entities: Section 4.3.1 - UE

Agenda item:	8.8.2

Document for:	Approval

1. Introduction

The document contains text that described the UE functionality for 5G ProSe UE.

Questions for drafting:

a)	Do we need to preface everything with 5G?  If no how do we differentiate with interworking with LTE ProSe (if and when).?

b)	Control information was a term taken from 23.303, a word search shows it has no definition.

c)	Some bullets have subbullets, does it make sense to collapse or is it easier to read write out those functions in full?

**** All that follows is new text****

### 4.3.1	UE

Any ProSe-enabled UE may support the following functions:

-	Exchange of ProSe control information between ProSe-enabled UE and the DDNMF over PC3 reference point.

-	Procedures for ProSe Direct Discovery of other ProSe-enabled UEs over PC5 reference point.

-	Procedures for ProSe Direct Communication over PC5 reference point, including Broadcast, Groupcast and Unicast mode ProSe Direct Communication.

-	Procedures to act as a 5G ProSe UE-to-Network Relay. The Remote 5G ProSe UE communicates with the 5G ProSe UE-to-Network Relay over PC5 reference point.

-	Procedures to act as a 5G ProSe UE-to-UE Relay. The Remote 5G ProSe UE communicates with the 5G ProSe UE-to-UE Relay over PC5 reference point.

-	Exchange of control information between 5G ProSe-UEs over PC5 reference point, e.g. for 5G UE-to-Network Relay Discovery and Group Member Discovery.

-	Exchange of ProSe control information between another 5G ProSe-enabled UE and the DDNMF over PC3 reference point. In the 5G ProSe UE-to-Network Relay case the Remote UE will send this control information over PC5 user plane to be relayed over the Uu interface towards the DDNMF.

-	Indicate 5G ProSe Policy Provisioning Request in UE Policy Container for UE triggered 5G ProSe Policy provisioning.

-	Configuration of parameters for ProSe Direct Discovery and ProSe Direct Communication (e.g., including IP addresses, ProSe Layer-2 Group IDs, radio resource parameters, see clause X.Y). These parameters can be pre-configured in the UE, or, if in coverage, provisioned or updated by signalling over the N1 reference point from the PCF in the HPLMN or over PC1 reference point from the ProSe Application Server.

-	Enforce 5G ProSe Policy information for 5G ProSe Direct Discovery and 5G ProSe Direct Communications.

-	Report the following capabilities over the N1 reference point:

-	PC5 Capability;

-	capability to act as a Remote 5G ProSe UE/UE-to-Network Relay;

-	capability to act as a Remote 5G ProSe UE/UE-to-UE Relay;