SA WG2 Meeting #S2-143E	S2-210xxxx

24 February – 09 March, 2021, Electronic, Elbonia

Source:	InterDigital

Title:	Session management for UAV communication

Document for:	Approval

Agenda Item:	8.7

Work Item / Release:	ID_UAS/Rel-17

Abstract of the contribution:

### 5.2.X	Session Management

An UAV uses PDU Sessions or PDN Connections for communication with the USS/UTM and for C2 communication with a networked UAV-C. A networked UAV-C uses existing procedures for establishing PDU Session or PDN Connection for communication with the USS/UTM as a normal UE, and the procedures described in this clause do not apply to a networked UAV-C.

PDU Session/PDN Connection for UAV communication shall only be established after a UAV has been authenticated and authorized by the USS/UTM, as described in clause 5.2.2 and clause 5.2.3. An UAV may use a common PDU Session/PDN Connections for communication with the USS/UTM and C2 communication with the UAV-C or may use separate PDU Sessions/PDN Connections for communication with the USS/UTM and C2 communication with the UAV-C respectively. If a common PDU Session/PDN Connection is used, the UAV may establish the PDU Session/PDN Connection and enable the C2 communication during the same procedure, or the UAV may establish the PDU Session/PDN Connection for communication with the USS/UTM first and then later enable the C2 communication using the PDU Session/bearer modification procedure. In both cases, C2 pairing authorization shall be successfully performed before the C2 communication is enabled as described in clause 5.2.3 and clause 5.2.5.

The UAV shall indicate that the PDU Session/PDN Connection is for the UAV communication and/or C2 communication in the PDU Session Establishment/PDN Connectivity request.

Editor's note:	How the UAVs indicate UAV communication and/or C2 communication, e.g. with an explicit indication or DNN/S-NSSAI combination, is FFS.

During the establishment or modification procedure of the PDU Session/PDN connection for C2 communication, the USS/UTM shall provide the 3GPP system with following information:

-	traffic filters for enabling C2 communication.

-	QoS requirements for C2 communication

Editor's note:	How the USS/UTM provides C2 QoS requirement through UAS-NF is FFS and may need coordination with SA6.

When the C2 pairing authorization is revoked by the USS/UTM, the SMF or SMF+PGW-C shall release the PDU Session/PDN connection for C2 communication (in case dedicated connection is used), or disable C2 communication for the PDU Session/PDN connection (in case separate connections are used). When the UUAA is revoked by the USS/UTM, all UAV related PDU Session/PDN connections shall be released.