SA WG2 Meeting #143E	S2-2100682

24 Feb – 9 Mar, 2021, Electronic meeting

Title:		[DRAFT] Reply LS on NSSAA at inter-PLMN mobility

Reply to:	S2-2100005 / C1-206508

Release:	Rel-16

Work Item:	eNS

Source:	SA2

To:		CT1

Cc:		CT4, SA3

Contact person:	Hoyeon Lee

hy50.lee@samsung.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

# 1	Overall description

SA2 thanks CT1 for their LS in S2-2100005/C1-206508.

SA2 would like to provide the following answers to CT1’s questions:

[CT1 question]: If the target VPLMN decides to run NSSAA for an S-NSSAI whose NSSAA status is EAP-Success when a UE accesses the target VPLMN for the first time, how will the target VPLMN handle the S-NSSAI that is subject to NSSAA: will it be put in the pending NSSAI or will it be put in the allowed NSSAI included in the registration accept message sent to the UE, and how are existing PDU sessions associated with the S-NSSAI handled?

[SA2 response]:

If, subject to operator policy, VPLMN can rely on the EAP status received from the source PLMN, the EAP status can be kept as EAP-Success and the AMF keeps the S-NSSAI only in the Allowed NSSAI, and once the NSSAA has been executed the AMF either do nothing (NSSAA successful) or the AMF updates the Allowed NSSAI and indicates the S-NSSAI as a rejected S-NSSAI (NSSAA unsuccessful). If there are existing PDU session(s) associated with the S-NSSAI, the existing PDU Session(s) are kept until the NSSAA is executed and if the NSSAA fails the corresponding PDU Sessions are released as described in TS 23.502 clause 4.2.9.2.

If, subject to operator policy, VPLMN cannot rely on the EAP status received from the source PLMN, the AMF puts the S-NSSAI only in the Pending NSSAI (provided in Registration Accept). If there are existing PDU session(s) associated with the S-NSSAI, the existing PDU Session(s) are released as described in TS 23.501 clause 5.15.5. The AMF executes NSSAA and afterwards updates the UE with UCU as per normal procedures.

# 2	Actions

To CT1

ACTION: 	SA2 kindly asks CT1 to take the answers above into account.

# 3	Dates of next TSG SA WG2 meetings

TSG SA WG2 Meeting 144E		12th – 16th April 2021	Electronic meeting

TSG SA WG2 Meeting 145E		17th – 28th May 2021	Electronic meeting