SA WG2 Meeting #143E e-meeting	S2-210xxxx

Elbonia, February 24 – March 09, 2021

Source:	Convida Wireless LLC, Qualcomm Incorporated, AT&T, LG Electronics, Samsung (?), NTT Docomo (?), InterDigital (?)

Title:	ECS address provisioning

Document for:	Approval

Agenda Item:	8.3

Work Item / Release:	eEDGE_5GC / Rel-17

Abstract: This contribution fille in the “ECS address provisioning” section of the 5GS Edge Computing TS 23.548.

# Discussion

In TR 23.748, Section 9.1.3, it was concluded that network will be able to provide ECS Address(es) to the UE during PDU Session Establishment.  This p-CR adds a description of the procedure to the new Edge TS. There is a companion TS 23.502 CR in S2-210xxxx.

# Proposal

This contribution proposes to implement the following updates to TS 23.548 v0.0.0.

* * * Start of Change * * *

## 6.5	Support of Application Layer Architecture for Enabling EC

### 6.5.1	General

Editor’s Note: This clause refers to TS 23.558 for the specifications of the Discovery over the Architecture Enabling Edge Applications.

### 6.5.2	ECS address provisioning

The Edge Configuration Server (ECS) provides supporting functions needed for the Edge Enabler Client (EEC) to connect with an Edge Enabler Server (EES). A UE that hosts an EEC(s) may support the ability to transfer ECS address(es) to the EEC(s).

It the UE hosts an EEC and supports transferring the ECS address received from the 5GC to the EEC, the UE indicates in the PCO at PDU Session establishment or modification that it supports the ability to transfer ECS address(es) to the EEC(s).

The ECS Information consists of one or more FQDN(s) and/or IP Address(es) of Edge Configuration Server(s). As described in TS 23.502 [3], if the UE supports the ability to transfer ECS address(es) to the EEC(s), the UE may receive ECS Information from the SMF via PCO during PDU Session Establishment. The SMF may derive the ECS Information based on local configuration, the UE's location, and/or UE subscription information. The SMF may decide to send updated ECS Information to the UE based on locally configured policy, updated UE subscription information, or a change of UE location. The PDU Session Modification procedure is used to send updated ECS Information to the UE as described in TS 23.502 [3].

NOTE:	In home routed sessions, the ECS Information comes from the H-SMF.