3GPP TSG-WG SA2 Meeting #143E e-meeting 	S2-210xxxx

Elbonia, Feb 24 – March 9, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR |  | rev | - | Current version: | 16.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | NWDAF Reselection for Multiple NWDAF deployments |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Samsung?, Ericsson?, China Mobile?, CATT?, KDDI?, Huawei?, NTT DOCOMO? |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_Ph2 |  |  |  |  |  | Date: |  |  | 2021-02-24 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | According to the conclusion of KI#2: Multiple NWDAF instances in TR 23.700-91, it is proposed that re-selection of NWDAFs is specified, as such re-selection is required in certain cases that may occur in a multiple NWDAF deployment. Principles in clause 8.2.2 shall be used as baseline for the normative work to support an NWDAF transfer analytics subscriptions as well as associated analytics context information to another NWDAF instance. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - add procedures for transfer of analytics subscriptions between NWDAFs - add procedures for transfer of analytics context information - add 2 new service operations, Nnwdaf_AnalyticsSubscription_Transfer and Nnwdaf_AnalyticsInfo_Transfer that are used in the new procedures - defined "analytics context information" and "analytics subscription information" |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Transfer/offloading of analytics subscriptions to another NWDAF instance, e.g., during overload situation, graceful shutdown, or due to UE mobility would not be possible, resulting in a degraded NWDAF service. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.3, 6.1A (new), 7.1, 7.2.1, 7.2.4, 7.2.x (new), 7.3.1, 7.3.x (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*** Start of the changes ***

### 6.1.3	Contents of Analytics Exposure

The consumers of the Nnwdaf_AnalyticsSubscription_Subscribe or Nnwdaf_AnalyticsInfo_Request service operations described in clause 7 provide the following input parameters listed below.

-	A list of Analytics ID(s): identifies the requested analytics.

-	Analytics Filter Information: indicates the conditions to be fulfilled for reporting Analytics Information. This set of optional parameter types and values enables to select which type of analytics information is requested. Analytics Filter Information are defined in procedures.

-	Target of Analytics Reporting: indicates the object(s) for which Analytics information is requested, entities such as specific UEs, a group of UE(s) or any UE (i.e. all UEs).

-	(Only for Nnwdaf_AnalyticsSubscription_Subscribe) A Notification Target Address (+ Notification Correlation ID) as defined in TS 23.502 [3] clause 4.15.1, allowing to correlate notifications received from NWDAF with this subscription.

-	Analytics Reporting Information with the following parameters:

-	(Only for Nnwdaf_AnalyticsSubscription_Subscribe) Analytics Reporting Parameters as per Event Reporting parameters defined in Table 4.15.1-1, TS 23.502 [3].

-	(Only for Nnwdaf_AnalyticsSubscription_Subscribe) Reporting Thresholds, which indicate conditions on the level of each requested analytics that when reached shall be notified by the NWDAF. A matching direction may be provided such as below, above, or crossed. If no matching direction is provided, the default direction is crossed.

-	Analytics target period: time interval [start..end], either in the past (both start time and end time in the past) or in the future (both start time and end time in the future). An Analytics target period in the past is a request or subscription for statistics. An Analytics target period in the future is a request or subscription for predictions. The time interval is expressed with actual start time and actual end time (e.g. via UTC time). When the Analytics Reporting Parameters indicate a periodic reporting mode, the time interval can also be expressed as positive or negative offsets to the reporting time, which indicates a subscription for predictions or statistics respectively. By setting start time and end time to the same value, the consumer of the analytics can request analytics or subscribe to analytics for a specific time rather than for a time interval.

-	Preferred level of accuracy of the analytics (e.g. Low/High).

-	(Only for Nnwdaf_AnalyticsInfo_Request) Time when analytics information is needed (if applicable). If the time is reached the consumer does not need to wait for the analytics information any longer, yet the NWDAF may send an error response to the consumer.

-	[OPTIONAL] Maximum number of objects requested by the consumer (max) to limit the number of objects in a list of analytics per Nnwdaf_AnalyticsSubscription_Notify or Nnwdaf_AnalyticsInfo_Request response.

-	[OPTIONAL] Maximum number of SUPIs (SUPImax) requested by the consumer to limit the number of SUPIs in an object. When SUPImax is not provided, the NWDAF shall return all SUPIs concerned by the analytics object. When SUPImax is set to 0, the NWDAF shall not provide any SUPI.

The NWDAF provides to the consumer of the Nnwdaf_AnalyticsSubscription_Notify or Nnwdaf_AnalyticsInfo_Request service operations described in clause 7, the output information listed below:

-	(Only for Nnwdaf_AnalyticsSubscription_Notify) The Notification Correlation Information.

-	For each Analytics ID the analytics information in the requested Analytics target period.

-	In addition, the following additional information:

-	Timestamp of analytics generation, which allows consumers to decide until when the received information shall be used. For instance, an NF can deem a received notification from NWDAF for a given feedback as invalid based on this timestamp;

-	Validity period, which defines the time period for which the analytics information is valid.

-	Probability assertion: confidence in prediction.

*** Next change (all new text) ***

## 6.1A	Procedures for analytics subscription transfer

### 6.1A.1	General

In multiple NWDAFs deployment scenario, it is possible that, within the time period of data collection and/or analytics computation, an NWDAF instance is required to transfer one or more of its analytics subscriptions to another NWDAF instance, e.g. due to load balancing, graceful shutdown, or UE mobility.

Procedures for analytics subscription transfer allow one NWDAF instance to transfer or handover its ongoing analytics subscriptions to another NWDAF instance. The transfer can be done for all subscriptions or just for specific area(s), specific Analytics ID(s), and/or specific UE(s).

The analytics subscription transfer can also be prepared if the source NWDAF instance anticipates that it will soon not be able to continue the analytics tasks that it has been requested for.

### 6.1A.2	Analytics Subscription Transfer

The procedure in figure 6.1A.2-1 is used by an NWDAF instance to request the transfer of analytics subscription(s) to another NWDAF instance, using the Nnwdaf_AnalyticsSubscription_Transfer service operation defined in clause 7.2.x. This service operation is also used by an NWDAF instance to request another NWDAF instance to prepare taking over analytics subscription(s) from the source NWDAF instance.

NOTE:	In this procedure, the analytics Consumer does not change.

Editor's Note:	It is FFS if the procedure for "change of analytics consumer" can co-exist/be merged with below procedure. If not, it is FFS how to ensure that only one of the procedures is triggered.

Figure 6.1A.2-1: Analytics Subscription Transfer

The procedure of analytics subscription transfer comprises the following steps:

1.	Consumer NF subscribes to NWDAFx for certain analytics as specified in clause 6.1.1.

NOTE 1:	In this procedure, NWDAFx is the NWDAF instance that needs to transfer analytics subscriptions. NWDAFy and NWDAFz are examples for NWDAF instances that are candidates to take over those analytic subscriptions.

2.	NWDAFx starts collecting required data from NFs or OAM as specified in clause 6.2. NWDAFx starts performing requested analytics.

If NWDAFx determines that it needs to prepare to transfer analytics subscriptions to another NWDAF instance, the following steps 3 to 8 shall be executed:

NOTE 2:	NWDAFs determines that it needs to prepare to transfer analytics subscriptions to another NWDAF instance, as NWDAFx estimates that the UE may enter an area which is not covered by NWDAFx, e.g., by subscribing to AMF event exposure service for UE mobility event notifications or to other NWDAF providing UE mobility analytics. Another example to prepare for an analytics transfer is that NWDAFx wants to resolve an overload situation or prepare for a graceful shutdown.

3.	The NWDAFx discovers via NRF candidate NWDAF instances NWDAFy and NWDAFz supporting the requested analytics information for the target area. NWDAF discovery and selection is specified in clause 6.3.13, TS 23.501 [2].

4-5.	In case of a prepared analytics subscription transfer, NWDAFx requests, using Nnwdaf_AnalyticsSubscription_Transfer Request, to NWDAFy and NWDAFz to prepare for an analytics subscription transfer by including a "prepared analytics subscription transfer indication" in the request message. The request message also includes information on the analytics subscriptions to be transferred. The request message may also include "analytics context identifier(s)" indicating the availability of analytics context for particular Analytic ID(s).

NWDAFy and NWDAFz respond to the request from NWDAFx using a Nnwdaf_AnalyticsSubscription_Transfer Response message.

6-7.	[Conditional] If "analytics context identifier(s)" had been included in the Nnwdaf_AnalyticsSubscription_Transfer Request received in step 4, NWDAFy and NWDAFz, based on the provided "analytics context identifier(s)", shall request the "analytics context" from the NWDAFx. The analytics information transfer procedure is specified in clause 6.1A.3.

Editor's Note:	It is FFS, whether procedure in clause 6.1A.3 is specified, or other new/existing service operations are used instead.

NOTE 2:	NWDAFy and NWDAFz allocate a new subscription Id to the received analytics subscriptions, following the behaviour of AmfEventSubscription as specified in clause 6.2.6.2.2, TS 29.518 [x] and UEContextTransfer as specified in clause 5.2.2.2.1.1, TS 29.518 [x].

8.	[Optional] Based on the information received from NWDAFx, NWDAFy and NWDAFz start data collection from NFs or OAM (as specified in clause 6.2) and analytics generation for the indicated analytics subscriptions.

NWDAFx determines to transfer analytics subscription(s) to another NWDAF instance NWDAFy and the following steps are executed:

NOTE 3:	The NWDAFx can find out that a UE has moved to an area not covered by NWDAFx by subscribing to Namf_EventExposure service.

9a.	NWDAFx requests, using Nnwdaf_AnalyticsSubscription_Transfer Request, to NWDAFy to take over a set of analytics subscription(s) as provided in the request message. The request message may also include "analytics context identifier(s)" indicating the availability of analytics context for particular Analytic ID(s).

NOTE 4:	If NWDAFy had accepted to prepare for an analytics subscription based on a prepared analytics subscription transfer request received from NWDAFx in step 4, the "analytics context identifier(s)" provided in step 9a indicate additional information / data collected or generated by NWDAFx between steps 6 and step 9a.

9b.	NWDAFy accepts the analytics subscription transfer and 	takes over the analytics generation based on the information received from NWDAFx. If necessary, it subscribes to relevant entities for data collection required for the Analytics ID(s).

NOTE 5:	NWDAFy might already have received information on some/all of the analytics subscriptions as part of the prepared analytics subscription transfer request received in step 4 and, thus, might already have started to prepare for the analytics generation, e.g., by having already subscribed to relevant event notifications.

NOTE 6:	If not yet done during a prepared analytics subscription transfer, NWDAFy allocates a new subscription Id to the received analytics subscriptions, following the behaviour of AmfEventSubscription as specified in TS 29.518 [x] clause 6.2.6.2.2 and UEContextTransfer as specified in TS 29.518 [x] clause 5.2.2.2.1.1.

9c.	NWDAFy informs the Consumer NF about the new subscription Id that had been assigned by NWDAFy (see note 6) via the Change Subscription Notification Correlation Id parameter in a Nnwdaf_AnalyticsSubscription_Notify message as specified in clause 7.2.4.

9d.	NWDAFy confirms the analytics subscription transfer request.

10.	[Conditional] If "analytics context identifier(s)" had been included in the Nnwdaf_AnalyticsSubscription_Transfer Request received in step 9, NWDAFy, based on the provided "analytics context identifier(s)", shall request the "analytics context" from the NWDAFx. The analytics information transfer procedure is specified in clause 6.1A.3.

11.	NWDAFy takes over the analytics generation based on the information received from NWDAFx and, if necessary, starts to subscribe to relevant entities for data collection required for the Analytics ID(s).

NOTE 6:	NWDAFy might already have received information on some/all of the analytics subscriptions as part of the prepared analytics subscription transfer request received in step 4 and, thus, might already have started to prepare for the analytics generation, e.g., by having already subscribed to relevant event notifications.

12.	NWDAFy informs the Consumer NF about the new subscription Id that had been assigned by NWDAFy (see note 5) via the Change Subscription Notification Correlation Id parameter in a Nnwdaf_AnalyticsSubscription_Notify message as specified in clause 7.2.4.

11.	[Conditional] If NWDAFz had accepted to prepare for an analytics subscription based on a prepared analytics subscription transfer request received from NWDAFx in step 5, NWDAFx informs NWDAFz, using Nnwdaf_AnalyticsSubscription_Transfer Request include an "analytics subscription transfer cancel indication". NWDAFz confirms the cancelation to NWDAFx and, if applicable, deletes any analytics data that is no longer needed. If NWDAFz, as part of the analytics subscription preparation, had already subscribed to entities to collect data, it unsubscribes to those entities if the subscriptions are not needed for other active analytics subscriptions with NWDAFz.

12-13.	NWDAFy at some point derives new output analytics based on new input data and notifies the Consumer NF about the new analytics using a Nnwdaf_AnalyticsSubscription_Notify message as specified in clause 6.1.1.

NOTE 7:	Notification correlation information allows the Consumer NF to correlate the notifications to the initial subscription request made with NWDAFx in step 1.

### 6.1A.3	Analytics Context Information Transfer

The procedure depicted in figure 6.1A.3-1 is used by an NWDAF instance to request analytics context information from another NWDAF instance, using the Nnwdaf_AnalyticsInfo_Transfer service operation as defined in clause 7.3.x. This procedure, for example, can be called during an analytics subscription transfer procedure (as specified in clause 6.1A.2) to request the transfer of relevant analytics information.

Figure 6.1A.3-1: Analytics Context InformationTransfer

The procedure of analytics information transfer comprises the following steps:

1.	The requester NWDAFx requests analytics context information by invoking Nnwdaf_AnalyticsInfo_Request service operation. The parameters that can be provided in the request message are listed in clause 6.1A.4.

2.	The provider NWDAFy responds with analytics context information to the requesting NWDAFx.

### 6.1A.4	Contents of Analytics Context Information

The consumers of the Nnwdaf_AnalyticsInfo_Transfer service operation (as specified in clause 7.3.x) provide the following input parameters listed below:

A list of analytics context identifier(s): identify a set of analytics context information that are available at the NWDAF instance providing this service and that are requested to be transferred to the consumer NWDAF instance.

NOTE:	A list of "analytics context identifier(s)" is received during an analytics subscriptions transfer request as described in clause 6.1A.2.

The NWDAF provides to the consumer of the Nnwdaf_AnalyticsInfo_Transfer service operation (as specified in clause 7.3.x), the output information listed below:

- (Set of) Analytics context information for each of the "analytics context identifier(s)" provided as input parameter. "Analytics context information" includes the following information parts:

-	Analytics related:

[OPTIONAL] Pending output analytics (i.e., not yet notified to the consumer) or historical output analytics information, if available at the source NWDAF. The content of the output analytics is specified in clause 6.1.3 as output information of the Nnwdaf_AnalyticsSubscription_Notify or Nnwdaf_AnalyticsInfo_Request service operations.

Timestamp(s) of the last output analytics provided to the Consumer NF(s). Value is set to 0 if no output analytics had been sent yet.

-	Data collection related:

[OPTIONAL] Historical data that is available at the source NWDAF and that is related to the Analytics to be handed over to the target NWDAF. This includes not only the raw data collected from the data source(s), but also "analytics metadata" such as the time period of the collected data, information about the data source(s), etc. If historical data is available at the source NWDAF, this data shall be provided.

Information about subscriptions with data sources that are related to the analytics.

Editor's Note:	Other parameters (analytics and/or data collection related) of the Analytics Context Information are FFS.

*** Next change ***

## 7.1	General

Table 7.1-1 illustrates the NWDAF Services.

Table 7.1-1: NF services provided by NWDAF

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Nnwdaf_AnalyticsSubscription | Subscribe | Subscribe / Notify | PCF, NSSF, AMF, SMF, NEF, AF, OAM |
|  | Unsubscribe |  | PCF, NSSF, AMF, SMF, NEF, AF, OAM |
|  | Notify |  | PCF, NSSF, AMF, SMF, NEF, AF, OAM |
|  | Transfer | Request / Response | NWDAF |
| Nnwdaf_AnalyticsInfo | Request | Request / Response | PCF, NSSF, AMF, SMF, NEF, AF, OAM |
|  | Transfer | Request / Response | NWDAF |
| NOTE: How OAM consumes Nnwdaf services and which Analytics information is relevant is defined in TS 28.550 [7] Annex H and out of the scope of this TS. |  |  |  |

Table 7.1-2 shows the analytics information provided by NWDAF service.

Table 7.1-2: Analytics information provided by NWDAF

| Analytics Information | Request Description | Response Description |
| --- | --- | --- |
| Slice Load level information | Analytics ID: load level information | Load level of a Network Slice Instance reported either as notification of crossing of a given threshold or as periodic notification (if no threshold is provided). |
| Observed Service experience information | Analytics ID: Service Experience | Observed Service experience statistics or predictions may be provided for a Network Slice or an Application. They may be derived from an individual UE, a group of UEs or any UE. For slice service experience, they may be derived from an Application, a set of Applications or all Applications on the Network Slice. |
| NF Load information | Analytics ID: NF load information | Load statistics or predictions information for specific NF(s). |
| Network Performance information | Analytics ID: Network Performance | Statistics or predictions on the load in an Area of Interest; in addition, statistics or predictions on the number of UEs that are located in that Area of Interest. |
| UE mobility information | Analytics ID: UE Mobility | Statistics or predictions on UE mobility. |
| UE Communication information | Analytics ID: UE Communication | Statistics or predictions on UE communication. |
| Expected UE behavioural parameters | Analytics ID: UE Mobility and/or UE Communication | Analytics on UE Mobility and/or UE Communication. |
| UE Abnormal behaviour information | Analytics ID: Abnormal behaviour | List of observed or expected exceptions, with Exception ID, Exception Level and other information, depending on the observed or expected exceptions. |
| User Data Congestion information | Analytics ID: User Data Congestion | Statistics or predictions on the user data congestion for transfer over the user plane, for transfer over the control plane, or for both. |
| QoS Sustainability | Analytics ID: QoS Sustainability | For statistics, the information on the location and the time for the QoS change and the threshold(s) that were crossed; or, for predictions, the information on the location and the time when a potential QoS change may occur and what threshold(s) may be crossed. |

## 7.2	Nnwdaf_AnalyticsSubscription Service

### 7.2.1	General

Service Description: This service enables the consumer to subscribe/unsubscribe for network data analytics.

When the subscription for network data analytics is accepted by the NWDAF, the consumer NF receives from the NWDAF an identifier (Subscription Correlation ID) allowing to further manage (modify, delete) this subscription. The modification of Analytics subscription can be enforced by NWDAF based on operator policy and configuration.

This service also enables the consumer to transfer analytics subscriptions to another NWDAF using the Nnwdaf_AnalyticsSubscription_Transfer service operation. Using this service operation, a consumer can request to the NWDAF to prepare for or take over analytics subscription(s), indicating also if and where analytics context is available.

*** Next change  ***

### 7.2.4	Nnwdaf_AnalyticsSubscription_Notify service operation

Service operation name: Nnwdaf_AnalyticsSubscription_Notify.

Description: NWDAF notifies the consumer instance of the analytics that has subscribed to the specific NWDAF service.

Inputs, Required: Notification Correlation Information: this parameter indicates the subscription Id that has been assigned to the consumer during event subscription. If this event is for informing the creation of a new subscription Id at the NWDAF, this IE shall indicate the old subscription Id.

Inputs, Optional:

- Set of the tuple (Analytics ID, Analytics specific parameters): this parameter shall be present if output analytic(s) is reported.

- Subscription Change Notification Correlation Id: this parameter shall be present if the notification is for informing the creation of a new subscription Id at the NWDAF.

- Timestamp of analytics generation.

- Validity period.

- Probability assertion.

NOTE 1:	Some NWDAF output analytics already include confidence of predictions, which provides the same information as probability assertion.

NOTE 2:	Validity period can also be provided as part of Analytics specific parameters for some NWDAF output analytics.

Outputs, Required: Operation execution result indication.

Outputs, Optional: None.

*** Next change (all new text) ***

### 7.2.x	Nnwdaf_AnalyticsSubscription_Transfer service operation

Service operation name: Nnwdaf_AnalyticsSubscription_Transfer.

Description: Requests to NWDAF to transfer analytics subscription(s).

Inputs, Required: None.

Inputs, Optional:

- If this service operation is for a prepared analytics subscription transfer the following parameter shall be provided:

  - (Set of) analytics subscription(s) with the following parameters: Id of consumer NF that has subscribed to receive analytics, Analytics ID(s), Analytics Filter Information, target of analytics reporting (as defined in clause 7.2.2),

- If this service operation is to request analytics subscriptions transfer the following parameter shall be provided:

  - (Set of) analytics subscription(s) with the following parameters: Id of consumer NF that has subscribed to receive analytics, all input parameters of the Nnwdaf_AnalyticsSubscription_Subscribe or Nnwdaf_AnalyticsInfo_Request service operations as defined in clause 7.2.2,

- Transfer type: indicating the type of the transfer request. The following values are supported:

  - Prepared analytics subscription transfer,

  - Analytics subscription transfer confirmation,

  - Analytics subscription transfer cancel.

- (Set of) analytics context identifier(s).

Outputs Required: None.

Outputs, Optional: None.

*** Next change ***

## 7.3	Nnwdaf_AnalyticsInfo service

### 7.3.1	General

Service description: This service enables the consumer to request and get from NWDAF network data analytics or enables NWDAF to request transfer of analytics context information from another NWDAF.

*** Next change (all new text) ***

### 7.3.x	Nnwdaf_AnalyticsInfo_Transfer service operation

Service operation name: Nnwdaf_AnalyticsInfo_Transfer.

Description: Requests to NWDAF to transfer information related to analytics subscriptions.

Inputs, Required: (Set of) Analytics context identifier(s).

Inputs, Optional: None.

Outputs Required: (Set of) Analytics context information.

Outputs, Optional: None.

### *** End of the changes ***

### *** For information only ***

From 23.700-91 KI#2 conclusions (green parts are addressed in this contribution):

a)	Extend or re-use existing Release-16 service operations, i.e. do not introduce new AnalyticsRelocation service.

b)	Support different configurable triggers for the NWDAF re-selection corresponding to specific situations such as UE mobility, load balancing, and graceful shutdown. Additional triggers will be decided during the normative phase.

c)	Support the handover of analytics subscriptions between NWDAFs.

If NWDAF re-selection is initiated by the source NWDAF, the source NWDAF will make use of NRF to find candidate NWDAF(s) and the request(s) for handover of analytics subscriptions in a Nnwdaf_Analytics message to the selected target NWDAF(s). The request(s) include information related to the subscriptions to be handed over, including for each subscription at least the subscribed entity, the Analytics ID and corresponding parameters (as defined in clause 6.1.3 of TS 23.288 [5]), as well as an information about additional analytics context available at the source NWDAF (see bullet e)).

d)	Support the transfer of analytics context:

-	NF consumer was not changed, i.e. NWDAF re-selection was initiated by the source NWDAF: the target NWDAF initiates the collection of information related to the subscriptions handed over.

-	NF consumer is changed, i.e. NWDAF re-selection is initiated by the target consumer NF: the analytics context is requested by the target NWDAF from the source NWDAF. When AMF change happens, information (such as source NWDAF ID) can be sent in extended existing message Namf_Communication_UEContextTransfer. Further, when a DCCF is deployed it can be used to support the context transfer at reselection of NWDAF when change of a consumer NF happens.

e)	Analytics context can include the following information parts:

-	Analytics related:

Information related to entities that are subscribed to receive analytics.

Pending Analytics (i.e. not yet notified to the consumer) or historical Analytics Output information - if available at the source NWDAF.

-	Historical data:

The target NWDAF may retrieve historical data that is available at the source NWDAF and related to the Analytics to be handed over to the target NWDAF (including not only the raw data collected from the data source(s), but also "analytics metadata" such as the time period of the collected data, information about the data source(s), etc.).

NOTE:	If DRF and DCCF are deployed, the target NWDAF can retrieve historical data from the DRF either directly or via a request to DCCF.

f)	If no change of Analytics consumer is performed, the target NWDAF notifies the NWDAF service consumer about the re-location. If change of Analytics consumer is performed no notification is needed.

g)	Support the prepared analytics handover to inform target NWDAF(s) about a potential analytics handover. This message is to check the acceptability of the analytics handover at the target NWDAF(s) and allows the target NWDAF(s) to prepare to take over those Analytics subscriptions from the source NWDAF by requesting the related analytics context from the source NWDAF, creating subscriptions with the required data sources, etc. In case the source NWDAF has contacted multiple target NWDAF(s) about such prepared analytics handover, the source NWDAF has to confirm the handover to the selected target NWDAF and cancel the prepared analytics handover to the other candidate NWDAF(s).

h)	Purging of UE related data in combination with or shortly after a UE has stopped being served by the source NWDAF is a good principle that shall be possible.