3GPP TSG-SA2 Meeting #143-E	S2-2100095

Elbonia, February 24 – March 09, 2021

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 2463 | rev | - | Current version: | 16.7.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: |  |  |  |  | UICC apps |  |  |  | ME |  |  | Radio Access Network |  |  |  |  |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Title: | Event Exposure in support of Dispersion Analytics |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Source to WG: | Spirent, Huawei, ATT |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_ph2 |  |  |  |  |  |  |  |  |  |  |  | Date: |  | 2021-02-18 |  |  |  |  |
| Category: | B |  |  |  |  |  |  |  |  |  |  |  | Release: |  | Rel-17 |  |  |  |  |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) |  |  |  |  |  |
| Reason for change: |  | With the introduction of Dispersion Analytics feature, as captured in the conclusion of Key Issue #9, TR 23.700-91, data dispersion analytics and transactions dispersion analytics require the extensions of EventExposure to provide transaction counts per UE, group of UEs or periodic reporting. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Summary of change: |  | An extension of the Namf_EvenExosure and Nsmf_Event Exposure services to include transactions count. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The Dispersion Analytic feature, will not be able to fetch key information required to determine transactions dispersion analytics. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.2.3, 5.2.8.3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other specs |  | X |  |  | Other core specifications |  |  |  |  |  |  | TS/TR 23.288 CR xxx |  |  |  |  |  |  |  |
| affected: |  |  | X |  | Test specifications |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| (show related CRs) |  |  | X |  | O&M Specifications |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

* * * * First Change * * * *

#### 5.2.2.3	Namf_EventExposure service

##### 5.2.2.3.1	General

Service description: This service enables an NF to subscribe and get notified about an Event ID.

Following UE access and mobility information event are considered (Event ID is defined in clause 4.15.1 and Table 4.15.3.1-1):

-	Location Report (TAI, Cell ID, N3IWF/TNGF node, UE local IP address and optionally UDP source port number);

-	UE moving in or out of a subscribed "Area Of Interest" as described in clauses 5.3.4.4 and 5.6.11 in TS 23.501 [2];

-	Number of UEs served by the AMF and located in "Area Of Interest";

-	Time zone changes (UE Time zone);

-	Access Type changes (3GPP access or non-3GPP access);

-	Registration state changes (Registered or Deregistered);

-	Connectivity state changes (IDLE or CONNECTED);

-	UE loss of communication;

-	UE reachability status;

-	UE indication of switching off SMS over NAS service;

-	Subscription Correlation ID change (implicit subscription);

-	UE Type Allocation code (TAC);

-	Frequent mobility re-registration;

-	Subscription Correlation ID addition (implicit subscription); and

-	User State Information in 5GS, as described in clause 5.4.4 in TS 23.632 [68];

-    Mobility Management transaction count information

Event Filters are used to specify the conditions to match for notifying the event (i.e. "List of Parameter values to match"). If there are no conditions to match for a specific Event ID, then the Event Filter is not provided. The following table provides some examples on how the conditions to match for event reporting can be specified for various Event IDs for AMF exposure.

When the consumer NF is the NWDAF, the mobility management transaction count is used to collect UE’s or group of UEs’ transaction count data for Dispersion Analytics as specified in TS 23.288 [50]. The transaction count is incremented when the transactions include messages from Authentication, Registration, De-Registration, Service Request and UE Configuration Update procedures.

NOTE:	The conditions to match can be set based on AMF-associated expected UE Behaviour parameter(s) to only notify the event when the UE's behaviour deviates from its expected UE behaviour as described in TS 23.288 [50].

Table 5.2.2.3.1-1: Example of Event Filters for AMF exposure events

| Event ID | Event Filter (List of Parameter Values to Match) |
| --- | --- |
| Location Report | <Parameter Type = LocationFilter, Value = TA1> |
| UE moving in or out of Area of Interest | <Parameter Type = TAI, Value = TA1> <Parameter Type = S-NSSAI, Value = S-NSSAI1> <Parameter Type = NSI ID, Value = NSI ID1> <Parameter Type = PRA ID, Value = PRA ID value> |
| Access Type | <Parameter Type=AN Type, Value=3GPP Access"> |
| Location | <Parameter Type=TAI, Value=wildcard> (to report any TAI change) |
| Location | <Parameter Type=TAI Value=abnormal> (to report only when the TAI deviates from expected values based on Expected UE Moving Trajectory). |
| Reachability Filter | Applicable to the event UE reachability. Value = UE reachability status change or UE reachable for DL traffic. Absence of this parameter in UE reachability event request is interpreted as "UE reachability status change". |
| Transaction Count | ,<Parameter Type = TAI, Value = TA1> <Parameter Type = Cell Id, Value Cell Id_1> <Parameter Type = S-NSSAI, Value = S-NSSAI1> |

The following service operations are defined for the Namf_EventExposure service:

-	Namf_EventExposure_Subscribe.

-	Namf_EventExposure_UnSubscribe.

-	Namf_EventExposure_Notify.

* * * * Second  Change * * * *

#### 5.2.8.3	Nsmf_EventExposure Service

##### 5.2.8.3.1	General

Service description: This service provides events related to PDU Sessions towards consumer NF. The service operations exposed by this service allow other NFs to subscribe and get notified of events happening on PDU Sessions. The following are the key functionalities of this NF service.

-	Allow consumer NFs to Subscribe and unsubscribe for an Event ID on PDU Session(s);

-	Allow the NWDAF to collect data for network data analytics as specified in TS 23.288 [50];

-	Notifying events on the PDU Session to the subscribed NFs; and

-	Allow consumer NFs to acknowledge or respond to an event notification.

The following events can be subscribed by a NF consumer (Event ID is defined in clause 4.15.1):

-	UE IP address / Prefix allocation/change: The event notification may contain a new UE IP address / Prefix or an indication of which UE IP address / Prefix has been released.

-	PDU Session Establishment and/or PDU Session Release.

The event notification may contain following information:

-	PDU Session Type.

-	DNN.

-	UE IP address/Prefix.

-	UP path change: a notification corresponding to this event is sent when the UE IP address / Prefix and / or DNAI and /or the N6 traffic routing information has changed.

The event notification may contain following information:

-	the type of notification ("EARLY" or "LATE").

-	for both the source and target UP path between the UE and the DN, the corresponding information is provided when it has changed:

-	DNAI.

-	UE IP address / Prefix.

-	N6 traffic routing information.

NOTE 1:	UP path change notification, DNAI and N6 traffic routing information are further described in TS 23.501 [2] clause 5.6.7.

-	QoS Monitoring for URLLC: the event notification may contain the QoS Monitoring report as described in clause 5.33.3.2 of TS 23.501 [2].

-	Change of Access Type; The event notification contains the new Access Type for the PDU Session.

-	PLMN change; The event notification contains the new PLMN Identifier for the PDU Session.

-	Downlink data delivery status. The event notification contains the status of downlink data buffering in the core network including:

-	First downlink packet per source of the downlink IP traffic in extended buffering and Estimated maximum wait time.

-	First downlink packet per source of the downlink IP traffic discarded.

-	First downlink packet per source of the downlink IP traffic transmitted after previous buffering and/or discarding of corresponding packet(s).

-	QFI allocation: The event notification is sent when a new QoS flow is established within a PDU session and contains:

-	If the Target of Event Reporting is a PDU session, both the allocated QFI and either one of the following (Application Identifier or IP Packet Filter Set or Ethernet Packet Filter Set). The DNN, S-NSSAI corresponding to the PDU session are also sent.

-	If the Target of Event Reporting is a SUPI, both the allocated QFI and either one of the following (Application Identifier or IP Packet Filter Set or Ethernet Packet Filter Set) for each PDU session ID established for this SUPI. The DNN, S-NSSAI corresponding to each PDU session are also sent.

-	If the Target of Event Reporting is an Internal-Group-Id or any UE, multiple instances of the tuple (allocated QFI and either one of the following (Application Identifier or IP Packet Filter Set or Ethernet Packet Filter Set). PDU session ID, SUPI). The DNN, S-NSSAI corresponding to each PDU session are also sent.

-	Session Management transaction count information.

When the consumer NF is the NWDAF, the event QFI allocation is used to collect data for Observed Service Experience analytics and UE communication analytics as specified in TS 23.288 [50]. The session management transaction count is used to collect UEs or group of UEs’ transaction count data for Dispersion Analytics as specified in TS 23.288 [50]. The transaction count is incremented when the transactions include messages from PDU Session Establishment, PDU Session Authentication, PDU Session Modification and PDU Session Release procedures.

Event Filters are used to specify the conditions to match for notifying the events (i.e. "List of Parameter values to match"). If there are no conditions to match for a specific Event ID, then the Event Filter is not provided. The following table provides as an example how the conditions to match for event reporting can be specified for various Event IDs for SMF exposure.

Table 5.2.8.3.1-1: Example of Event Filters for SMF exposure events

| Event ID for SMF exposure | Event Filter (List of Parameter Values to Match) |
| --- | --- |
| DNAI Change | None |
| PDU Session Release | None |
| PDU Session Establishment |  |
| QoS Monitoring for URLLC | None |
| QFI allocation | <Parameter Type = DNN, Value = DNN1> <Parameter Type = S-NSSAI, Value = S-NSSAI1> |
| QFI allocation | <Parameter Type = Application Identifier, Value = Application Identifier1> |
| Transaction Count | ,<Parameter Type = TAI, Value = TA1> (NOTE) <Parameter Type = Cell Id, Value Cell Id_1> (NOTE) <Parameter Type = S-NSSAI, Value = S-NSSAI1> |

NOTE: Optionally the SMF may fetch the location information from the AMF but transaction information correlation at the location can also be achieved without it and through transaction information associated with the requested time period, which corresponds to the UE’s time span at the location of interest.

The target of SMF event reporting may correspond to a PDU Session ID, an UE ID (SUPI) an Internal Group Identifier or an indication that any UE is targeted (e.g. on a specific DNN).

When acknowledgment is expected the SMF also provides Notification Correlation Information to the consumer NF in the event notification.

The consumer NF may provide the following event-specific information when acknowledging an event notification:

-	For UP path change event:

-	N6 traffic routing information related to the target DNAI.

NOTE 2:	Acknowledgement to a UP path change event notification is further described in TS 23.501 [2] clause 5.6.7.

* * * * End of  Changes * * * *