3GPP TSG-WG SA2 Meeting #143E e-meeting 	S2-2100479r04

Elbonia, February 24 – March 09, 2021	(revision of S2-210xxxx)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0218 | rev |  | Current version: | 16.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | ML model sharing between NWDAF instances |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_Ph2 |  |  |  |  |  | Date: |  |  | 2021-02-18 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | TR23.700-91 clause 8.19 agrees that For the Key Issue #19, the following services are proposed for the normative work. - Nnwdaf_MLModelProvision is a new service introduced to share trained ML models between NWDAF instances. - Nnwdaf_MLModelInfo is a new service introduced to request parameter(s) related to a ML model. In addition, NWDAF instances can discover NWDAF instances providing ML models via NRF. This contribution capatures the agreed new services and related service procedures in TS 23.288. Meanwhile, some service parameters are further aligned with the existing terminologies in TS 23.288. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add the description and eference to TS 23.501 regarding NWDAF discovery based on ML model in clause 5.2 - Add the general description and procedure for ML model provision in clause 6.2x - Add the description of Nnwdaf_MLModelProvision and Nnwdaf_MLModelInfo service in clause 7.1, 7.y and 7.z |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The conclusion of KI#19 in the study phase not captured. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2, 6.2x (new), 7.1, 7.y (new), 7.z (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

5.2	NWDAF Discovery and Selection

The NWDAF service consumer selects an NWDAF that supports requested analytics information and/or requested Machine Learning (ML) model information  by using the NWDAF discovery principles defined in clause 6.3.13, TS 23.501 [2].

# * * * * Second change * * * *

7.1	General

The following table illustrates the NWDAF Services.

Table 7.1-1: NF services provided by NWDAF

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Nnwdaf_AnalyticsSubscription | Subscribe | Subscribe / Notify | PCF, NSSF, AMF, SMF, NEF, AF, OAM, CEF |
|  | Unsubscribe |  | PCF, NSSF, AMF, SMF, NEF, AF, OAM, CEF |
|  | Notify |  | PCF, NSSF, AMF, SMF, NEF, AF, OAM, CEF |
| Nnwdaf_AnalyticsInfo | Request | Request / Response | PCF, NSSF, AMF, SMF, NEF, AF, OAM, CEF |
| Nnwdaf_MLModelProvision | Subscribe | Subscribe / Notify | NWDAF |
|  | Unsubscribe |  | NWDAF |
|  | Notify |  | NWDAF |
| NOTE 1: How OAM consumes Nnwdaf services and which Analytics information is relevant is defined in TS 28.550 [7] Annex H. and out of the scope of this TS. NOTE 2: How CEF consumes Nnwdaf services and which Analytics information is relevant is defined in TS 28.201 [21] and out of the scope of this TS. NOTE x: The Nnwdaf_MLModelProvision service and the Nnwdaf_MLModelInfo service are provided by an NWDAF containing MTLF and consumed by an NWDAF (i.e. NWDAF (AnLF), NWDAF (MTLF) or NWDAF (AnLF+MTLF)). |  |  |  |

# * * * * Third change * * * *

7.y	Nnwdaf_MLModelProvision services

7.y.1	General

Service Description: This service enables the consumer to receive a notification when a ML model matching the subscription parameters becomes available.

When the subscription is accepted by the NWDAF, the consumer NF receives from the NWDAF an identifier (Subscription Correlation ID) allowing to further manage (modify, delete) this subscription. The modification of ML model subscription can be enforced by NWDAF based on operator policy and configuration.

7.y.2	Nnwdaf_MLModelProvision_Subscribe service operation

Service operation name: Nnwdaf_MLModelProvision_Subscribe.

Description: Subscribes to NWDAF ML model provision with specific parameters.

Inputs, Required: (set of) Analytics ID(s) defined in Table 7.1-2, Notification Target Address (+ Notification Correlation ID).

Inputs, Optional: Subscription Correlation ID (in the case of modification of the ML model subscription).

Outputs Required: When the subscription is accepted: Subscription Correlation ID (required for management of this subscription).

Outputs, Optional: None.

7.y.3	Nnwdaf_MLModelProvision_Unsubscribe service operation

Service operation name: Nnwdaf_MLModelProvision_Unsubscribe.

Description: unsubscribe to NWDAF ML model provision

Inputs, Required: Subscription Correlation ID.

Inputs, Optional: None.

Outputs, Required: Operation execution result indication.

Outputs, Optional: None.

7.y.4	Nnwdaf_MLModelProvision_Notify service operation

Service operation name: Nnwdaf_MLModelProvision_Notify.

Description: NWDAF notifies the ML model information to the consumer instance which has subscribed to the specific NWDAF service.

Inputs, Required: Set of the tuple (Analytics ID, address (e.g. URL or FQDN) of Model file), Notification Correlation Information.

Inputs, Optional: None.

Outputs, Required: Operation execution result indication.

Outputs, Optional: None.

# * * * * Fourth change * * * *

Editor's note:	It is FFS to define the Nnwdaf_MLModelInfo service i.e. a request-response mode to request the ML Model.

# * * * * End of changes * * * *