SA WG2 Meeting #144E	S2-210eeee

Elbonia, April 12 – 16, 2021

Source:	vivo

Title:	Updates to Procedures for Service Authorization and Provisioning to UE with relay

Document for:	Approval

Agenda Item:	8.8.2

Work Item / Release:	5G_ProSe / Rel-17

Abstract of the contribution: This contribution updates.

# Discussion

- .

# Proposal

It is proposed to include the following changes in TS 23.304.

* * * * Start of the 1st Change * * * *

## 6.2	Procedures for Service Authorization and Provisioning to UE

### 6.2.1	General

The procedures for service authorization and provisioning to UE may be initiated by the PCF (as described in clause 6.2.2), by the UE (as described in clause 6.2.4), or by the AF (as described in clause 6.2.5).

### 6.2.2	PCF based Service Authorization and Provisioning to UE

For PCF based Service Authorization and Provisioning to UE, the Registration procedures as defined in clause 4.2.2.2 of TS 23.502 [5], UE Policy Association Establishment procedure as defined in clause 4.16.11 of TS 23.502 [5] and UE Policy Association Modification procedure as defined in clause 4.16.12 of TS 23.502 [5] apply with the following additions:

-	If the UE indicates ProSe capability in the Registration Request message and if the UE is authorized to use ProSe service based on subscription data, the AMF selects the PCF which supports ProSe Policy/Parameter provisioning as described in clause 6.2.3 and establishes a UE policy association with the PCF for ProSe Policy/Parameter delivery.

-	For UE-to-Network relay case, the ProSe Capability includes the capability of as a Layer-2 Remote UE/UE-to-Network Relay, as a Layer-3 Remote UE/UE-to-Network Relay or both to indicate whether the UE is capable for ProSe service of Layer-2 UE-to-Network relay, Layer-3 UE-to-Network relay or both, respectively.

-	If the UE supports ProSe Direct Discovery, ProSe Direct Communication and/or UE-to-Network relay and it does not have valid ProSe policy/parameters, the UE includes the UE Policy Container with indicating the ProSe Policy Provisioning Request during registration procedure.

-	If the UE indicates the ProSe Policy Provisioning Request in the UE Policy Container, the PCF determines whether to provision ProSe Policy/parameters for ProSe Direct Discovery, ProSe Direct Communication and/or UE-to-Network relay to the UE, as specified in clause 6.1.2.2.2 of TS 23.503 [9], and the PCF provides the ProSe Policy/parameters (see clause 5.1.2.1 and clause 5.1.3.1) to the UE by using the procedure as defined in clause 4.2.4.3 "UE Configuration Update procedure for transparent UE Policy Delivery" in TS 23.502 [5].

The PCF may update the ProSe Policy/parameters to the UE in following conditions:

-	UE Mobility, e.g. UE moves from one PLMN to another PLMN. This is achieved by using the procedure of UE Policy Association Modification initiated by the AMF, as defined in clause 4.16.12.1 of TS 23.502 [5].

-	When there is a subscription change in the list of PLMNs where the UE is authorized to perform ProSe Direct Discovery, ProSe Direct Communication or and/or UE-to-Network relay over PC5 reference point. This is achieved by using UE Policy Association Modification initiated by the PCF procedure as defined in clause 4.16.12.2 of TS 23.502 [5].

-	When there is a change of service specific parameter (including path selection policy) as described in clause 6.2.5 (performing the procedure in clause 4.15.6.7 of TS 23.502 [5]).

If the serving PLMN is removed from the list of PLMNs in the service authorization parameters, the service authorization is revoked in the UE.

When the UE is roaming, the change of subscription resulting in updates of the service authorization parameters are transferred to the UE by H-PCF via V-PCF.

The UE may perform UE triggered Policy Provisioning procedure to the PCF as specified in clause 6.2.4 when the UE determines the ProSe Policy/Parameter is invalid (e.g. Policy/Parameter is outdated, missing or invalid).

### 6.2.3	PCF discovery

PCF discovery and selection mechanism defined in clause 6.3.7.1 of TS 23.501 [4] applies with the following addition to enable a PCF instance is selected for ProSe service and for UE:

-	Based on the indication from the UE and/or UE subscription data during the Registration procedure as specified in clause 6.5.2, the AMF may include the ProSe capability indication in the Nnrf_NFDiscovery_Request message as the optional input parameter. If provided, the NRF takes the information into account for discovering the PCF instance.

### 6.2.4	Procedure for UE triggered ProSe Policy provisioning

The UE triggered Policy Provisioning procedure is initiated by the UE to request ProSe Policy/Parameter from the PCF when UE determines the ProSe Policy/Parameter is invalid in the following cases:

-	if the validity timer indicated in the ProSe Policy/Parameter expires;

-	if there are no valid parameters, e.g. for the ProSe service type a UE wants to use, for current area, or due to abnormal situation.

Figure 6.2.4-1: UE triggered ProSe Policy provisioning procedure

1.	The UE sends the UE Policy provisioning request including the UE Policy Container (ProSe Policy Provisioning Request) to the AMF.

2.	The AMF sends the Npcf_UEPolicyControl_Update request to the PCF including the UE Policy Container received from UE.

3.	The UE Policy delivery procedure defined in clause 4.2.4.3 of TS 23.502 [5] is triggered.

### 6.2.5	AF-based service parameter provisioning for ProSe over control plane

For ProSe service parameter provisioning, the procedure defined in clause 4.15.6.7 of TS 23.502 [5] is performed with the following considerations:

-	The AF in TS 23.502 [5] is considered as ProSe Application Server in this specification.

-	Service Description indicates ProSe service domain information.

-	Service Parameters include parameters for ProSe Direct Discovery,ProSe Direct Communications and and/or UE-to-Network relay. The detailed information on the parameters is described in clause 5.1.2.1 and clause 5.1.3.1.

NOTE:	It is assumed that the ProSe service domain information is set based on the Service Level Agreement with the operator.

* * * * End of Changes * * * *