3GPP TSG-SA WG2#144E e-meeting	S2-210xxxx

Elbonia, 12 – 16 April 2021	(was S2-200xxxx)

Source:	Qualcomm Incorporated, Ericsson?

Title:	TS 23.247 MBS session ID and TMGI

Document for:	Approval

Agenda Item:	8.9

Work Item / Release:	5MBS

Abstract of the contribution: this contribution introduces the architecture and the solution for 5MBS-eMBMS interworking as per conclusions of TR 23.757.

# 1.	Discussion

The current text in 6.5.1/2 regarding MBS session ID and TMGI does not reflect all the agreements and modifications performed at SA2#143E.

In particular:

- Detail on for which procedure source IP multicast address applies, and the assignment of TMGI for internal signalling.

- How the MBS session is identified in SNPN, i.e. by including NID.

# 2.	Text proposal

It is proposed to agree the following changes vs. TS 23.xxx:

### 6.5.1	MBS Session ID

The MBS session ID is used to identify a MBS Multicast/Broadcast Session throughout the 5G system transport on external interface towards AF and between AF and UE, and towards the UE.

MBS Session ID may have the following types:

-	TMGI (for MBS broadcast and MBS multicast Session),

-	source specific IP multicast address (for MBS multicast Session).

If an MBS multicast session is provided within an SNPN, the MBS multicast session can still be identified by a (globally unique) source specific IP multicast address or TMGI. In 5GS internal signalling the PLMN ID, included in TMGI, is complemented with the NID to identify an SNPN.

Source specific IP multicast address or TMGI may be used as MBS Session ID in NAS messages exchange between a UE and a CN when the UE requests to join/leave a multicast session.

For MBS multicast sessions that the UE joined with a source specific IP multicast address, a TMGI is also allocated by 5GC and is sent to the UE and used in other signalling messages between NG-RAN, CN and UE.

The UE shall be able to obtain at least one MBS Session ID via MBS service announcement.

For MBS multicast Session, a source specific IP multicast address can be assigned by 5GC or an external network.

### 6.5.2	Temporary Mobile Group Identity

TMGI (Temporary Mobile Group Identity) is defined in TS 23.003 [12] and is used to identify a broadcast MBS Session or a multicast MBS Session. The TMGI is also used to identify the Broadcast service context and Multicast service context at MBSF..

In SNPN (Stand-alone Non-Public Network), TMGI is used together with NID (Network Identifier) in TS 23.003 [12] together identify an MBS Session.

>>>>END CHANGES<<<<