SA WG2 Meeting #144E	S2-210xxxx

12 April – 16 April, Electronic, Elbonia

Source:	Ericsson

Title:	New Clause, MBS Session Delivery Status Notification for Broadcast

Document for:	Approval

Agenda Item:	8.9

Work Item / Release:	5MBS/Rel-17

Abstract of the contribution: This paper introduces a new clause 7.3.X about MBS Session Delivery Status Notification for Broadcast

# 1. 	Discussion

In SA2#143E, in S2-2100349, a clause “MBS Delivery Status Notification for Broadcast” was proposed, however, , the clausewas removed during the revision(see S2-2100349r01).

In eMBMS, GCS AS may receive notification events from BM-SC, below are examples from TS 29.468:

- Bearer Terminated: It is essential to be implemented in 5GS. If the MBS Session is terminated in 5GS, AF needs to be notified, so that AF can take further actions (e.g. switch to unicast).

- Bearer Activated: In case the broadcast session start is not a synchronized operation (e.g. the AF request MB-SMF to start broadcast session at a future time), this event can bring a clear indication towards AF about the session status, so that AF can start content delivery.

It is necessary in our view to keep feature parity in 5MBS with eMBMS to maintain the same level of functionality for broadcast communication.

[Proposal-1] Introduce MBS Delivery Status Notification, so that 5GS can inform AF the session status change in time, and then AF may take proper actions.

# 2	Proposals

It’s proposed to update TR 23.247 v0.1.0 as follows.

* * * Start of change* * *

### 7.3.X	MBS Session Delivery Status Indication for Broadcast

The MBS Session Delivery Status Indication for broadcast is used by the MB-SMF to notify the AF of conditions affecting the delivery of the MBS session (e.g. MBS session activated, MBS session terminated, etc.). The occurrence of the indicated condition may have been detected at the MB-SMF or may have been reported to the MB-SMF by other entities involved in the MBS session delivery.

Figure 7.3.X-1: MBS Session Delivery Status Indication for Broadcast

- The MB-SMF sends Delivery Status Indication to NEF/MBSF with the TMGI and the event.

- The NEF/MBSF forward the Delivery Status Indication to the AF.

Depending on how the requests of MBS Session Establishment are sent from the AF to the MB-SMF, the Delivery Status Indication shall be delivered in the reverse path. The Delivery Status Indication may be delivered from the MB-SMF to the AF directly, via NEF, via MBSF, or via MBSF and NEF.

Editor’s note: whether more events are needed in the delivery status notification are FFS.

* * * End of change* * *