| 3GPP TS 23.304 V0.2.0 (2021-04) |  |
| --- | --- |
| Technical Specification |  |
| 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Proximity based Services (ProSe) in the 5G System (5GS) (Release 17) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2021, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	6

1	Scope	8

2	References	8

3	Definitions of terms, symbols and abbreviations	9

3.1	Terms	9

3.2	Abbreviations	10

4	Architecture model and concepts	11

4.1	General concept	11

4.2	Architectural reference model	11

4.2.1	Non-roaming reference architecture	11

4.2.2	Roaming reference architecture	12

4.2.3	Inter-PLMN reference architecture	13

4.2.4	AF-based service parameter provisioning	15

4.2.5	Reference points	16

4.2.6	Service-based interfaces	16

4.2.7	UE-to-Network Relay reference architecture	17

4.2.7.1	Layer-3 UE-to-Network Relay reference architecture	17

4.2.7.2	Layer-2 UE-to-Network Relay reference architecture	19

4.3	Functional Entities	20

4.3.1	UE	20

4.3.2	5G DDNMF	21

4.3.2.1	General	21

4.3.2.2	5G DDNMF Discovery	21

4.3.3	PCF	21

4.3.4	AMF	22

4.3.5	UDM	22

4.3.6	UDR	22

4.3.7	NRF	22

4.3.8	ProSe Application Server	22

4.3.9	ProSe UE-to-Network Relay	23

4.3.9.1	Layer-3 UE-to-Network Relay	23

4.3.10	SMF	23

5	High level functionality and features	24

5.1	Authorization and Provisioning for ProSe service	24

5.1.1	General	24

5.1.2	Authorization and Provisioning for ProSe Direct Discovery	25

5.1.2.1	Policy/Parameter provisioning for ProSe Direct Discovery	25

5.1.2.2	Principles for applying parameters for ProSe Direct Discovery	27

5.1.3	Authorization and Provisioning for ProSe Direct Communication	28

5.1.3.1	Policy/Parameter provisioning for ProSe Direct Communication	28

5.1.3.2	Principles for applying parameters for ProSe Direct Communication	29

5.1.4	Authorization and Provisioning for ProSe UE-to-Network Relay	31

5.1.4.1	Policy/Parameter provisioning for ProSe UE-to-Network Relay	31

5.1.4.2	Principles for applying parameters for ProSe UE-to-Network Relay	32

5.1.4.3	Principles for Network controlled Remote authorization for ProSe UE-to-Network Relay	32

5.1.5	Authorization and Provisioning for Prose UE-to-UE Relay	32

5.1.5.1	Policy/Parameter provisioning for Prose UE-to-UE Relay	32

5.1.5.2	Principles for applying parameters for Prose UE-to-UE Relay	32

5.2	ProSe Direct Discovery	32

5.3	ProSe Direct Communication	33

5.3.1	General	33

5.3.2	Broadcast mode ProSe Direct Communication	34

5.3.3	Groupcast mode ProSe Direct Communication	34

5.3.4	Unicast mode ProSe Direct Communication	34

5.4	ProSe UE-to-Network Relay	35

5.4.1	Layer-3 UE-to-Network Relay	35

5.4.1.1	General	35

5.4.1.2	Layer-3 UE-to-Network Relay with N3IWF support	35

5.4.2	Layer-2 UE-to-Network Relay	35

5.5	ProSe UE-to-UE Relay	36

5.6	IP address allocation	36

5.7	QoS handling	36

5.7.1	QoS handling for ProSe Direct Communication	36

5.7.2	QoS handling for Relay operations	37

5.7.2.1	QoS handling for Layer-3 UE-to-Network Relay without N3IWF	37

5.7.2.2	QoS handling for Layer-3 UE-to-Network relay with N3IWF	38

5.7.2.3	QoS handling for Layer-2 UE-to-Network Relay	40

5.8	Subscription to ProSe	40

5.9	Identifiers	41

5.9.1	Identifiers for ProSe Direct Discovery	41

5.9.1.0	General	41

5.9.1.1	ProSe Application ID	41

5.9.1.2	Destination Layer 2 ID	41

5.9.1.3	Source Layer 2 ID	41

5.9.1.4	Discovery Group ID	41

5.9.1.5	ProSe Application Code	41

5.9.1.6	ProSe Restricted Code	41

5.9.1.7	ProSe Query Code	41

5.9.1.8	ProSe Response Code	41

5.9.1.9	User Info ID	41

5.9.1.10	ProSe Discovery UE ID	41

5.9.1.11	Restricted ProSe Application User ID	42

5.9.1.12	Announcing PLMN ID	42

5.9.2	Identifiers for ProSe Direct Communication	42

5.9.2.1	General	42

5.9.2.2	Identifiers for broadcast mode ProSe direct communication	42

5.9.2.3	Identifiers for groupcast mode ProSe direct communication	42

5.9.2.4	Identifiers for unicast mode ProSe direct communication	42

5.9.3	Identifiers for ProSe UE-to-Network Relay	43

5.9.3.1	Common identifiers for UE-to-Network Relay	43

5.9.3.2	Identifiers for Layer-3 UE-to-Network Relay	43

5.9.3.3	Identifiers for Layer-2 UE-to-Network Relay	44

5.9.4	Identifiers for ProSe UE-to-UE Relay	44

5.10	Support for ProSe for UEs in limited service state	44

5.11	PC5 operation in EPS for Public Safety UE	44

5.12	Communication path selection between PC5 and Uu reference points	45

6	Functional description and information flows	45

6.1	Control and user plane stacks	45

6.1.1	Control Plane	45

6.1.1.1	General	45

6.1.1.2	UE - UE	45

6.1.1.2.1	Discovery plane PC5 interface	45

6.1.1.2.2	PC5 Signalling Protocol	46

6.1.1.3	UE - 5G DDNMF	46

6.1.1.4	5G DDNMF – UDM	46

6.1.1.5	5G DDNMF – 5G DDNMF	47

6.1.1.6	5G DDNMF – ProSe Application Server	47

6.1.1.7	UE - UE-to-Network Relay	47

6.1.1.7.1	Layer-3 UE-to-Network Relay	47

6.1.1.7.2	Layer-2 UE-to-Network Relay	48

6.1.1.8	UE - UE-to-UE Relay	49

6.1.2	User Plane	49

6.1.2.1	UE - UE	49

6.1.2.2	UE - UE-to-Network Relay	49

6.1.2.2.1	Layer-3 UE-to-Network Relay	49

6.1.2.2.2	Layer-2 UE-to-Network Relay	50

6.1.2.3	UE - UE-to-UE Relay	50

6.2	Procedures for Service Authorization and Provisioning to UE	50

6.2.1	General	50

6.2.2	PCF based Service Authorization and Provisioning to UE	51

6.2.3	PCF discovery	51

6.2.4	Procedure for UE triggered ProSe Policy provisioning	52

6.2.5	AF-based service parameter provisioning for ProSe over control plane	52

6.3	ProSe Direct Discovery	52

6.3.1	ProSe Direct Discovery with 5G DDNMF	52

6.3.1.1	Overview	52

6.3.1.2	Overall procedure for ProSe Direct Discovery (Model A)	53

6.3.1.3	Overall procedure for ProSe Direct Discovery (Model B)	54

6.3.2	ProSe Direct Discovery procedures over PC5 reference point	56

6.3.2.1	General	56

6.3.2.2	Group Member Discovery	58

6.3.2.2.1	General	58

6.3.2.2.2	Procedure for Group Member Discovery with Model A	58

6.3.2.2.3	Procedure for Group Member Discovery with Model B	58

6.3.2.3	UE-to-Network Relay Discovery	59

6.3.2.3.1	General	59

6.3.2.3.2	Procedure for UE-to-Network Relay Discovery with Model A	59

6.3.2.4	UE-to-UE Relay Discovery	60

6.4	ProSe Direct Communication	61

6.4.1	Broadcast mode ProSe Direct Communication	61

6.4.2	Groupcast mode ProSe Direct Communication	61

6.4.3	Unicast mode ProSe Direct Communication	63

6.4.3.1	Layer-2 link establishment over PC5 reference point	63

6.4.3.2	Link identifier update for a unicast link	67

6.4.3.3	Layer-2 link release over PC5 reference point	68

6.4.3.4	Layer-2 link modification for a unicast link	69

6.4.3.5	Layer-2 link maintenance over PC5 reference point	70

6.5	ProSe UE-to-Network Relay Communication	71

6.5.1	ProSe Communication via Layer-3 UE-to-Network Relay	71

6.5.2	ProSe Communication via Layer-2 UE-to-Network Relay	71

6.6	ProSe UE-to-UE Relay Communication	73

6.7	Procedures for Service Authorization to NG-RAN	73

6.7.1	General	73

6.7.2	Registration procedure	73

6.7.3	Service Request procedure	74

6.7.4	N2 Handover procedure	74

6.7.5	Xn Handover procedure	74

6.7.6	Subscriber Data Update Notification to AMF	74

6.7.7	Delivery of PC5 QoS parameters for ProSe to NG-RAN	74

7	Network Function Services	75

7.1	5G DDNMF Services	75

7.1.1	General	75

7.1.2	N5g-ddnmf_Discovery service	75

7.1.2.1	General	75

7.1.2.2	N5g-ddnmf_Discovery_AnnounceAuthorize service operation	75

7.1.2.3	N5g-ddnmf_Discovery_AnnounceUpdate service operation	75

7.1.2.4	N5g-ddnmf_Discovery_MonitorAuthorize service operation	76

7.1.2.5	N5g-ddnmf_Discovery_MonitorUpdate service operation	76

7.1.2.6	N5g-ddnmf_Discovery_MonitorUpdateResult service operation	76

7.1.2.7	N5g-ddnmf_Discovery_DiscoveryAuthorize service operation	76

7.1.2.8	N5g-ddnmf_Discovery_MatchReport service operation	77

7.1.2.9	N5g-ddnmf_Discovery_MatchInformation service operation	77

Annex A (informative): Change history	78

Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall	indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should	indicates a recommendation to do something

should not	indicates a recommendation not to do something

may	indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can	indicates that something is possible

cannot	indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will	indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not	indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

1	Scope

The present document specifies the Stage 2 of the Proximity based Services (ProSe) features in 5GS. ProSe features consist of: ProSe discovery, ProSe Direct Communication and ProSe UE-to-Network Relay.

ProSe discovery identifies that specific ProSe-enabled UEs are in proximity.

ProSe Direct Communication enables establishment of communication paths between two or more ProSe-enabled UEs that are in direct communication range.

ProSe UE-to-Network Relay enables relay communication between NG-RAN and UEs (e.g. for UEs that are out of coverage of the network).

Editor's note: Security aspects of ProSe are to be defined by SA3. The reference to SA3 specification will be added when defined in SA3.

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.287: "Architecture enhancements for 5G System (5GS) to support Vehicle-to-Everything (V2X) services".

[3]	3GPP TS 23.303: "Proximity-based services (ProSe); Stage 2".

[4]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[5]	3GPP TS 23.502: "Procedures for the 5G System (5GS); Stage 2".

[6]	3GPP TS 22.261: "Service requirements for next generation new services and markets; Stage 1".

[7]	3GPP TS 22.278: "Service requirements for the Evolved Packet System (EPS)".

[8]	3GPP TS 23.288: "Architecture enhancements for 5G System (5GS) to support network data analytics services".

[9]	3GPP TS 23.503: "Policy and Charging Control Framework for the 5G System".

[10]	3GPP TS 29.345: "Inter-Proximity-services (Prose) Function signalling aspects; Stage 3".

[11]	3GPP TS 36.300: "Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Overall description; Stage 2".

[12]	3GPP TS 38.300: "NR; NR and NG-RAN Overall Description; Stage 2".

[13]	3GPP TS 38.304: "NR; User Equipment (UE) procedures in idle mode".

[14]	3GPP TS 23.122: "Non-Access-Stratum (NAS) functions related to Mobile Station in idle mode".

[15]	3GPP TS 36.331: "Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Resource Control (RRC); Protocol specification".

[16]	3GPP TS 38.331: "NR; Radio Resource Control (RRC); Protocol Specification".

[17]	IETF RFC 4862: "IPv6 Stateless Address Autoconfiguration".

[18]	IETF RFC 3927: "Dynamic Configuration of IPv4 Link-Local Addresses".

[19]	IETF RFC 826: "An Ethernet Address Resolution Protocol".

[20]	IETF RFC 4960: "Stream Control Transmission Protocol".

[21]	3GPP TR 23.752: "Study on system enhancement for Proximity based Services (ProSe) in the 5G System (5GS)".

[22]	3GPP TS 32.277: "Proximity-based Services (ProSe) charging".

[23]	3GPP TS 24.554: "Proximity-services (ProSe) in 5G System (5GS) protocol aspects; Stage 3".

3	Definitions of terms, symbols and abbreviations

3.1	Terms

For the purposes of the present document, the terms given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

Application Layer ID: An identity identifying a ProSe-enabled UE within the context of a specific application. The format of this identifier is outside the scope of 3GPP.

Direct Network Communication: One mode of network communication, where there is no ProSe UE-to-Network Relay between a UE and the 5G network.

Indirect Network Communication: One mode of network communication, where there is a ProSe UE-to-Network Relay between a UE and the 5G network.

Member ID: An identifier uniquely identifying a member in the Application Layer managed group and that is managed by the ProSe application layer.

Mode of communication: Mode of communication to be used by the UE over PC5 reference point, i.e. broadcast mode, groupcast mode or unicast mode.

Open ProSe Discovery: ProSe Direct Discovery without explicit permission from the ProSe-enabled UE being discovered, according to TS 22.278 [7].

ProSe Direct Discovery: A procedure employed by a ProSe-enabled UE to discover other ProSe-enabled UEs in its vicinity based on direct radio transmissions between the two UEs with NR technology.

ProSe Direct Communication: A communication between two or more UEs in proximity that are ProSe-enabled, by means of user plane transmission using NR technology via a path not traversing any network node.

ProSe UE-to-Network Relay: A ProSe-enabled UE that provides functionality to support connectivity to the network for Remote UE(s).

Remote UE: A ProSe-enabled UE that communicates with a DN via a ProSe UE-to-Network Relay.

Restricted ProSe Discovery: ProSe Direct Discovery that only takes place with explicit permission from the ProSe-enabled UE being discovered, according to TS 22.278 [7].

For the purposes of the present document, the following term and definition given in TS 23.303 [3] apply:

Application Layer Group ID

Destination Layer-2 ID

Discovery Entry ID

Discovery Filter

Discovery Group ID

Discovery Query Filter

Discovery Response Filter

Geographical Area

Local PLMN

Model A

Model B

Metadata Index

Metadata Index Mask

ProSe Application ID

ProSe Application Code

ProSe Application Mask

ProSe Query Code

ProSe Response Code

ProSe Restricted Code

ProSe Restricted Code Prefix

ProSe Restricted Code Suffix

ProSe Discovery UE ID

ProSe Layer-2 Group ID

ProSe enabled UE

Restricted ProSe Application User ID

Source Layer-2 ID

User Info ID

3.2	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

AS layer	Access Stratum layer

ProSe	Proximity based Services

RPAUID	Restricted ProSe Application User ID

PDUID	ProSe Discovery UE ID

PFI	PC5 QoS Flow Identifier

PQI	PC5 5QI

4	Architecture model and concepts

4.1	General concept

Proximity based Services (ProSe) are services that can be provided by the 5GS based on UEs being in proximity to each other.

The 5GS enablers for ProSe include the following functions:

-	ProSe Direct Discovery;

-	ProSe Direct Communication;

-	ProSe UE-to-Network Relay.

4.2	Architectural reference model

4.2.1	Non-roaming reference architecture

Figure 4.2.1-1 shows the high-level view of the non-roaming 5G System architecture for Proximity-based Services (ProSe). In this figure, UE A and UE B use a subscription of the same PLMN.

Figure 4.2.1-1: Non-roaming 5G System architecture for Proximity-based Services

Figure 4.2.1-2 shows the high-level view of the non-roaming 5G System architecture for Proximity-based Services (ProSe) in reference point representation. In this figure, UE A and UE B use a subscription of the same PLMN.

Figure 4.2.1-2: Non-roaming 5G System architecture for Proximity-based Services in reference point representation

4.2.2	Roaming reference architecture

Figure 4.2.2-1 show the high-level view of the roaming 5G System architecture for Proximity-based Services (ProSe). In the figure, UE A uses a subscription of HPLMN.

Figure 4.2.2-1: roaming 5G System architecture for Proximity-based Services

4.2.3	Inter-PLMN reference architecture

The following figure 4.2.3-1 show the high level view of the non-roaming inter-PLMN architecture. In this figure, PLMN A is the HPLMN of UE A and PLMN B is the HPLMN of UE B.

Figure 4.2.3-1: non-roaming Inter-PLMN 5G System architecture for Proximity-based Services

Figure 4.2.3-2 shows the high level view of the roaming architecture. In this figure, UE A uses a subscription of PLMN A and UE B uses a subscription of PLMN B; UE A is roaming in PLMN C while UE B is not roaming.

Figure 4.2.3-2: roaming Inter-PLMN 5G System architecture for Proximity-based Services

### 4.2.4	AF-based service parameter provisioning

The 5G System provides NEF services to enable communication between NFs in the PLMN and a ProSe Application Server. Figure 4.2.4-1 shows the high level view of AF-based service parameter provisioning for ProSe communications. The ProSe Application Server may provide ProSe service parameters to the PLMN via NEF. The NEF stores the ProSe service parameters in the UDR.

Figure 4.2.4-1: 5G System architecture for AF-based service parameter provisioning for ProSe communications

### 4.2.5	Reference points

PC1:	The reference point between the ProSe application in the UE and in the ProSe Application Server. It is used to define application level signalling requirements. This reference point is not specified in this release of the specification.

PC3a:	The reference point between the UE and the 5G DDNMF. PC3a relies on 5GC user plane for transport (i.e. an "over IP" reference point). It is used to authorise ProSe Direct Discovery request, and perform allocation of ProSe Application Codes / ProSe Restricted Codes corresponding to ProSe Application Identities used for ProSe Direct Discovery.

PC5:	The reference point between ProSe-enabled UEs used for control and user plane for ProSe Direct Discovery, ProSe Direct Communication and ProSe UE-to-Network Relay.

Nxx:	The reference point between the ProSe Application Server and the 5G DDNMF. It is used to define the interaction between ProSe Application Server and 5G DDNMF for ProSe Direct Discovery.

N64:	The reference point between the UDM and 5G DDNMF. It is used to provide subscription information in order to authorise ProSe Direct Discovery request.

N65:	The reference point between the 5G DDNMF in the HPLMN and the 5G DDNMF in a Local PLMN (ProSe Direct Discovery). This reference point is used for HPLMN control of ProSe service authorization.

N66:	The reference point between the 5G DDNMF in the HPLMN and the 5G DDNMF in the VPLMN. It is used for HPLMN control of ProSe service authorization.

NOTE:	Nxx, N64, N65 and N66 show the interactions that exist between the NF services in the NFs. These reference points are realised by corresponding NF service-based interfaces and by specifying the identified consumer and producer NF service as well as their interaction in order to realize a particular system procedure.

### 4.2.6	Service-based interfaces

N5g-ddnmf:	Services provided by 5G DDNMF to manage inter-PLMN ProSe Direct Discovery operations.

In addition to the relevant services defined in TS 23.501 [4] for the following service-based interfaces, in the case of ProSe Service, the services can be provided by corresponding NF are as follows.

Nudm:	Services provided by UDM are used to get ProSe Service related subscription information to the AMF during Initial Registration procedure or UE Configuration Update (UCU) procedure to inform the AMF the subscription information has changed. The subscription information is described in TS 23.502 [5].

Npcf:	Services provided by H-PCF are used to provide ProSe Service related parameters to V-PCF for the UE and NG-RAN in the roaming case.

Nudr:	Services provided by UDR are used to notify the PCF and the UDM of the update of the ProSe Service related information as described in TS 23.502 [5].

Nnef:	Services provided by NEF are used by the ProSe Application Server to update ProSe Service related information of 5GC.

Namf:	Services provided by AMF are consumed by PCF to provide the ProSe Service related parameters for the UE and the NG-RAN to the AMF, and to enable the AMF create or update the UE context related to ProSe service.

Nnrf:	Services provided by NRF are used to discover the PCF that supports ProSe service and for 5G DDNMF in HPLMN to discover other 5G DDNMFs in VPLMN or local PLMN.

### 4.2.7	UE-to-Network Relay reference architecture

#### 4.2.7.1	Layer-3 UE-to-Network Relay reference architecture

The following figure 4.2.7.1-1 show the high level reference architecture for Layer-3 UE-to-Network Relay. In this figure, the Layer-3 UE-to-Network Relay may be in the HPLMN or a VPLMN.

Figure 4.2.7.1-1: Reference architecture for Layer-3 UE-to-Network Relay

The following figure 4.2.7.1-2 show the non-roaming reference architecture for Layer-3 UE-to-Network Relay when N3IWF is supported. In this figure, the PLMN A and PLMN B may be the same or different. When the Remote UE may connect to NG-RAN directly to access PLMN B, and in that case it would take the role of UE in the figure.

Figure 4.2.7.1-2: Non-roaming architecture model for Layer-3 UE-to-Network Relay with N3IWF support

The following figure 4.2.7.1-3 show the roaming reference architecture for Layer-3 UE-to-Network Relay. In this figure, the PLMN A and PLMN C may be the same or different.

Figure 4.2.7.1-3: Roaming architecture model for Layer-3 UE-to-Network Relay with N3IWF support

4.2.7.2	Layer-2 UE-to-Network Relay reference architecture

Figure 4.2.7.2-1 show the Layer-2 UE-to-Network Relay reference architecture. The Remote UE and ProSE UE-to-Network Relay may be served by the same or different PLMNs. If the serving PLMNs of the Remote UE and the ProSe UE-to-Network Relay are different then NG-RAN is shared by the serving PLMNs, see the 5G MOCN architecture in clause 5.18 of TS 23.501 [4].

Figure 4.2.7.2-1: Layer-2 UE-to-Network Relay reference architecture

NOTE 1:	Uu between the Remote UE and NG-RAN consists of RRC, SDAP and PDCP.

NOTE 2:	The Remote UE and ProSe UE-to-Network Relay are served by the same NG-RAN. The Core Network entities (e.g., AMF, SMF, UPF) serving the Remote UE and the ProSe UE-to-Network Relay can be the same or different.

4.3	Functional Entities

4.3.1	UE

Any ProSe-enabled UE may support the following functions:

-	Exchange of information for ProSe Direct Discovery between ProSe-enabled UE and the 5G DDNMF over PC3 reference point.

-	Procedures for ProSe Direct Discovery of other ProSe-enabled UEs over PC5 reference point.

-	Procedures for ProSe Direct Communication over PC5 reference point, including Broadcast, Groupcast and Unicast mode ProSe Direct Communication.

-	Procedures to act as a 5G ProSe Layer-2 UE-to-Network Relay.

-	Procedures to act as a 5G ProSe Layer-3 UE-to-Network Relay.

-	Procedures to act as a 5G ProSe Layer-2 Remote UE.

-	Procedures to act as a 5G ProSe Layer-3 Remote UE.

-	Indicate 5G ProSe Policy Provisioning Request in UE Policy Container for Registration Request or UE triggered 5G ProSe Policy provisioning, which may contain one or multiple capabilities as listed below:

-	5G ProSe UE for ProSe Direct Discovery;

-	5G ProSe UE for ProSe Direct Communication;

-	5G ProSe Layer-2 Remote UE;

-	5G ProSe Layer-3 Remote UE;

-	5G ProSe Layer-2 UE-to-Network Relay;

-	5G ProSe Layer-3 UE-to-Network Relay.

Editor’s Note:	It is to be determined whether the UE needs to indicate to the PCF support of relaying via N3IWF in addition to L3 relay capabilities.

-	Receive the 5G ProSe Policy from 5GC over N1 reference point.

-	Configuration of parameters for ProSe Direct Discovery and ProSe Direct Communication (e.g., including IP addresses, ProSe Layer-2 Group IDs, see clause 5.1). These parameters can be pre-configured in the UE, or, if in coverage, provisioned or updated by signalling over the N1 reference point from the PCF in the HPLMN or over PC1 reference point from the ProSe Application Server.

-	Report the following capabilities to 5GC over the N1 reference point:

-	5G ProSe Capability;

4.3.2	5G DDNMF

4.3.2.1	General

The 5G DDNMF is the logical function handling network related actions required for dynamic ProSe Direct Discovery. In this version of the specification, it is assumed that there is only one logical 5G DDNMF in each PLMN that supports ProSe Direct Discovery service.

NOTE:	If multiple 5G DDNMFs are deployed within the same PLMN (e.g., for load reasons), the method to locate the 5G DDNMF that has allocated a specific ProSe Application Code or ProSe Restricted Code (e.g. through a database lookup, etc.) is not defined in this version of the specification.

The 5G DDNMF interacts with the ProSe-enabled UE using procedures over PC3a reference point defined in clause 6.3.1 to allocate and resolve the mapping of ProSe Applications IDs and ProSe Application Codes used in ProSe Direct Discovery. It uses ProSe related subscriber data stored in UDM for the authorisation of each discovery request. It also provides the UE with the necessary security material in order to protect discovery messages transmitted over the air. In restricted ProSe Direct Discovery, it also interacts with the Application Server via Nxx reference points or N5g-ddnmf interface for the authorization of the discovery requests.

The ProSe-enabled UE use procedure defined in clause 4.3.2.2 to discovery the 5G DDNMF in the HPLMN. Based on the UE Local Configuration or URSP as defined in TS 23.503 [9], an existing PDU session is selected or a new PDU session is established, to carry the control signalling between the UE and the 5G DDNMF in the HPLMN.

The 5G DDNMF provides the necessary charging functionality or charging information for the usage of ProSe Direct Discovery and/or ProSe Direct Communication to interact with CHF or for the provision to CEF.

The 5G DDNMF in the HPLMN may interact with the 5G DDNMF in a VPLMN or Local PLMN in order to manage the ProSe Direct Discovery service.

4.3.2.2	5G DDNMF Discovery

The 5G DDNMF of HPLMN is discovered through interaction with the Domain Name Service function. The FQDN of a 5G DDNMF in the Home PLMN may either be pre-configured on the UE or provisioned by the network or self-constructed by the UE, e.g. derived from PLMN ID of the HPLMN. The IP address of a 5G DDNMF in the Home PLMN may also be provisioned to the UE.

The 5G DDNMF in the HPLMN uses the NRF to discovery other 5G DDNMFs in a VPLMN or local PLMN.

4.3.3	PCF

In addition to the functions defined in TS 23.501 [4] and TS 23.503 [9], the PCF includes functions to provision the UE with necessary policies and parameters to use ProSe direct discovery and communication, as part of the UE Policy information as defined in TS 23.503 [9] clause 4.2.2, based on the 5G ProSe Policy Provisioning Request:

-	PC5 usage reporting configuration.

-	Authorization policy and parameters for ProSe Direct Discovery and Communication.

-	Authorization policy and parameters for UE-to-Network Relay Discovery and Communication (i.e. as 5G ProSe Layer-2 Remote UE, as 5G ProSe Layer-3 Remote UE, as 5G ProSe Layer-2 UE-to-Network Relay, as 5G ProSe Layer-3 UE-to-Network Relay).

Editor’s Note:	It is to be determined whether and how the L3 policy information that is provided to the UE takes support of relaying via N3IWF into account.

The PCF may update the 5G ProSe policy and parameters to the UE under certain conditions.

4.3.4	AMF

In addition to the functions defined in TS 23.501 [4], the AMF performs the following functions:

-	Select a PCF supporting ProSe Policy/Parameter provisioning based on indication of 5G ProSe Capability as part of the "5GMM capability" in the Registration Request.

-	Stores the 5G ProSe Capability.

-	Obtain from UDM the subscription information related to ProSe and store them as part of the UE context data.

-	Obtain PC5 QoS parameters from the PCF and store them as part of the UE context data.

-	Provision the NG-RAN with indication about the UE authorization status about ProSe Direct Discovery and ProSe Direct Communication (i.e. as 5G ProSe UE for ProSe Direct Discovery, as 5G ProSe UE for ProSe Direct Communication), UE-to-Network Relay Discovery and Communication(i.e. as 5G ProSe Layer-2 Remote UE, as 5G ProSe Layer-2 UE-to-Network Relay, as Layer-3 UE-to-Network Relay).

-	Provision the NG-RAN with PC5 QoS parameters related to ProSe communication.

4.3.5	UDM

In addition to the functions defined in TS 23.501 [4], the UDM performs the following functions:

-	Subscription management for ProSe Direct Discovery and Communication.

-	Subscription management for UE-to-Network Relay Discovery and Communication.

4.3.6	UDR

In addition to the functions defined in TS 23.501 [4], the UDR performs the following functions:

-	Stores a path preference for ProSe services provided by the AF.

-	Stores ProSe service parameters.

4.3.7	NRF

In addition to the functions defined in TS 23.501 [4], the NRF performs the following functions:

-	PCF discovery by considering 5G ProSe capability.

-	5G DDNMF Discovery.

4.3.8	ProSe Application Server

The ProSe Application Server supports the following functionalities.

For Direct Discovery,

-	Maintains permission information for the restricted ProSe Direct Discovery using RPAUIDs;

-	Storage of ProSe Discovery UE IDs and metadata;

-	Mapping of RPAUID and PDUID for restricted ProSe Direct Discovery;

-	Provisioning parameters for Group Member Discovery to UE.

-	Interaction with 5G DDNMF for ProSe Direct Discovery, including

-	Allocation of the ProSe Restricted Code Suffix pool, if restricted Direct Discovery with application-controlled extension is used;

-	Allocation of the mask(s) for ProSe Restricted Code Suffix, if restricted Direct Discovery with application-controlled extension is used.

For ProSe Direct Communication,

-	Provisioning a path preference for ProSe Services to UDR;

-	Provisioning parameters for ProSe Direct Communication to UE.

For ProSe UE-to-Network Relay service,

-	Provisioning parameters for ProSe Relay Discovery and ProSe Relay Communication to UDR.

4.3.9	ProSe UE-to-Network Relay

4.3.9.1	Layer-3 UE-to-Network Relay

Layer-3 UE-to-Network Relay entity provides the relaying functionality to support connectivity to the network for Remote UEs. It can be used for both public safety services and commercial services (e.g. interactive service).

Layer-3 UE-to-Network Relay supports the following functions:

-	UE-to-Network Relay Discovery service as defined in clause 6.3.2.3, to allow discovery by the Remote UE;

-	ProSe Direct Communication as specified in clause 6.5.1, for the communication with the Remote UEs for the relay operation;

-	access the 5GS as a UE as defined in TS 23.501 [4] with the enhancements as specified in clauses 6.2 and 6.7;

-	relays unicast traffic (uplink and downlink) between the Remote UE and the network, supporting IP, Ethernet or Unstructured traffic type;

-	End-to-end QoS treatment for the Remote UE's traffic as defined in clause 5.7.2.1;

-	IP address management for the Remote UE as defined in clause 5.6 in case the Remote UE uses IP traffic type;

-	providing access to an N3IWF for the Remote UE when supported by the serving PLMN.

4.3.10	SMF

In addition to the functions defined in TS 23.501 [4], the SMF supports the following function:

-	Receiving Remote UE report and maintaining the information of Remote UE(s) handled by a Layer-3 ProSe UE-to-Network Relay in the ProSe UE-to-Network Relay's SM context for the PDU Session associated with the relay.

5	High level functionality and features

5.1	Authorization and Provisioning for ProSe service

5.1.1	General

In 5GS, the parameters for ProSe Direct Discovery, ProSe Direct Communication, and ProSe UE-to-Network Relay service may be made available to the UE in following ways:

-	provisioned in the ME; or

-	configured in the UICC; or

-	provisioned in the ME and configured in the UICC; or

-	provided/updated by the ProSe Application Server via PCF and/or PC1 reference point; or

-	provided/updated by the PCF to the UE.

If the same parameters described in clauses 5.1.2.1, 5.1.3.1 and 5.1.4.1 are provided by different sources, the UE shall consider them in the following priority order:

-	provided/updated by the PCF;

-	provided/updated by the ProSe Application Server via PC1 reference point;

-	configured in the UICC;

-	provisioned in the ME.

The parameters provided/updated by the ProSe Application Server via PC1 reference point may need to be complemented with configuration data from other sources listed above.

For the Group Member Discovery and Groupcast mode Direct Communication parameters, the parameters provided/updated by ProSe Application Server shall take a higher precedence than the same parameters that are provided/updated by the PCF, provisioned in the ME or configured in the UICC.

The basic principles of service authorization and provisioning for ProSe Direct Discovery, ProSe Direct Communication, and ProSe UE-to-Network Relay service are as follows:

-	The PCF in the HPLMN may configure a list of PLMNs where the UE is authorized to use ProSe Direct Discovery.

-	The PCF in the HPLMN may configure a list of PLMNs where the UE is authorised to use ProSe Direct Communication.

-	The PCF in the HPLMN may configure a list of PLMNs where the UE is authorised to act as ProSe UE-to-Network Relay. Authorisation for Layer-2 relaying and Layer-3 relaying are independent of each other.

-	The PCF in the HPLMN may configure a list of PLMNs where the UE is authorised to access 5GC via ProSe UE-to-Network Relay (i.e. to act as Remote UE). Authorisation to access via Layer-2 relaying and via Layer-3 relaying are independent of each other.

-	The PCF in the HPLMN merges authorization information from home and other PLMNs and provides the UE with the final authorization information.

-	The PCF in the VPLMN or HPLMN may revoke the authorization (via H-PCF when roaming) at any time by using the UE Configuration Update procedure for transparent UE Policy delivery procedure defined in clause 4.2.4.3 of TS 23.502 [5].

-	The ProSe Policy/parameters provisioning to UE is controlled by the PCF and may be triggered by UE. The PCF provisions one or more of the following ProSe Policy/parameters:

-	ProSe Policy/parameters for ProSe Direct Discovery as specified in clause 5.1.2.1;

-	the ProSe Policy/parameters for ProSe Direct Communications as specified in clause 5.1.3.1;

-	ProSe Policy/parameters for Layer-2 and/or Layer-3 ProSe UE-to-Network Relay as specified in clause 5.1.4.1;

-	ProSe Policy/parameters for Layer-2 and/or Layer-3 Remote UE as specified in clause 5.1.4.1.

-	The PCF includes the ProSe Policy/parameters in a Policy Section identified by a Policy Section Identifier (PSI) as specified in clause 6.1.2.2.2 of TS 23.503 [9].

In addition to the above, ProSe usage reporting configuration and rules for charging can be (pre)configured in the UE or provided by the PCF.

In addition to the above, the path selection policy can be (pre)configured in the UE or provided by the PCF as defined in clause 5.12. A path preference for ProSe Services can be provided by ProSe Application Server to UDR, and may be used by PCF for path selection policy generation and update.

When a Remote UE is using a Layer-3 ProSe UE-to-Network Relay without involving N3IWF, the PCF based provisioning and update of ProSe Policy/parameters to the Remote UE are not supported.

5.1.2	Authorization and Provisioning for ProSe Direct Discovery

5.1.2.1	Policy/Parameter provisioning for ProSe Direct Discovery

The following sets of information for ProSe Direct Discovery over PC5 reference point is provisioned to the UE:

1)	Authorization policy for ProSe Direct Discovery:

-	When the UE is "served by NG-RAN":

-	For open ProSe Direct Discovery:

a)	open ProSe Direct Discovery Model A monitoring authorization policy:

-	PLMNs in which the UE is authorised to perform ProSe Direct Discovery monitoring.

b)	open ProSe Direct Discovery Model A announcing authorization policy:

-	PLMNs in which the UE is authorized to perform announcing;

-	Authorised discovery range for announcing per PLMN.

-	For restricted ProSe Direct Discovery:

a)	restricted ProSe Direct Discovery Model A monitoring authorization policy:

-	PLMNs in which the UE is authorised to perform restricted ProSe Direct Discovery Model A monitoring.

b)	restricted ProSe Direct Discovery Model A announcing authorization policy:

-	PLMNs in which the UE is authorized to perform restricted ProSe Direct Discovery Model A announcing;

-	Authorised discovery range for announcing per PLMN.

c)	restricted ProSe Direct Discovery Model B Discoverer operation authorization policy:

-	PLMNs in which the UE is authorized to perform Model B Discoverer operation;

-	Authorised discovery range for announcing per PLMN.

d)	restricted ProSe Direct Discovery Model B Discoveree operation authorization policy:

-	PLMNs in which the UE is authorized to perform Model B Discoveree operation;

-	Authorised discovery range for announcing per PLMN.

NOTE 1:	In this specification, [When the UE is "served by NG-RAN"] and [When the UE is "not served by NG-RAN"] are relevant to ProSe Direct discovery/communications over NR PC5 reference point.

-		When the UE is "not served by NG-RAN":

-	Indicates whether the UE is authorized to perform ProSe Direct Discovery for Model A and Model B when "not served by NG-RAN".

2)	Identifiers used for ProSe Direct Discovery:

-	The mapping of ProSe services (i.e. Application IDs) to Destination Layer-2 ID(s) for sending/receiving initial signaling of discovery messages.

NOTE 2:	The same Destination Layer-2 ID for ProSe Direct Discovery can be mapped to more than one ProSe services.

3)	Radio parameters when the UE is "not served by NG-RAN":

-	Includes the radio parameters per NR PC5 with Geographical Area(s) and an indication of whether they are "operator managed" or "non-operator managed". The UE uses the radio parameters to perform ProSe Direct Discovery over PC5 reference point when "not served by NG-RAN" only if the UE can reliably locate itself in the corresponding Geographical Area. Otherwise, the UE is not authorized to transmit.

NOTE 3:	Whether a frequency band is "operator managed" or "non-operator managed" in a given Geographical Area is defined by local regulations.

4)	Restricted ProSe Discovery UE ID for Restricted Direct Discovery:

-	ProSe Discovery UE ID.

5)	Group Member Discovery parameters:

-	For each discovery group that the UE belongs to include the following parameters that enable the UE to perform Group Member Discovery when provisioned in ME from PCF or configured in UICC:

-	Application Layer Group ID: Identifies an application layer group that the UE belongs to;

-	Layer-2 Group ID: layer-2 ID for Discovery Group ID;

-	User Info ID: For Model A, this corresponds to the Announcer Info parameter when the UE is acting as an announcing UE. For Model B, this corresponds to the Discoverer Info in Solicitation messages and the Discoveree Info in Response messages, when the UE is acting as a discoverer or discoveree UE respectively.

NOTE 4:	User Info ID is expected to be assigned uniquely to a user within the discovery group.

-	Discovery Group ID: identifier of a discovery group that the UE belongs to.

NOTE 5:	Group Member Discovery parameters can be provided from ProSe Application Server to the UE.

6)	Application information for ProSe Direct Discovery:

-	Application identifiers to be used for direct discovery over PC5 interface.

7)	Security parameters used for direct discovery over PC5:

Editor's note:	Whether the security parameters can be provided by the PCF and details of security parameters will be determined by SA3 WG.

8)	Validity timer indicating the expiration time of the Policy/Parameter for ProSe Direct Discovery.

The above parameter sets bullet 2), bullet 3), and from bullet 5) to 8) may be configured in the UE by the ProSe Application Server.

5.1.2.2	Principles for applying parameters for ProSe Direct Discovery

For ProSe Direct Discovery over PC5 reference point, the operator may pre-configure the UEs with the required provisioning parameters for ProSe Direct Discovery, without the need for the UEs to connect to the 5GC to get this initial configuration. The following apply:

-	The provisioning parameters for ProSe Direct Discovery may be configured in the UICC, in the ME, or in both the UICC and the ME.

-	The ME provisioning parameters shall not be erased when a USIM is deselected or replaced.

-	If both the UICC and the ME contain the same set of overlapping provisioning parameters, the set of parameters from the UICC shall take precedence.

-	The provisioning parameters from the PCF or ProSe Application Server via PC1 reference point shall take precedence over the pre-configured parameters in the ME and UICC as defined in clause 5.1.1.

-	The UE shall use radio resources for ProSe Direct Discovery as follows:

-	While a UE has a serving cell and is camped on a cell and the UE intends to use for ProSe the radio resources (i.e. carrier frequency) operated by this cell, then the UE shall use the radio resource description indicated by this cell the UE is camped on and ignore any radio resource description of the same radio resource provisioned in the ME or the UICC. If the cell does not provide radio resources for ProSe, the UE shall not perform ProSe message transmission and reception on radio resources operated by this cell. The UE is allowed to perform ProSe Direct Discovery with another UE not served by the same PLMN.

-	If the UE intends to use "operator-managed" radio resources (i.e. carrier frequency) for ProSe that are not operated by the UE's serving cell, as specified in clause 5.1.2.1, or if the UE is out of coverage, the UE shall search for a cell in any PLMN that is operating the provisioned radio resources (i.e. carrier frequency) as defined in TS 38.300 [12] and TS 38.304 [13], and:

-	If the UE finds such a cell in the registered PLMN or a PLMN equivalent to the registered PLMN, and authorization for ProSe Direct Discovery to this PLMN is confirmed, the UE shall use the radio resource description indicated by that cell. If that cell does not provide radio resources for ProSe, the UE shall not perform ProSe message transmission and reception on those radio resources.

-	If the UE finds such a cell but not in the registered PLMN or a PLMN equivalent to the registered PLMN, and that cell belongs to a PLMN authorized for ProSe Direct Discovery and provides radio resources for ProSe then the UE shall perform PLMN selection triggered by ProSe Direct Discovery as defined in TS 23.122 [14].

-	If the UE finds such cell but not in a PLMN authorized for ProSe Direct Discovery the UE shall not use ProSe Direct Discovery.

-	If the UE does not find any such cell in any PLMN, then the UE shall consider itself "not served by NG-RAN" and use radio resources provisioned in the ME or the UICC. If no such provision exists in the ME or the UICC or the provision does not authorize ProSe Direct Discovery, then the UE is not authorized to transmit.

-	The UE is allowed to use "operator-managed" radio resources (i.e. carrier frequency) for ProSe Direct Discovery when the UICC shall indicate it.

-	If the UE intends to use "non-operator-managed" radio resources (i.e. carrier frequency) for ProSe, according to TS 36.331 [15] or TS 38.331 [16] and as specified in clause 5.1.2.1, then the UE shall perform ProSe Direct Discovery using resource provisioned in the ME or the UICC. If no such provision exists in the ME or the UICC or the provision does not authorize ProSe Direct Discovery, then the UE is not authorized to transmit.

NOTE 1:	It is possible for operators to configure UEs (e.g. Public Safety UEs) to use only "operator-managed" radio resources (i.e. carrier frequency) for ProSe Direct Discovery when the UE is "not served by NG-RAN".

-	The UE provisioning shall support setting Geographical Areas.

NOTE 2:	It is possible for a UE to use other radio resources for ProSe based on the Geographical Area instead of those operated by the serving NG-RAN cell, when provisioned in the UE, even if the UE's serving cell offers normal service and the SIBs for NR sidelink communication defined in TS 38.331 [16] indicates that the service (ProSe Direct Discovery) is available. This is to cover the scenario when e.g. the radio resources used for ProSe Direct Discovery are not owned by the serving network of the UE.

NOTE 3:	When cross-carrier operation is supported, according to TS 36.331 [15] or TS 38.331 [16], a UE can be instructed by its serving cell to perform ProSe Direct Discovery over a different carrier frequency. The UE is still considered as "served by NG-RAN" in this case.

NOTE 4:	The scenario that a cell is detected and the cell does not provide support for ProSe Direct Discovery when the UE attempts to use a carrier frequency configured for ProSe Direct Discovery, is considered a configuration error. Therefore, the UE does not transmit on that frequency to avoid interference to the network.

-	The ProSe Direct Discovery is only specified for NR.

NOTE 5:	It is out of scope of the present specification to define how the UE can locate itself in a specific Geographical Area. When the UE is in coverage of a 3GPP RAT, it can for example, use information derived from the serving PLMN. When the UE is not in coverage of a 3GPP RAT, it can use other techniques, e.g. Global Navigation Satellite System (GNSS). User provided location is not a valid input.

5.1.3	Authorization and Provisioning for ProSe Direct Communication

5.1.3.1	Policy/Parameter provisioning for ProSe Direct Communication

The following sets of information for ProSe Direct Communications over PC5 reference point is provisioned to the UE:

1)	Authorization policy:

-	When the UE is "served by NG-RAN":

-	PLMNs in which the UE is authorized to perform 5G ProSe Direct Communications over PC5 reference point when "served by NG-RAN".

-	When the UE is "not served by NG-RAN":

-	Indicates whether the UE is authorized to perform ProSe Direct Communications over PC5 reference point when "not served by NG-RAN".

NOTE 1:	In this specification, {When the UE is "served by NG-RAN"} and {When the UE is "not served by NG-RAN"} are relevant to ProSe Direct Communications over NR PC5 reference point.

2)	Groupcast mode ProSe Direct Communication policy/parameters:

-	For each application layer group supported include the parameters that enable the UE to perform Groupcast mode ProSe Direct Communication when provisioned from PCF or pre-configured in the ME or configured in the UICC:

-	Application Layer Group ID: Identifies an application layer group that the UE belongs to.

-	ProSe Layer-2 Group ID

-	ProSe Group IP multicast address

-	Indication whether the UE should use IPv4 or IPv6 for that group

-	For a specific Group configured to operate using IPv4, optionally an IPv4 address to be used by the UE as a source address. If none is provisioned, then the UE shall use Dynamic Configuration of IPv4 Link-Local Addresses RFC 3927 [18] to obtain a link local address for the Group.

-	Include group security related content for Groupcast mode ProSe Direct Communication.

NOTE 2:	More details on the necessary security aspect will be defined in SA3 specifications.

NOTE 3:	Groupcast mode ProSe Direct Communication parameters can be provided from ProSe Application Server to the UE.

3)	Radio parameters when the UE is "not served by NG-RAN":

-	Includes the radio parameters NR PC5 with Geographical Area(s) and an indication of whether they are "operator managed" or "non-operator managed". The UE uses the radio parameters to perform ProSe Direct Communications over PC5 reference point when "not served by NG-RAN" only if the UE can reliably locate itself in the corresponding Geographical Area. Otherwise, the UE is not authorized to transmit.

NOTE 4:	Whether a frequency band is "operator managed" or "non-operator managed" in a given Geographical Area is defined by local regulations.

4)	Policy/parameters related to privacy:

-	The list of ProSe services (i.e. ProSe Application IDs) with Geographical Area(s) that require privacy support.

5)	Policy/parameters when NR PC5 is selected:

-	The mapping of ProSe services (i.e. ProSe Application IDs) to radio frequencies with Geographical Area(s).

-	The mapping of ProSe services (i.e. ProSe Application IDs) to Destination Layer-2 ID(s) for broadcast.

-	The mapping of ProSe services (i.e. ProSe Application IDs) to Destination Layer-2 ID(s) for groupcast.

-	The mapping of ProSe services (i.e. ProSe Application IDs) to default Destination Layer-2 ID(s) for initial signalling to establish unicast connection.

NOTE 5:	The same default Destination Layer-2 ID for unicast initial signalling can be mapped to more than one ProSe service. In the case where different ProSe services are mapped to distinct default Destination Layer-2 IDs, when the UE intends to establish a single unicast link that can be used for more than one ProSe service, the UE can select any of the default Destination Layer-2 IDs to use for the initial signalling.

-	The mapping of ProSe services (i.e. ProSe Application IDs) to PC5 QoS parameters defined in clause 5.4.2 (i.e. PQI and conditionally other parameters such as MFBR/GFBR, etc.).

-	AS layer configurations (see TS 38.331 [16]), i.e. the mapping of PC5 QoS profile(s) to radio bearer(s), when the UE is "not served by NG-RAN".

-	The PC5 QoS profile contains PC5 QoS parameters described in clause 5.4.2 of TS 23.287 [2], and value for the QoS characteristics regarding Priority Level, Averaging Window, Maximum Data Burst Volume if default value is not used as defined in Table 5.4.4-1 of TS 23.287 [2] and in Table 5.7.1-1.

6)	Path selection policy:

-	The mapping of ProSe services to path preference (i.e. PC5 preferred, Uu preferred, or no preference) as defined in clause 5.12.

Editor's note:	It is FFS on how to identify ProSe services for Path selection.

NOTE 6:	The path selection policy can be a one mapping for all ProSe services, i.e. same path preference for all ProSe services.

7)	Validity time indicating the expiration time of the Policy/Parameter for ProSe Direct Communication.

The above parameter sets from bullet 2) to 7) may be configured in the UE by the ProSe Application Server.

5.1.3.2	Principles for applying parameters for ProSe Direct Communication

For ProSe Direct Communication over PC5 reference point, the operator may pre-configure the UEs with the required provisioning parameters for ProSe Direct Communication, without the need for the UEs to connect to the 5GC to get this initial configuration. The following apply:

-	The provisioning parameters for ProSe Direct Communication could be from different sources and their priorities are described in clause 5.1.1.

-	The ME provisioning parameters shall not be erased when a USIM is deselected or replaced.

-	The UE shall use radio resources for ProSe Direct Communication as follows:

-	While a UE has a serving cell and is camped on a cell and the UE intends to use for ProSe the radio resources (i.e. carrier frequency) operated by this cell, then the UE shall use the radio resource description indicated by this cell the UE is camped on and ignore any radio resource description of the same radio resource provisioned in the ME or the UICC. If the cell does not provide radio resources for ProSe, the UE shall not perform ProSe message transmission and reception on radio resources operated by this cell.

-	If the UE intends to use "operator-managed" radio resources (i.e. carrier frequency) for ProSe that are not operated by the UE's serving cell, as specified in clause 5.1.3.1, or if the UE is out of coverage, the UE shall search for a cell in any PLMN that is operating the provisioned radio resources (i.e. carrier frequency) as defined in TS 38.300 [12] and TS 38.304 [13], and:

-	If the UE finds such a cell in the registered PLMN or a PLMN equivalent to the registered PLMN, and authorization for ProSe Direct Communication to this PLMN is confirmed, the UE shall use the radio resource description indicated by that cell. If that cell does not provide radio resources for ProSe, the UE shall not perform ProSe message transmission and reception on those radio resources.

-	If the UE finds such a cell but not in the registered PLMN or a PLMN equivalent to the registered PLMN, and that cell belongs to a PLMN authorized for ProSe Direct Communications and provides radio resources for ProSe then the UE shall perform PLMN selection triggered by ProSe Direct Communication as defined in TS 23.122 [14].

-	If the UE finds such cell but not in a PLMN authorized for ProSe Direct Communications the UE shall not use ProSe Direct Communication.

-	If the UE does not find any such cell in any PLMN, then the UE shall consider itself "not served by NG-RAN" and use radio resources provisioned in the ME or the UICC. If no such provision exists in the ME or the UICC or the provision does not authorize ProSe Direct Communication, then the UE is not authorized to transmit.

-	The UE is allowed to use "operator-managed" radio resources (i.e. carrier frequency) for ProSe Direct Communication when the UICC shall indicate it.

-	If the UE intends to use "non-operator-managed" radio resources (i.e. carrier frequency) for ProSe, according to TS 36.331 [15] or TS 38.331 [16] and as specified in clause 5.1.3.1, then the UE shall perform ProSe Direct Communication using resource provisioned in the ME or the UICC. If no such provision exists in the ME or the UICC or the provision does not authorize ProSe Direct Communications, then the UE is not authorized to transmit.

NOTE 1:	It is possible for operators to configure UEs (e.g. Public Safety UEs) to use only "operator-managed" radio resources (i.e. carrier frequency) for ProSe Direct Communication when the UE is "not served by NG-RAN.

-	The UE provisioning shall support setting Geographical Areas.

NOTE 2:	It is possible for a UE to use other radio resources for ProSe based on the Geographical Area instead of those operated by the serving NG-RAN cell, when provisioned in the UE, even if the UE's serving cell offers normal service and the SIBs for NR sidelink communication defined in TS 38.331 [16] indicates that the service (ProSe Direct Communication) is available. This is to cover the scenario when e.g. the radio resources used for ProSe Direct Communication are not owned by the serving network of the UE.

NOTE 3:	When cross-carrier operation is supported, according to TS 36.331 [15] or TS 38.331 [16], a UE can be instructed by its serving cell to perform ProSe Direct Communication over a different carrier frequency. The UE is still considered as "served by NR or E-UTRA" in this case.

NOTE 4:	The scenario that a cell is detected and the cell does not provide support for ProSe Direct Communication when the UE attempts to use a carrier frequency configured for ProSe Direct Communications, is considered a configuration error. Therefore, the UE does not transmit on that frequency to avoid interference to the network.

-	The ProSe Direct Communication is only specified for NR.

NOTE 5:	It is out of scope of the present specification to define how the UE can locate itself in a specific Geographical Area. When the UE is in coverage of a 3GPP RAT, it can for example, use information derived from the serving PLMN. When the UE is not in coverage of a 3GPP RAT, it can use other techniques, e.g. Global Navigation Satellite System (GNSS). User provided location is not a valid input.

5.1.4	Authorization and Provisioning for ProSe UE-to-Network Relay

5.1.4.1	Policy/Parameter provisioning for ProSe UE-to-Network Relay

The following information is provisioned in the UE in support of the UE assuming the role of a ProSe UE-to-Network Relay:

1)	Authorisation policy for acting as a ProSe UE-to-Network Relay when "served by NG-RAN":

-	PLMNs in which the UE is authorized to relay traffic for Remote UEs.

2)	ProSe Relay Discovery policy/parameters for ProSe UE-to-Network Relay:

-	Includes the parameters that enable the UE to perform ProSe Relay Discovery when provisioned from the PCF in the ME or configured in the UICC:

-	Indication to be a ProSe UE-to-Network Relay;

-	ProSe UE-to-Network Relay Discovery parameters (User Info ID, Relay Service Code(s));

-	For Layer 3 ProSe UE-to-Network Relay, the PDU Session parameters (PDU Session type, DNN, SSC Mode, S-NSSAI, Access Type Preference) to be used for the relayed traffic for each ProSe Relay Service Code;

Editor’s Note:	Whether the associations between PDU session parameters and ProSe Relay Service Codes are provided to Relay UE and Remote UE as ProSe policy/parameters or as USRP rules will be decided at normative phase. Whether Remote UE provides other assistant info for handling PDU session parameters, e.g. PDU session requirement or Traffic descriptors to Relay UE will be decided at normative phase. The security and privacy aspects of this procedure will be coordinated with SA3.

-	Includes security related content for ProSe Relay Discovery for each ProSe Relay Service Code.

NOTE 1:	More details on the necessary security aspect are defined in TS 33.XXX [XX].

NOTE 2:	ProSe Relay Discovery policy/parameters can be provided from ProSe Application Server to the ProSe UE-to-Network Relay.

3)	For Layer 3 ProSe UE-to-Network Relay, QoS mapping rules:

-	Includes the rules that determine how the ProSe UE-to-Network Relay maps the 5QI of the QoS flow into a 5G ProSe PQI value over NR PC5;

The following information is provisioned in the UE in support of the UE assuming the role of a Remote UE and thereby enabling the use of a ProSe UE-to-Network Relay:

1)	Authorisation policy for using a ProSe UE-to-Network Relay:

-	Indicates whether the UE is authorised to use a ProSe UE-to-Network Relay.

2)	Policy/parameters for ProSe Relay Discovery:

-	Includes the parameters for ProSe Relay Discovery and for enabling the UE to connect to the ProSe UE-to-Network Relay after discovery when provisioned from the PCF in the ME or configured in the UICC:

-	ProSe UE-to-Network Relay Discovery parameters (User Info ID, Relay Service Code(s));

-	For Layer 3 ProSe UE-to-Network Relay, the PDU Session parameters (PDU Session type, DNN, SSC Mode, S-NSSAI, Access Type Preference) to be used for the relayed traffic for each ProSe Relay Service Code;

-	Includes security related content for ProSe Relay Discovery for each ProSe Relay Service Codes.

NOTE 3:	More details on the necessary security aspect are defined in TS 33.XXX [XX].

NOTE 4:	ProSe Relay Discovery policy/parameters can be provided from ProSe Application Server to the Remote UE.

The following information is provisioned in the UE in support of the UE assuming the role of a ProSe UE-to-Network Relay as well as in the UE in support of the UE assuming the role of a Remote UE and thereby enabling the use of a ProSe UE-to-Network Relay:

1)	Radio parameters for ProSe Relay Discovery when the UE is not "served by NG-RAN ":

-	Includes the radio parameters NR PC5 with Geographical Area(s) and an indication of whether they are "operator managed" or "non-operator managed". The UE uses the radio parameters to perform ProSe Direct Discovery over PC5 reference point when "not served by NG-RAN" only if the UE can reliably locate itself in the corresponding Geographical Area. Otherwise, the UE is not authorized to transmit.

2)	Radio parameters for ProSe Relay Communication when the UE is not "served by NG-RAN":

-	Includes the radio parameters NR PC5 with Geographical Area(s) and an indication of whether they are "operator managed" or "non-operator managed". The UE uses the radio parameters to perform ProSe Direct Communication over PC5 reference point when "not served by NG-RAN" only if the UE can reliably locate itself in the corresponding Geographical Area. Otherwise, the UE is not authorized to transmit.

5.1.4.2	Principles for applying parameters for ProSe UE-to-Network Relay

Editor's note:	This clause will document the principles for applying parameters for the ProSe UE-to-Network Relay and base on the KI# 8 conclusions of TR 23.752 [21].

5.1.4.3	Principles for Network controlled Remote authorization for ProSe UE-to-Network Relay

Editor's note:	This clause will document the Principles for Network controlled Remote authorization for the ProSe UE-to-Network Relay and base on the KI# 3 conclusions of TR 23.752 [21].

5.1.5	Authorization and Provisioning for Prose UE-to-UE Relay

5.1.5.1	Policy/Parameter provisioning for Prose UE-to-UE Relay

Editor's note:	This clause will document the policy/parameter provisioning for the ProSe UE-to-UE Relay and base on the KI# 4, 7 &8 conclusions of TR 23.752 [21].

5.1.5.2	Principles for applying parameters for Prose UE-to-UE Relay

Editor's note:	This clause will document the principles for applying parameters for the ProSe UE-to-UE Relay and base on the KI# 8 conclusions of TR 23.752 [21].

5.2	ProSe Direct Discovery

### 5.2.1	General

ProSe Direct Discovery is defined as the process that detects and identifies another UE in proximity via NR PC5 reference point. As defined in clause 5.3.3.1 in TS 23.303 [3], ProSe Direct Discovery can be open or restricted; it can be standalone or used for subsequent actions e.g. to initiate ProSe Direct Communication.

In the case of inter-PLMN ProSe discovery and communication over PC5 reference point, the PC5 parameters need to be configured in a consistent way among the UEs within a certain region. The architecture for the Inter-PLMN PC5 case is defined in clause 4.2.3.

### 5.2.2	ProSe Direct Discovery Models

There are two models for ProSe Direct Discovery: Model A and Model B which are defined in clause 5.3.1.2 in TS 23.303 [3].

### 5.2.3	UE-to-Network Relay and UE-to-UE Relay Discovery

For a high-level description of ProSe direct discovery of UE-to-UE Relay and UE-to-Network Relay, see clauses 5.4 and 5.5.

### 5.2.4	ProSe Direct Discovery Characteristics

ProSe Direct Discovery over the PC5 reference point has the following characteristics:

-	PC5 communication channel is used to carry the discovery message over PC5. The discovery message over PC5 is differentiated with other PC5 messages by AS layer.

-	ProSe layer shall indicate to AS layer whether the signalling is discovery message or PC5-S signalling.

NOTE:	The discovery message format is defined in stage 3.

Editor's note:	Whether and how to transmit metadata in discovery message depends on the size of the discovery message, it needs to be confirmed by RAN WG.

Group discovery/management to support on demand-based group communication for commercial services has the following characteristics:

-	The group discovery/formation/management can be carried out in the Application layer in coordination with the Application Server.

-	Application layer signalling between the UE and the Application Server is out of scope of this specification.

Editor's note:	It is FFS on how the Application layer discovery messages are exchanged over PC5 reference point between UEs.

Dynamic ProSe Direct Discovery has the following characteristics:

-	5G DDNMF in the 5GS is used for ProSe Discovery Code management (including allocation, and resolution).  5G DDNMF is defined in clause 4.3.2 and the detail procedure for Dynamic ProSe Direct Discovery is defined in clause 6.3.

Group discovery/management to support public safety has the following characteristics:

-	Pre-configured or provisioned information can be used for the ProSe Direct Discovery procedure as defined in clause 5.1.2.

The information elements included in the Direct Discovery messages are described in clause 5.9.1 and clause 6.3.2.

5.3	ProSe Direct Communication

5.3.1	General

ProSe direct communication over PC5 reference point is supported when the UE is "served by NR or E-UTRA" or when the UE is "not served by NR or E-UTRA". A UE is authorized to perform ProSe direct communication when it has valid authorization and configuration as specified in clause 5.1.3. ProSe direct communication supports both the cases of public safety and commercial service.

ProSe direct communication over NR based PC5 reference point supports broadcast mode, groupcast mode, and unicast mode.

For broadcast and groupcast mode ProSe communication, the following data unit types are supported: IPv4, IPv6, Ethernet, Unstructured, and Address Resolution Protocol (see RFC 826 [19]).

Editor's note:	Whether the data unit type of Address Resolution Protocol is supported needs to be coordinated with RAN WG.

For unicast mode ProSe direct communication, the following data unit types are supported: IPv4, IPv6, Ethernet, and Unstructured.

The identifiers used in the ProSe direct communication over PC5 reference point are described in clause 5.9.2.

The QoS handling and procedures for the ProSe direct communication over PC5 reference point are defined in clauses 5.7 and 6.4.

5.3.2	Broadcast mode ProSe Direct Communication

Broadcast mode of ProSe direct communication is supported over NR based PC5 reference point. The transmitting UE in broadcast communication determines the destination Layer-2 ID for broadcast as specified in clause 5.9.2 and assigns itself a source Layer-2 ID. The receiving UE determines the destination Layer-2 ID for broadcast reception as specified in clause 5.9.2. The transmitting UE determines the PC5 QoS parameters for this broadcast as specified in clauses 5.7.1. The transmitting UE sends the service data using the source Layer-2 ID and the destination Layer-2 ID.

5.3.3	Groupcast mode ProSe Direct Communication

Groupcast mode of ProSe direct communication is supported over NR based PC5 reference point. Group management is carried out by the application layer in coordination with Application Server and is out of scope of this specification. For commercial services, the Application Layer Group ID is provided by Application Server; and for public safety services, the pre-configured or provisioned Application Layer Group ID will be used for groupcast communication. The group size and member ID information could be used for groupcast control if it is provided by the application layer.

The transmitting UE in groupcast communication determines a source Layer-2 ID and a destination Layer-2 ID and the receiving UE determines destination Layer-2 ID, as specified in clauses 5.9.2. The transmitting UE determines the PC5 QoS parameters for this groupcast as specified in clauses 5.7.1. The transmitting UE sends the service data using the source Layer-2 ID and the destination Layer-2 ID.

5.3.4	Unicast mode ProSe Direct Communication

Unicast mode of ProSe direct communication is supported over NR based PC5 reference point. A PC5 unicast link between two UEs is established for the ProSe direct communication; and the PC5 unicast link could be maintained, modified, and released according to the application layer requests or communication requirements.

For the PC5 unicast link of the ProSe direct communication, the principal for the PC5 unicast link of V2X communication described in TS 23.287 [2] clause 5.2.1.4 is reused with the following differences:

-	V2X service is replaced by ProSe Application;

-	V2X service type is replaced by Application ID.

-	New data unit types are supported (including IPv4, Ethernet and Unstructured).

For IP type ProSe direct communication over PC5 reference point, the mechanism for IP address/prefix allocation is described in clause 5.6. The PC5 QoS handling for the unicast mode ProSe direct communication is specified in clauses 5.7.1.

5.4	ProSe UE-to-Network Relay

5.4.1	Layer-3 UE-to-Network Relay

5.4.1.1	General

The Layer-3 UE-to-Network Relay shall provide generic function that can relay any IP, Ethernet or Unstructured traffic:

-	For IP traffic over PC5 reference point, the Layer-3 UE-to-Network Relay uses IP type PDU Session towards 5GC.

-	For Ethernet traffic over PC5 reference point, the Layer-3 UE-to-Network Relay can use Ethernet type PDU Session or IP type PDU Session towards 5GC.

-	For Unstructured traffic over PC5 reference point, the Layer-3 UE-to-Network Relay can use Unstructured type PDU Session or IP type PDU Session (i.e. IP encapsulation/de-capsulation by Layer-3 UE-to-Network Relay) towards 5GC.

The type of traffic supported over PC5 reference point is indicated by the Layer-3 UE-to-Network Relay e.g. using the corresponding RSC. The Layer-3 UE-to-Network Relay determines the PDU Session Type based on configuration of the mapping between PDU Session parameters and RSC, as specified in clause 5.1.4.1.

IP type PDU Session and Ethernet type PDU Session can be used to support more than one Remote UEs while Unstructured type PDU Session can be used to support only one Remote UE.

NOTE 1:	The maximum number of PDU Sessions can affect the maximum number of Remote UEs that the UE-to-Network Relay can support.

NOTE 2: Non-unicast mode communication (i.e. one-to-many communication/broadcast or multicast) between network and Layer-3 UE-to-Network Relay UE and between Layer-3 UE-to-Network Relay and Remote UE(s) is not supported in this release of the specification.

5.4.1.2	Layer-3 UE-to-Network Relay with N3IWF support

To support Remote UE services with end-to-end confidentiality and IP address preservation requirements, the Layer-3 UE-to-Network Relay with N3IWF shall provide access to the 5GC for the Remote UE via N3IWF using the features defined in clause 4.2.8 and clause 5.30.2.7 of TS 23.501 [4].

Layer-3 UE-to-Network Relay is provisioned with RSC(s) and the corresponding PDU session parameters (e.g. S-NSSAI) to support N3IWF access as part of Layer-3 UE-to-Network Relay Policy/parameters. When a Remote UE connects with the corresponding RSC, the Layer-3 UE-to-Network Relay determines the corresponding PDU session parameters based on the requested RSC.

NOTE:	The Layer-3 UE-to-Network Relay only includes a RSC in discovery message when the corresponding PDU session parameters (e.g. S-NSSAI) are authorized to be used in the accessed network.

Editor's Note:	It is FFS whether a Layer-3 UE-to-Network Relay may include multiple supported RSCs in the same discovery message.

When selecting the N3IWF as specified in TS 23.501 [4] clause 6.3.6, the UE uses N3IWF configuration. Selection of the N3IWF follows the regulatory rules of the country where it is located, and when required by the regulations the Remote UE only selects a N3IWF within the local country. QoS differentiation can be provided on per-IPsec Child Security Association basis and the details are provided in clause 5.7.2.2

The 5GC to which the Layer-3 UE-to-Network Relay registers and the 5GC to which the Remote UE registers may be in the same PLMN or different PLMN.

5.4.2	Layer-2 UE-to-Network Relay

The Layer-2 UE-to-Network Relay provides forwarding functionality that can relay any type of traffic over the PC5 link.

The Layer-2 UE-to-Network Relay provides the functionality to support connectivity to the 5GS for Remote UEs. A UE is considered to be a Remote UE if it has successfully established a PC5 link to the Layer-2 UE-to-Network Relay. A Remote UE can be located within NG-RAN coverage or outside of NG-RAN coverage.

For Relay Discovery, the standalone discovery is used, and both Model A and Model B are supported.

For PLMN selection and relay selection in the remote UE:

- The Remote UE checks whether the Relay's PLMN is authorized for relay, and only the authorized PLMNs are the available PLMNs for NAS PLMN selection;

- The Remote UE selects the relay based on the selected PLMN by NAS layer.

5.5	ProSe UE-to-UE Relay

Editor's note:	This clause will document the general description of the direct communication via ProSe UE-to-UE Relay and base on the KI# 3 conclusions of TR 23.752 [21].

5.6	IP address allocation

For unicast mode of ProSe direct communication, the following mechanism for IP address/prefix allocation may be used:

a)	DHCP-based IPv4 address allocation with one of the two UEs acting as a DHCP server.

b)	IPv6 Stateless Address auto configuration specified in RFC 4862 [17] for assignment of IPv6 prefix, with one of the two UEs acting as IPv6 default router.

NOTE 1:	Which UE acts as a DHCPv4 server or IPv6 default router is negotiated during secure layer-2 link establishment by exchanging the IP Address Configuration as described in clause 6.4.3.

c)	IPv6 link-local addresses as defined in RFC 4862 [17] are formed by UEs locally. The IPv6 link-local addresses are exchanged during the establishment of a secure layer-2 link over PC5. The UEs shall disable duplicate address detection after the layer-2 link is established.

For broadcast and groupcast modes of ProSe direct communication, the following source IP address management applies:

a)	the UE configures a link local IPv4 address to be used as the source IP address, as defined in clause 4.5.3 of TS 23.303 [3]. If it is not configured with an address, it uses Dynamic Configuration of IPv4 Link-Local Addresses RFC 3927 [18].

b)	the UE configures a link local IPv6 address to be used as the source IP address, as defined in clause 4.5.3 of TS 23.303 [3]. The UE may use this IP address for direct communication without sending Neighbour Solicitation and Neighbour Advertisement message for Duplicate Address Detection.

NOTE 2:	The destination IP address management for broadcast and groupcast modes of ProSe direct communication is left to UE implementation.

5.7	QoS handling

5.7.1	QoS handling for ProSe Direct Communication

In order to support QoS handling for ProSe Direct Communication, the mechanism defined in clause 5.4 of TS 23.287 [2] is reused with the following differences:

-	Only NR PC5 QoS model is used.

-	V2X IP Packet Filter Set is replaced by ProSe IP Packet Filter Set.

-	V2X Packet Filter Set is replaced by ProSe Packet Filter Set.

Editor's note:	It is FFS which filter combination the ProSe Packet Filter Set is based on, e.g. Source/Destination Layer-2 ID, ProSe Application ID, etc.

-	V2X application layer is replaced by ProSe application layer.

-	V2X layer is replaced by ProSe layer.

-	V2X service type is replaced by Application ID.

-	UE-PC5-AMBR is only applied for NR PC5.

-	The PQI values are additionally defined. The one-to-one mapping of standardized PQI values that are additionally defined to PC5 QoS characteristics is specified in table 5.7.1-1.

Table 5.7.1-1: Standardized PQI values that are additionally defined to QoS characteristics mapping

| PQI Value | Resource Type | Default Priority Level | Packet Delay Budget | Packet Error Rate | Default Maximum Data Burst Volume | Default Averaging Window | Example Services |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 24 | GBR (NOTE 1) | 1 | 150 ms | 10-2 | N/A | 2000 ms | Mission Critical user plane Push To Talk voice (e.g. MCPTT) |
| 25 |  | 2 | 200 ms | 10-2 | N/A | 2000 ms | Non-Mission-Critical user plane Push To Talk voice |
| 26 |  | 2 | 200 ms | 10-3 | N/A | 2000 ms | Mission Critical Video user plane |
| 60 | Non-GBR | 1 | 120 ms | 10-6 | N/A | N/A | Mission Critical delay sensitive signalling (e.g. MC-PTT signalling) |
| 61 |  | 6 | 400 ms | 10-6 | N/A | N/A | Mission Critical Data (e.g. example services are the same as 5QI 6/8/9 as specified in TS 23.501 [4]) |
| 92 | Delay Critical GBR (NOTE 1) | 5 | 5ms | 10-4 | 20000 bytes | 2000 ms | Interactive service - consume VR content with high compression rate via tethered VR headset (See TS 22.261 [6]) |
| 93 |  | 6 | 10ms | 10-4 | 20000 bytes | 2000 ms | interactive service - consume VR content with low compression rate via tethered VR headset; Gaming or Interactive Data Exchanging (See TS 22.261 [6]) |
| NOTE 1: GBR and Delay Critical GBR PQIs can only be used for unicast PC5 communications. |  |  |  |  |  |  |  |

5.7.2	QoS handling for Relay operations

5.7.2.1	QoS handling for Layer-3 UE-to-Network Relay without N3IWF

For a Remote UE accessing network via Layer-3 UE-to-Network Relay without N3IWF, the QoS requirement between Remote UE and UPF can be satisfied by the corresponding QoS control for the PC5 link between Remote UE and UE-to-Network Relay (PC5 QoS control) and the QoS control for the PDU session established between UE-to-Network Relay and UPF (Uu QoS control). The PC5 QoS is controlled with PC5 QoS rules and PC5 QoS parameters (e.g. PQI, GFBR, MFBR, PC5 LINK-AMBR, Range) as specified in clause 5.4 of TS 23.287 [2]. The Uu QoS is controlled with QoS rules and 5G QoS parameters (e.g. 5QI, GFBR, MFBR) as specified in clause 5.7 of TS 23.501 [4].

As shown in figure 5.7.2.1-1 below, the end-to-end QoS can be met only when the QoS requirements are properly translated and satisfied over the two legs respectively.

Figure 5.7.2.1-1: End-to-End QoS translation for Layer 3 UE-to-Network Relay solution

To achieve this, the QoS mapping can be pre-configured or provided to the UE-to-Network Relay by the PCF using Prose Policy as specified in clause 5.1.4.1. The QoS mapping includes combinations of the 5QIs and PQIs mapping as entries. The PQI shall have standardized values as defined in Table 5.7.1-1 and in Table 5.4.4-1 of TS 23.287 [2]. The 5QI shall have standardized values as defined in TS 23.501 [4] clause 5.7.4.

If the QoS Flows setup are initiated by network, the SMF can base on the PCC rules or its local configuration to generates the QoS rules and QoS Flow level QoS parameters (e.g. 5QI, GFBR, MFBR) and signal to the UE-to-Network Relay using PDU Session Establishment/Modification procedure. Then the UE-to-Network Relay, based on the QoS mapping, decides the PC5 QoS parameters (e.g. PQI, GFBR, MFBR) for the corresponding PC5 QoS Flow. The PCF differentiates the relay traffic based on either local configuration, e.g.by DNN or S-NSSAI or by the traffic filters.

Editor's note:	The granularity of the QoS mapping rule , e.g. applicable for all the relay traffic or for the traffic of a specific service or a group of services is FFS.

If the Remote UE initiates PC5 QoS Flows setup, the Remote UE provides PC5 QoS context as defined in TS 23.287 [2], clause 5.4.1.1.3 to the UE-to-Network Relay, the PC5 QoS context indicates the end-to-end QoS requirements for the traffic transmission between Remote UE and UPF. If end-end QoS requirements can be supported by an entry in QoS mapping, the UE-to-Network Relay uses the 5QI of the entry for the Uu QoS control, and uses the PQI of the entry for the PC5 QoS control. If end-end QoS requirements cannot be supported by any entries in QoS mapping, the UE-to-Network Relay, based on its implementation, decides the 5QI for the Uu QoS control and PQI for the PC5 QoS control. The UE-to-Network Relay performs the UE requested PDU session Modification as defined in TS 23.502 [5], clause 4.3.3 for authorizing the requested QoS including the 5QI and if the PCF authorizes the requested QoS with a different 5QI value, the UE-to-Network Relay performs the Layer-2 link modification procedure as defined in clause 6.4.3.4 to update the corresponding PC5 QoS Flow.

Alternatively, reflective QoS control over Uu as defined in TS 23.501 [4], clause 5.7.5.3 can be leveraged for dynamic QoS handling of Remote UE to save on signalling between SMF and Layer-3 UE-to-Network Relay. Upon reception of a DL packet with RQI on the Uu for the remote UE, based on the indicated QFI, the UE-to-Network Relay creates a new derived QoS rule or updates existing derived QoS rule corresponding to the remote UE, as defined in TS 23.501 [4]. The derived QoS rule is for UL packets from the remote UE at Uu interface.

Based on signalled QoS rules (via SMF) or derived QoS rules (Uplink Uu via reflective QoS), the UE-to-Network Relay may use the L2 Link Modification procedures as defined in clause 6.4.3.4 to either update existing PC5 QoS flow or to set up a new PC5 QoS flow (when the QFI to PC5 QoS flow mapping does not exist).

When the UE-to-Network relay deletes the derived QoS rule e.g. after the RQ Timer expires, the UE-to-Network Relay may perform L2 Link Modification procedures defined in clause 6.4.3.4 accordingly using the PQI mapped from the 5QI of the currently used QoS rule after the deletion of the derived QoS rule(s).

5.7.2.2	QoS handling for Layer-3 UE-to-Network relay with N3IWF

When accessing 5GS via a Layer-3 UE-to-Network Relay with N3IWF, the Remote UE can request for PDU Session establishment or handover an existing PDU session to the N3IWF using UE requested PDU Session Establishment procedure defined in TS 23.502 [5] clause 4.12.5.

Figure 5.7.2.2-1: End-to-End QoS support via Layer-3 UE-to-Network Relay with N3IWF

For the Remote UE's PDU session(s) established via N3IWF, QoS differentiation can be provided on per-IPsec Child Security Association basis. N3IWF determines the IPsec child SAs as defined in TS 23.502 [5] clause 4.12. The N3IWF is preconfigured to allocate different IPsec child SAs for QoS Flows with different QoS profiles.

Based on configuration, the N3IWF can use one of the options below for QoS support in Relay UE's serving PLMN:

-	a static QoS mapping mechanism;

-	a dynamic QoS signalling based mechanism.

For the static QoS mapping mechanism, a SLA is established to govern the QoS handling between the Remote UE's 5GC and the Relay UE's 5GC, e.g. when the RSC is configured. The SLA can include the mapping between the DSCP markings for the IPsec child SAs with the Remote UE and the corresponding QoS, and N3IWF IP address(es). The non-alteration of the DSCP field between N3IWF and the Relay UE's UPF is also assumed to be governed by an SLA and by transport-level arrangements that are outside of 3GPP scope. The packet detection filters at the Relay UE's UPF can be based on the N3IWF IP address and the DSCP markings.

When the dynamic QoS signalling based mechanism is used by N3IWF, it works as follows:

-	When the Remote UE establishes or handovers a PDU session via the N3IWF as described in clause 4.12.5 of TS 23.502 [5], the PCF serving the PDU Session in the Remote UE's 5GC detects need for specific QoS and provides corresponding PCC rules to SMF in the Remote UE's 5GC. The resulted QoS information is provided to N3IWF in step 2b of clause 4.12.5 of TS 23.502 [5]. The N3IWF determines the IPSec Child SA(s) and signals to the Remote UE, as in step 4 of clause 4.12.5 of TS 23.502 [5] via IKE signalling including the PDU Session ID, the QFI(s), optionally a DSCP value, and optionally the Additional QoS Information specified in clause 4.12.5 of TS 23.502 [5]. The PDU Session Establishment Accept message will be sent to the Remote UE as in step 5 of clause 4.12.5 of TS 23.502 [5].

-	Based on Additional QoS Information received from the N3IWF, the Remote UE determines whether it is necessary to request for QoS session modification for the dedicated QoS Flows toward the UE-to-Network Relay as described in clause 5.7.2.1. The Remote UE also provides the N3IWF address, DSCP and the SPI as the traffic filter to enable filtering and mapping of DL traffic towards the right PDU Session/QoS Flow within the Relay UE's 5GC.

NOTE:	This mechanism allows to communicate GBR related parameters such as GFBR and MFBR from the PCF of the Remote UE via the N3IWF and the Remote UE to the Relay UE. The Relay UE would be able to request the GBR resources from its serving network using UE requested PDU session modification as in clause 4.3.3. of TS 23.502 [5].

-	If the UE-to-Network Relay performs the PDU Session Modification procedure, the PCF in the Relay UE's 5GC authorizes the QoS parameters. If the PDU Session Modification procedure authorized the requested QoS parameters, the UE-to-Network Relay acknowledges the Remote UE over PC5.

-	The PSA UPF in the Relay UE's 5GC maps the DL traffic from IPSec Child SA tunnel to appropriate PDU Session/QoS Flow considering SPI and N3IWF address (filters provided by the Remote UE).

-	The Remote UE's or the Remote UE's 5GC may initiated PDU Session Modification procedures as specified in clause 4.12.6 of TS 23.502 [5]. When the Remote UE received QoS information from the N3IWF, the same interactions between the Remote UE and UE-to-Network Relay, and between the UE-to-Network Relay and its 5GC as described above apply.

5.7.2.3	QoS handling for Layer-2 UE-to-Network Relay

For a Remote UE accessing network via Layer-2 UE-to-Network Relay, the existing 5G QoS control is reused between the Remote UE and the Remote UE's core network. The Remote UE's SMF provides QoS profiles to RAN, how RAN performs QoS enforcement for PC5 interface (between the Remote UE and UE-to-Network Relay UE) and Uu interface (between the UE-to-Network Relay UE and RAN) is out of the scope of this specification.

5.8	Subscription to ProSe

The subscription information in the UDM contains information to give the user permission to use 5G ProSe.

At any time, the operator can amend or remove the ProSe UE subscription rights from subscription information in the UDM, or to revoke the user's permission to use 5G ProSe.

The following subscription information is defined for 5G ProSe:

-	subscription for open ProSe Direct Discovery for NR PC5:

-	open ProSe Direct Discovery Model A.

-	subscription for restricted ProSe Direct Discovery for NR PC5:

-	restricted ProSe Direct Discovery Model A;

-	restricted ProSe Direct Discovery Model A with application-controlled extension;

-	restricted ProSe Direct Discovery Model A with "on demand" announcing;

-	restricted ProSe Direct Discovery Model B.

-	subscription for Broadcast, Groupcast and Unicast mode ProSe Direct Communication for NR PC5.

-	subscription for ProSe UE acting as Layer-2 UE-to-Network Relay.

-	subscription for ProSe UE acting as Layer-3 UE-to-Network Relay.

-	subscription for Layer-2 Remote UE access via Layer-2 5G ProSe UE-to-Network Relay

-	subscription for Layer-3 Remote UE access via Layer-3 ProSe UE-to-Network Relay

-	UE-PC5-AMBR for NR PC5.

-	PC5 QoS parameters as defined in clause 5.7.1 used by NG-RAN.

-	the list of the PLMNs authorized for 5G ProSe services, including:

-	the list of the PLMNs where the UE is authorised for open Direct Discovery Model A, i.e. to announce or monitor or both.

-	the list of the PLMNs where the UE is authorised for restricted ProSe Direct Discovery Model A, i.e. to announce or monitor or both.

-	the list of the PLMNs where the UE is authorised for restricted ProSe Direct Discovery Model B, i.e. to perform Discoverer operation or Discoveree operation or both.

-	the list of the PLMNs where the UE is authorised to perform Broadcast, Groupcast and Unicast mode ProSe Direct Communication for NR PC5.

5.9	Identifiers

5.9.1	Identifiers for ProSe Direct Discovery

5.9.1.0	General

NOTE:	The 5G DDNMF takes the role of "ProSe Function" if it exists in the following definitions in TS 23.303 [3].

5.9.1.1	ProSe Application ID

ProSe Application ID is defined in TS 23.303 [3].

5.9.1.2	Destination Layer 2 ID

Destination Layer 2 ID is defined in clause 5.6.1 of TS 23.287 [2].

The Destination Layer-2 ID for ProSe Direct Discovery with Model A is selected based on the configuration as described in clause 5.1.2.1. The Destination Layer-2 ID for a Solicitation message for Model B is selected based on the configuration as described in clause 5.1.2.1.

5.9.1.3	Source Layer 2 ID

Source Layer 2 ID is defined in clause 5.6.1 of TS 23.287 [2].

The UE self-selects a Source Layer-2 ID for ProSe Direct Discovery.

5.9.1.4	Discovery Group ID

Discovery Group ID is defined in TS 23.303 [3].

5.9.1.5	ProSe Application Code

ProSe Application Code is defined in TS 23.303 [3].

5.9.1.6	ProSe Restricted Code

ProSe Restricted Code is defined in TS 23.303 [3].

5.9.1.7	ProSe Query Code

ProSe Query Code is defined in TS 23.303 [3].

5.9.1.8	ProSe Response Code

ProSe Response Code is defined in TS 23.303 [3].

5.9.1.9	User Info ID

User Info ID (including Announcer Info, Discoverer Info, Target Info, Discoveree Info) is defined in TS 23.303 [3].

5.9.1.10	ProSe Discovery UE ID

ProSe Discovery UE ID is defined in TS 23.303 [3].

5.9.1.11	Restricted ProSe Application User ID

Restricted ProSe Application User ID is defined in TS 23.303 [3].

5.9.1.12	Announcing PLMN ID

Announcing PLMN ID is defined in TS 32.277 [22].

5.9.2	Identifiers for ProSe Direct Communication

5.9.2.1	General

Each UE has one or more Layer-2 IDs for ProSe direct communication over PC5 reference point, consisting of:

-	Source Layer-2 ID(s); and

-	Destination Layer-2 ID(s).

Source and Destination Layer-2 IDs are included in layer-2 frames sent on the layer-2 link of the PC5 reference point identifying the layer-2 source and destination of these frames. Source Layer-2 IDs are always self-assigned by the UE originating the corresponding layer-2 frames.

The selection of the Source and Destination Layer-2 ID(s) by a UE depends on the communication mode of ProSe direct communication over PC5 reference point for this layer-2 link, as described in clauses 5.9.2.2, 5.9.2.3, and 5.9.2.4. The Source Layer-2 IDs may differ between different communication modes.

5.9.2.2	Identifiers for broadcast mode ProSe direct communication

For broadcast mode of ProSe direct communication over PC5 reference point, the UE is configured with the Destination Layer-2 ID(s) to be used for ProSe applications. The Destination Layer-2 ID for a ProSe direct communication is selected based on the configuration as described in clause 5.1.3.1.

The UE self-selects a Source Layer-2 ID.

5.9.2.3	Identifiers for groupcast mode ProSe direct communication

For groupcast mode of ProSe direct communication over PC5 reference point, the application layer may provide Application Layer Group ID. When the Application Layer Group ID is provided, the UE converts the Application Layer Group ID into a Destination Layer-2 ID. When the Application Layer Group ID is not provided, the UE determines the Destination Layer-2 ID based on configuration of the mapping between ProSe service (i.e. Application ID) and Layer-2 ID, as specified in clause 5.1.3.1.

NOTE:	The mechanism for converting the application layer provided Application Layer Group ID to the Destination Layer-2 ID is defined in Stage 3.

The UE self-selects a Source Layer-2 ID.

5.9.2.4	Identifiers for unicast mode ProSe direct communication

For unicast mode of ProSe direct communication over PC5 reference point, the Destination Layer-2 ID used depends on the communication peer. The Layer-2 ID of the communication peer, identified by the peer's Application Layer ID, may be discovered during the establishment of the PC5 unicast link, or known to the UE via prior ProSe direct communications, e.g. existing or prior unicast link to the same Application Layer ID, or obtained from ProSe direct discovery process. The initial signalling for the establishment of the PC5 unicast link may use the known Layer-2 ID of the communication peer, or a default destination Layer-2 ID associated with the ProSe service (i.e. Application ID) configured for PC5 unicast link establishment, as specified in clause 5.1.3.1. During the PC5 unicast link establishment procedure, Layer-2 IDs are exchanged, and should be used for future communication between the two UEs, as specified in clause 6.4.3.

5.9.3	Identifiers for ProSe UE-to-Network Relay

5.9.3.1	Common identifiers for UE-to-Network Relay

The following parameters are used in the UE-to-Network Relay Discovery Announcement message (Model A):

-	ProSe Relay UE ID: link layer identifier that is used for direct communication and is associated with a Relay Service Code. A UE-to-Network Relay shall have a distinct ProSe Relay UE ID for each Relay Service Code.

Editor's Note:	the use of Relay UE ID needs to be revisited whether it should be made in line with V2X design approach (i.e. using source and destination layer-2 IDs).

-	Announcer Info: provides information about the announcing user.

-	Relay Service Code: parameter identifying a connectivity service the ProSe UE-to-Network Relay provides to a Remote UE. The Relay Service Codes are configured in a ProSe UE-to-Network Relay for advertisement. Additionally, the Relay Service Code may also identifies authorized users the ProSe UE-to-Network Relay would offer service to, and may be used to select the related security policies or information e.g. necessary for authentication and authorization between the Remote UE and the ProSe UE-to-Network Relay (e.g. a Relay Service Code for relays for police members only would be different than a Relay Service Code for relays for Fire Fighters only, even though potentially they provided connectivity to same DN e.g. to support Internet Access).

The following parameters are used in the UE-to-Network Relay Discovery Solicitation message (Model B):

-	Discoverer Info: provides information about the discoverer user.

-	Relay Service Code: information about connectivity that the discoverer UE is interested in. The Relay Service Codes are configured in the Remote UEs interested in related connectivity services.

-	ProSe Relay UE ID: link layer identifier of a UE-to-Network Relay that is used for direct communication and is associated with a Relay Service Code. A UE-to-Network Relay shall have a distinct ProSe Relay UE ID for each Relay Service Code. The ProSe Relay UE ID is optional.

Editor's Note:	the use of Relay UE ID needs to be revisited whether it should be made in line with V2X design approach (i.e. using source and destination layer-2 IDs).

The following parameters are used in the UE-to-Network Relay Discovery Response message (Model B):

-	ProSe Relay UE ID: link layer identifier that is used for direct communication and is associated with a Relay Service Code. A UE-to-Network Relay shall have a distinct ProSe Relay UE ID for each Relay Service Code.

Editor's Note:	the use of Relay UE ID needs to be revisited whether it should be made in line with V2X design approach (i.e. using source and destination layer-2 IDs).

-	Relay Service Code: identifies the connectivity service the ProSe UE-to-Network Relay provides to Remote UEs that matches the Relay Service Code from the corresponding Discovery Solicitation message.

-	Discoveree Info: provides information about the discoveree.

Editor’s note:	Whether User Info ID (i.e. Announcer Info, Discoverer Info, Discoveree Info) can be used for both public safety and commercial purpose is FFS.

5.9.3.2	Identifiers for Layer-3 UE-to-Network Relay

The following parameters may be used in addition to the parameters specified in 5.9.3.1, in Layer-3 UE-to-Network Relay supporting N3IWF discovery procedure (for both Model A and Model B) to assist with N3IWF selection:

-	Relay TAI: indicates the Tracking Area Identity corresponding to the serving cell of the Layer-3 UE-to-Network Relay.

For Layer-3 UE-to-Network relay, a Relay Service Code in the Announcement Message is associated with a set of PDU session parameters (e.g. PDU Session type, DNN, SSC Mode, S-NSSAI, Access Type Preference). The Relay Service Code may also represent if the relay UE can provide secure N3IWF connection.

For remote UE discovering Layer-3 UE-to-Network relay, the Relay Service Code in the Solicitation Message represents the PDU session parameters that a PDU session of the relay should be able to support. The Relay Service Code may also represent if the remote UE requires secure N3IWF connection.

5.9.3.3	Identifiers for Layer-2 UE-to-Network Relay

Editor’s note:	It is FFS what identifiers are used in addition to the identifiers specified in 5.9.3.1 for Layer-2 UE-to-Network Relay use case.

Editor’s note:	It is FFS with what information the RSC is associated for Layer-2 UE-to-Network Relay use case.

5.9.4	Identifiers for ProSe UE-to-UE Relay

Editor's note:	This clause will base on the KI#4 conclusion of TR 23.752 [21].

5.10	Support for ProSe for UEs in limited service state

For UE in limited service state, as defined in TS 23.122 [14], ProSe can be used over PC5 reference point with the following considerations.

UEs that are authorized to use ProSe over PC5 reference point according to clause 5.1 shall be able to use the corresponding services following the principles defined in clause 5.1.2.2 for ProSe Direct Discovery, clause 5.1.3.2 for ProSe Direct Communication, clause 5.1.4.2 for ProSe UE-to-Network Relay, and clause 5.1.5.2 for ProSe UE-to-UE Relay, when the UE enters in limited service state in 5GS:

-	because UE cannot find a suitable cell of the selected PLMN as described in TS 23.122 [14]; or

-	as the result of receiving one of the following reject reasons defined in TS 23.122 [14]:

-	a "PLMN not allowed" response to a registration request or;

-	a "5GS services not allowed" response to a registration request or service request.

A UE in limited service state shall only use the radio resources and procedure available in CM-IDLE mode for ProSe over PC5 reference point, for details see TS 36.300 [11] and TS 38.300 [12].

UEs shall not use ProSe over PC5 reference point using the "operator-managed" radio resources, as specified in clause 5.1.2.1, 5.1.3.1, 5.1.4.1, and 5.1.5.1, if the UE has entered in limited service state due to all other situations (e.g. no SIM in the MS, an "illegal MS" or "illegal ME" response to a registration request, or an "IMSI unknown in HLR" response to a registration request) defined in TS 23.122 [14], where the UE is unable to obtain normal service from a PLMN. The UEs may use ProSe over PC5 reference point using the "non-operator-managed" radio resources, as specified in clause 5.1.2.1, 5.1.3.1, 5.1.4.1, and 5.1.5.1, according to the corresponding principles defined in clause 5.1.2.2, 5.1.3.2, 5.1.4.2, and 5.1.5.2.

5.11	PC5 operation in EPS for Public Safety UE

When the UE is in EPS, the UE shall use the valid ProSe policy and parameters provisioned by the ProSe Function in EPC for ProSe Direct Discovery and Prose Direct Communication. If the UE does not have valid ProSe policy and parameters, the UE shall request the network to provision the ProSe policy and parameters.

The UE that is authorized to perform ProSe Direct Discovery and/or ProSe Direct Communication in EPS can perform the authorized PC5 operation in EPS as specified in TS 23.303 [3].

Editor's note:	Description about support for NR PC5 operation in network scheduled operation mode in EPS (i.e. EN-DC architecture) including UE movement from 5GS to EPS is FFS and depending on outcome for cross-RAT control of NR PC5 in RAN WGs.

5.12	Communication path selection between PC5 and Uu reference points

The "communication path selection between PC5 and Uu reference points" refers to the procedure on how a UE selects a communication path between PC5 reference point and Uu reference point before it establishes connection with another UE or the network.

Path selection policy is provided to the UE to indicate which path(s) is preferred for all or specific ProSe services (i.e. PC5 preferred, Uu preferred or no preference indicated) as specified in clause 5.1.3.1.

The ProSe Application Server can provide a path preference for ProSe Services to UDR as specified in clause 6.2.5 and this may be used by PCF for path selection policy generation and update as specified in clause 6.2.2.

NOTE 1:	ProSe Application Server can use QoS Sustainability analytics defined in TS 23.288 [8] to determine the path preference.

The UE may use the provisioned path selection policy to select the appropriate communication path.

UE operation related to the path selection for ProSe service is as follows:

-	The UE evaluates the path selection policy in the policy and parameters for ProSe Direct Communication applicable to the ProSe service and selects the communication path as below:

-	If PC5 preferred is indicated, the UE should prefer to use the PC5 for communication path for the ProSe service.

-	If Uu preferred is indicated, the UE should prefer to use the Uu for communication path for the ProSe service.

-	If no preference is indicated or no path selection policy is provisioned, the UE selects either a Uu or PC5 communication path based on its pre-configuration or implementation for the ProSe service.

NOTE 2:	When either PC5 preferred or Uu preferred is indicated, the UE can still select the other non-preferred path, e.g. because the peer UE is not in proximity.

6	Functional description and information flows

6.1	Control and user plane stacks

6.1.1	Control Plane

6.1.1.1	General

The control plane stack consists of protocols for controlling:

-	ProSe Direct Discovery;

-	ProSe Direct Communication;

-	ProSe UE-to-Network Relay.

6.1.1.2	UE - UE

6.1.1.2.1	Discovery plane PC5 interface

The PC5 communication channel is used to carry the discovery messages over PC5 which are differentiated from other PC5 messages by the AS layer.

Figure 6.1.1.2.1-1 depicts a discovery plane for NR PC5 reference point.

Legend:

-	PC5-D: The PDCP/RLC/MAC/PHY functionality is specified in TS 38.300 [12].

-	The "ProSe Discovery protocol" is used for handling ProSe Direct Discovery as specified in clause 6.3.2.

Figure 6.1.1.2.1-1: Discovery Plane PC5 Interface

6.1.1.2.2	PC5 Signalling Protocol

The PC5 Signalling Protocol stack specified in clause 6.1.2 of TS 23.287 [2] is used. The protocol used for the control plane signalling over the PC5 reference point for the secure layer-2 link is specified in clauses 6.4.3, 6.4.4 and 6.4.5.

6.1.1.3	UE - 5G DDNMF

Legend:

-	ProSe Control Signalling between UE and 5G DDNMF is carried over the user plane and using the PC3a protocol as specified in TS 24.554 [23].

NOTE 1:	PC3a may be realized with one or more protocols.

NOTE 2:	If 5G DDNMF is integrated with ProSe Application Server, 5G DDNMF provides PC3a interface towards UE and ProSe Application Server provides PC1 interface towards UE.

Figure 6.1.1.3-1 Control Plane for PC3a Interface

Editor's note:	It is FFS if PC3a interface can be replaced with application level interactions via PC1 when 5G DDNMF is integrated with ProSe Application Server.

6.1.1.4	5G DDNMF – UDM

5G DDNMF uses Nudm interface defined in TS 23.501 [4] to obtain the UE's subscription information for the authorization of the ProSe Direct Discovery requests.

6.1.1.5	5G DDNMF – 5G DDNMF

Legend:

-	N65 is an inter-PLMN interface between the 5G DDNMF in HPLMN and the 5G DDNMF in Local PLMN. N66 is a roaming interface between the 5G DDNMF in the HPLMN and the 5G DDNMF in VPLMN. N65 and N66 have the same function as the PC6 and PC7 specified in TS 29.345 [10]

-	DDNMF Application Protocol (DDNMF-AP): Application Layer Protocol between the 5G DDNMFs.

-	Stream Control Transmission Protocol (SCTP): This protocol transfers signalling messages. SCTP is defined in RFC 4960 [20].

Figure 6.1.1.5-1: Control Plane for N65/N66 reference point

When service based interfaces are implemented, the 5G DDNMFs uses N5g-ddnmf interface defined in clause 7.1 to access the services provided by the other 5G DDNMFs. The 5G DDNMF in HPLMN uses NRF to discovery the 5G DDNMFs in VPLMN and Local PLMNs.

6.1.1.6	5G DDNMF – ProSe Application Server

The 5G System architecture supports the service based Nxx interface between 5G DDNMF and ProSe Application Server and optionally supports PC2 interface between 5G DDNMF and ProSe Application Server, to enable Proximity Services. See TS 23.501 [4] and TS 23.303 [3].

NOTE:	PC2 support between 5G DDNMF and ProSe Application Server is for backwards compatibility for early deployments using Diameter.

6.1.1.7	UE - UE-to-Network Relay

#### 6.1.1.7.1	Layer-3 UE-to-Network Relay

The UE-UE protocol stacks for discovery and PC5 link management as defined in clause 6.1.1.2 apply to Remote UE and Layer-3 UE-to-Network Relay.

Additionally, when N3IWF is supported by the Layer-3 UE-to-Network Relay, the following control plane protocol stack apply.

Legend:

-	NAS, EAP-5G, and IKEv2 between the Remote UE and the N3IWF are defined in TS 23.501 [4] clause 8.2.4.

Figure 6.1.1.7.1-1: Control plane protocol stacks between Remote UE and N3IWF over Layer-3 UE-to-Network Relay before the signalling IPSec SA is established

Legend:

-	NAS, TCP, and IPsec between the Remote UE and the N3IWF are defined in TS 23.501 [4] clause 8.2.4.

Figure 6.1.1.7.1-2: Control plane protocol stacks between Remote UE and N3IWF over Layer-3 UE-to-Network Relay after the signalling IPSec SA is established

6.1.1.7.2	Layer-2 UE-to-Network Relay

Figure 6.1.1.7.2-1 illustrates the protocol stack of the NAS connection for the Remote UE for NAS-MM and NAS-SM. The NAS messages are transparently transferred between the Remote UE and NG-RAN over the Layer-2 UE-to-Network Relay UE using:

-	PDCP end-to-end connection between the Remote UE and NG-RAN, where the role of the UE-to-Network Relay UE is to relay the PDUs over the signalling radio bear without any modifications and using the functionality of the adaptation layer as specified in TS 38.XXX [x].

-	Connection between NG-RAN and AMF over N2.

-	Connection between AMF and SMF over N11.

Editor's Note:	Whether the adaptation layer is supported over PC5 will be determined by RAN WG2.

Figure 6.1.1.7.2-1: End-to-End Control Plane for a Remote UE using Layer-2 UE-to-Network Relay

The control plane protocol stack used by the Layer-2 UE-to-Network Relay is defined in clause 8.2.2 of TS 23.501 [4].

6.1.1.8	UE - UE-to-UE Relay

Editor's note:	This clause will document the CP stack based on the KI#4 conclusion of TR 23.752 [21].

6.1.2	User Plane

6.1.2.1	UE - UE

Editor's note:	This clause will document the UP stack based on the KI#2 conclusion of TR 23.752 [21].

6.1.2.2	UE - UE-to-Network Relay

#### 6.1.2.2.1	Layer-3 UE-to-Network Relay

Legend:

-	GPRS Tunnelling Protocol for the user plane (GTP-U): This protocol tunnels user data between NG-RAN node and UPF as well as between the UPFs in the backbone network (not shown in the figure). GTP-U shall encapsulate all end user PDU packets.

-	SMF controls the user plane tunnel establishment and establishes User Plane Bearers between NG-RAN node and UPF.

-	UDP/IP: These are the backbone network protocols used for routing user data and control signalling.

-	Uu: The NR Uu radio protocols of NG-RAN between the UE-to-Network Relay and the NG-RAN node are specified in TS 38.300 [12].

-	PC5-U: The radio protocols between the UE and the UE-to-Network Relay are specified in clause 6.1.2.1.

Figure 6.1.2.2.1-1: User plane protocol stack for Layer-3 UE-to-Network Relay

Legend:

-	IPSec, Inner IP, and GRE between the UE and the N3IWF are defined in TS 23.501 [4] clause 8.3.2.

Figure 6.1.2.2.1-2: User plane protocol stacks for Layer-3 UE-to-Network Relay with N3IWF support

6.1.2.2.2	Layer-2 UE-to-Network Relay

Figure 6.1.2.2.2-1 illustrates the protocol stack for the user plane transport, related to a PDU Session, including a Layer 2 UE-to-Network Relay UE. The PDU layer corresponds to the PDU carried between the Remote UE and the Data Network (DN) over the PDU session. The SDAP and PDCP protocols are specified in TS 38.300 [12]. PDCP end-to-end connection is between the Remote UE and NG-RAN. The functionality of the adaptation layer is specified in TS 38.XXX.

Editor's Note:	Whether the adaptation layer is supported over PC5 will be determined by RAN WG2.

Figure 6.1.2.2.2-1: User Plane Stack for L2 UE-to-Network Relay

6.1.2.3	UE - UE-to-UE Relay

Editor's note:	This clause will document the UP stack based on the KI#4 conclusion of TR 23.752 [21].

6.2	Procedures for Service Authorization and Provisioning to UE

6.2.1	General

The procedures for service authorization and provisioning to UE may be initiated by the PCF (as described in clause 6.2.2), by the UE (as described in clause 6.2.4), or by the AF (as described in clause 6.2.5).

6.2.2	PCF based Service Authorization and Provisioning to UE

For PCF based Service Authorization and Provisioning to UE, the Registration procedures as defined in clause 4.2.2.2 of TS 23.502 [5], UE Policy Association Establishment procedure as defined in clause 4.16.11 of TS 23.502 [5] and UE Policy Association Modification procedure as defined in clause 4.16.12 of TS 23.502 [5] apply with the following additions:

-	If the UE indicates 5G ProSe capability in the Registration Request message and if the UE is authorized to use ProSe service based on subscription data, the AMF selects the PCF which supports ProSe Policy/Parameter provisioning as described in clause 6.2.3 and establishes a UE policy association with the PCF for ProSe Policy/Parameter delivery. The 5G ProSe capability indicates whether the UE is capable of one or more of the following 5G ProSe capabilities: ProSe Direct Discovery, ProSe Direct Communication, Layer-2 and/or Layer-3 ProSe UE-to-Network Relay, and Layer-2 and/or Layer-3 Remote UE.

-	If the UE supports the 5G ProSe capability and it does not have valid ProSe policy/parameters, the UE includes the UE Policy Container with indicating the ProSe Policy Provisioning Request during registration procedure.

-	If the UE indicates the ProSe Policy Provisioning Request in the UE Policy Container, the PCF determines whether to provision ProSe Policy/parameters as specified in clause 5.1.1 to the UE, as specified in clause 6.1.2.2.2 of TS 23.503 [9], and the PCF provides the ProSe Policy/parameters (see clause 5.1.2.1, clause 5.1.3.1 and clause 5.1.4.1) to the UE by using the procedure as defined in clause 4.2.4.3 "UE Configuration Update procedure for transparent UE Policy Delivery" in TS 23.502 [5].

Editor's note:	It is to be determined whether the UE needs to indicate to the AMF and/or PCF support of relaying via N3IWF in addition to L3 relay capability.

The PCF may update the ProSe Policy/parameters to the UE in following conditions:

-	UE Mobility, e.g. UE moves from one PLMN to another PLMN. This is achieved by using the procedure of UE Policy Association Modification initiated by the AMF, as defined in clause 4.16.12.1 of TS 23.502 [5].

-	When there is a subscription change in the list of PLMNs where the UE is authorized to perform ProSe Direct Discovery or ProSe Direct Communication, or to act as a ProSe UE-to-Network Relay (Layer-2 and/or Layer-3) or to act as a Remote UE (Layer-2 and/or Layer-3. This is achieved by using UE Policy Association Modification initiated by the PCF procedure as defined in clause 4.16.12.2 of TS 23.502 [5].

Editor's note:	It is to be determined whether and how the L3 policy information that is provided to the UE takes support of relaying via N3IWF into account.

-	When there is a change of service specific parameter (including path selection policy) as described in clause 6.2.5 (performing the procedure in clause 4.15.6.7 of TS 23.502 [5]).

If the serving PLMN is removed from the list of PLMNs in the service authorization parameters, the service authorization is revoked in the UE.

When the UE is roaming, the change of subscription resulting in updates of the service authorization parameters are transferred to the UE by H-PCF via V-PCF.

The UE may perform UE triggered Policy Provisioning procedure to the PCF as specified in clause 6.2.4 when the UE determines the ProSe Policy/Parameter is invalid (e.g. Policy/Parameter is outdated, missing or invalid).

When a Remote UE is accessing to 5GC via a Layer-3 ProSe UE-to-Network Relay without involving N3IWF, the PCF based provisioning and update of ProSe Policy/parameters to the Remote UE are not supported.

6.2.3	PCF discovery

PCF discovery and selection mechanism defined in clause 6.3.7.1 of TS 23.501 [4] applies with the following addition to enable a PCF instance is selected for ProSe service and for UE:

-	Based on the indication from the UE and/or UE subscription data during the Registration procedure as specified in clause 6.5.2, the AMF may include the ProSe capability indication in the Nnrf_NFDiscovery_Request message as the optional input parameter. If provided, the NRF takes the information into account for discovering the PCF instance.

6.2.4	Procedure for UE triggered ProSe Policy provisioning

The UE triggered Policy Provisioning procedure is initiated by the UE to request ProSe Policy/Parameter from the PCF when UE determines the ProSe Policy/Parameter is invalid in the following cases:

-	if the validity timer indicated in the ProSe Policy/Parameter expires;

-	if there are no valid parameters, e.g. for the ProSe service type a UE wants to use, for current area, or due to abnormal situation.

Figure 6.2.4-1: UE triggered ProSe Policy provisioning procedure

1.	The UE sends the UE Policy provisioning request including the UE Policy Container (ProSe Policy Provisioning Request) to the AMF.

2.	The AMF sends the Npcf_UEPolicyControl_Update request to the PCF including the UE Policy Container received from UE.

3.	The UE Policy delivery procedure defined in clause 4.2.4.3 of TS 23.502 [5] is triggered.

6.2.5	AF-based service parameter provisioning for ProSe over control plane

For ProSe service parameter provisioning, the procedure defined in clause 4.15.6.7 of TS 23.502 [5] is performed with the following considerations:

-	The AF in TS 23.502 [5] is considered as ProSe Application Server in this specification.

-	Service Description indicates ProSe service domain information.

-	Service Parameters include parameters for ProSe Direct Discovery and ProSe Direct Communications. The detailed information on the parameters is described in clause 5.1.2.1 and clause 5.1.3.1.

-	Service Parameters for ProSe Relay Discovery and ProSe Relay Communications. The detailed information on the parameters is described in clause 5.1.4.1.

NOTE:	It is assumed that the ProSe service domain information is set based on the Service Level Agreement with the operator.

6.3	ProSe Direct Discovery

6.3.1	ProSe Direct Discovery with 5G DDNMF

6.3.1.1	Overview

ProSe Direct Discovery is defined as the process that detects and identifies another UE in proximity using NR radio signals. There are two types of ProSe Direct Discovery supported over PC3a reference point: open and restricted, as defined in TS 23.303 [3]. ProSe Direct Discovery can be a standalone service or can be used for subsequent actions e.g. to initiate ProSe Direct Communication.

ProSe-enabled UEs which have obtained authorization to participate in ProSe Direct Discovery shall not continue in participating in ProSe Direct Discovery procedures over PC3a reference point defined in clause 6.3.1 when they detect loss of NG-RAN coverage in the serving PLMN.

With ProSe Direct Discovery, the UE can use inter-PLMN discovery transmission based on the indication from the serving NG-RAN or the provisioned radio resource on the UE. How the serving cell authorizes the UE to use inter-PLMN radio resource is specified in TS 38.331 [16].

6.3.1.2	Overall procedure for ProSe Direct Discovery (Model A)

Figure 6.3.1.2-1: Overall procedure for Model A ProSe Direct Discovery

This procedure is applied for open and restricted ProSe Direct Discovery when the ProSe enabled UE is served by NG-RAN.

1.	Service authorisation for ProSe Direct Discovery services is performed for as defined in clauses 6.2.

If the UE is authorised to announce:

2a.	When the UE is triggered to announce, then it sends a discovery request for announcing to the 5G DDNMF in HPLMN as defined in clauses 6.3.1.4. In addition, for restricted ProSe Direct Discovery, the 5G DDNMF further interacts with the ProSe Application server for the authorization of the discovery request.

3a.	If the request is successful and is provided with ProSe Application Code/ProSe Restricted Code, it starts announcing on PC5 interface.

For ProSe restricted discovery and UE requests "on demand" announcing, ProSe Restricted Code may be provided to UE after this procedure. In this case, UE waits for the ProSe Restricted Code allocation and starts to announce the ProSe Restricted Code on PC5 after receiving it in Announcing Alert procedure specified in clause 6.3.1.6.

NOTE 1:	More details on the Access Stratum protocol of this step are provided in RAN specifications.

If the UE is authorised to monitor:

2b.	When the UE is triggered to monitor, it sends a discovery request for monitoring to the 5G DDNMF as defined in clauses 6.3.1.4. In addition, for restricted ProSe Direct Discovery, the 5G DDNMF further interacts with the ProSe Application server for the authorization of the discovery request.

3b.	If the request is successful and the UE is provided with a Discovery Filter consisting of ProSe Application Code(s)/ProSe Restricted Code(s) and/or ProSe Application Mask(s), it starts monitoring for these ProSe Application Codes/ProSe Restricted Codes on the PC5 interface.

NOTE 2:	More details on the Access Stratum protocol of this step are provided in RAN specifications.

4b.	When the UE detects that one or more ProSe Application Code(s)/ProSe Restricted Code(s) that match the filter (see clause 5.9.1), it reports the ProSe Application Code(s)/ProSe Restricted Code(s) to the 5G DDNMF as defined in clause 6.3.1.5.

Non roaming direct discovery procedures cover the case where both the "announcing UE" and "monitoring UE" are served by their respective HPLMN. Roaming direct discovery procedures cover the other cases.

6.3.1.3	Overall procedure for ProSe Direct Discovery (Model B)

Figure 6.3.1.3-1: Overall procedure for Model B ProSe Direct Discovery

This procedure is applied for restricted ProSe Direct Discovery when the ProSe enabled UE is served by NG-RAN.

1.	Service authorisation for ProSe Direct Discovery services is performed as defined in clauses 6.2.

If the UE is authorised to perform restricted ProSe Direct Discovery, Model B, as a Discoveree UE, the following steps take place:

2a.	When the UE is triggered to perform restricted ProSe Direct Discovery, Model B, it sends a discovery request to the 5G DDNMF in the HPLMN to obtain a ProSe Response Code as defined in clauses 6.3.1.4. The 5G DDNMF further interacts with ProSe Application Server for the authorization of the discovery request.

3a.	If the request is successful and the UE is provided with a ProSe Response Code and an associated Discovery Query Filter(s), then the UE starts monitoring for the ProSe Query Code on PC5 interface.

4a.	If a received ProSe Query Code matches any of the Discovery Query Filter(s), the UE announces the associated ProSe Response Code on the PC5 interface.

NOTE 1:	More details on the Access Stratum protocol of this step are provided in RAN specifications.

If the UE is authorised to perform restricted ProSe Direct Discovery, Model B, as a Discoverer UE, the following steps take place:

2b.	When the UE is triggered to perform restricted ProSe Direct Discovery, Model B, it sends a discovery request to the 5G DDNMF in the HPLMN for a ProSe Query Code as defined in clauses 6.3.1.4. The 5G DDNMF further interacts with ProSe Application Server for the authorization of the discovery request.

3b.	If the request is successful and the UE is provided with a ProSe Query Code and the Discovery Response Filter(s) consisting of ProSe Response Code(s) and ProSe Application Mask(s), the UE announces the ProSe Query Code on the PC5 interface.

4b.	The UE starts to monitor on PC5 interface for any ProSe Response Code(s) that might match the Discovery Response Filter(s).

NOTE 2:	More details on the Access Stratum protocol of this step are provided in RAN specifications.

5b.	When the UE detects a match for one or more ProSe Response Code(s), it reports the ProSe Response Code to the 5G DDNMF as defined in clauses 6.3.1.5.

Non roaming direct discovery procedures cover the case where both the Discoveree UE and Discoverer UE are served by their respective HPLMN. Roaming direct discovery procedures cover the other cases.

#### 6.3.1.4	Discovery Request procedures

The Discovery Request procedure can be used by the "announcing UE" or "monitoring UE" in order to be authorised to access the discovery resources and perform ProSe Direct Discovery. The exact signalling procedures involving the UE, the 5G DDNMFs, and the ProSe Application Server are specified in TS 23.303 [3] clause 5.3.3, with the following modifications:

-	the 5G DDNMF takes the role of "ProSe Function" in the procedure;

-	The 5G DDNMF gets the PDUID and subscribes to notifications on Change of PDUID using Npcf_AMPolicyAuthorization_Subscribe as defined in the clause 5.2.5.8.6 in TS 23.502 [5], including the SUPI, Event ID set to "Change of PDUID", and immediate reporting flag to indicate that the current PDUID value should be provided to the consumer. The PCF provides the PDUID and its validity timer.

-	At the time the PCF generates a new PDUID, if the subscription to "Change of PDUID" is active it sends Npcf_AMPolicyAuthorization_Notify to the 5G DDNMF to report a new PDUID and its validity timer.

-	the HSS is replaced by UDM;

-	corresponding 5GS identifiers replace the EPS identifiers, e.g. use SUPI instead of IMSI, and use GPSI instead of MSISDN;

-	PC5_tech parameter is omitted and the intended PC5 radio technology is NR.

The Discovery Request procedure can also be used by the Discoveree UE or the Discoverer UE in order to be authorised to access the discovery resources and perform ProSe Direct Discovery, Model B. The exact signalling procedures are defined in TS 23.303 [3] clause 5.3.3A, with the same modifications as in the above list apply.

The events reported by the PCF, described in clause 6.1.3.18 in TS 23.503 [9] are extended to report "Change of PDUID" to the 5G DDNMF.

#### 6.3.1.5	Discovery Reporting procedures

The Discovery Reporting procedure can be used by the "monitoring UE" (in Model A) and Discoverer UE (in Model B) to request the 5G DDNMF to resolve a matched ProSe Discovery Code(s) (ProSe Application Code for open discovery, and ProSe Restricted Code for restricted discovery) and obtain the corresponding ProSe Application ID(s) or RPAUID, and additional information, e.g. metadata.

The signalling procedures for the "monitoring UE" (in Model A) is specified in TS 23.303 [3] clause 5.3.4, and the signalling procedures for the Discoverer UE (in Model B) is specified in TS 23.303 [3] clause 5.3.4A, with the following modifications:

-	the 5G DDNMF takes the role of "ProSe Function" in the procedure;

-	the HSS is replaced by UDM;

-	corresponding 5GS identifiers replace the EPS identifiers, e.g. use SUPI instead of IMSI, and use GPSI instead of MSISDN;

-	PC5_tech parameter is omitted and the intended PC5 radio technology is NR.

#### 6.3.1.6	Announcing Alert Procedures for restricted discovery

When supported by the 5G DDNMF and the UE, the Announcing Alert procedure allows the 5G DDNMF to postpone the ProSe Restricted Code allocation, so that the announcing UE would be only triggered by this procedure to announce when the 5G DDNMF receives a Monitor Request from a UE in the vicinity of the announcing UE. This procedure is an optional step of the Discovery Request procedure defined in clause 6.3.1.4.

The signalling procedure of Announcing Alert Procedure is specified in TS 23.303 [3] clause 5.3.5, with the same modifications listed in clause 6.3.1.4.

#### 6.3.1.7	Direct Discovery Update Procedures

The 5G DDNMF can at any time update/revoke a previously allocated ProSe Application Code, or Discovery Filters. The UE can decide at any time to stop announcing a ProSe Application Code or monitoring set of Discovery Filter(s). The Direct Discovery Update procedure as specified in TS 23.303 [3] clause 5.3.6A allows both the network and the UE to update or revoke the previously authorized discovery. In the defined signalling procedures, the 5G DDNMF(s) takes the role of the "ProSe Function".

6.3.2	ProSe Direct Discovery procedures over PC5 reference point

6.3.2.1	General

A PC5 communication channel is used to carry the discovery message over PC5 and the discovery message over PC5 is differentiated from other PC5 messages by AS layer.

Both Model A and Model B discovery as defined in TS 23.303 [3] are supported:

-	Model A uses a single discovery protocol message (Announcement).

-	Model B uses two discovery protocol messages (Solicitation and Response).

Depicted in figure 6.3.2.1-1 is the procedure for ProSe Direct Discovery with Model A.

Figure 6.3.2.1-1: ProSe direct discovery with Model A

1.	The Announcing UE sends an Announcement message. The Announcement message may include the Type of Discovery Message (i.e. Announcement), ProSe Application Code or ProSe Restricted Code, ProSe Application ID, User Info ID, security protection element, metadata information.

The Destination Layer-2 ID and Source Layer-2 ID used to send the Announcement message are specified in clause 5.9.1.2 and clause 5.9.1.3.

The Monitoring UE determines the Destination Layer-2 ID for signalling reception. The Destination Layer-2 ID is configured with the UE(s) as specified in clause 5.9.1.2.

Depicted in figure 6.3.2.1-2 is the procedure for ProSe Direct Discovery with Model B.

Figure 6.3.2.1-2: ProSe direct discovery with Model B

1.	The Discoverer UE sends a Solicitation message. The Solicitation message may include Type of Discovery Message (i.e. Solicitation), ProSe Query Code, ProSe Application ID, User Info ID, security protection element.

The Destination Layer-2 ID and Source Layer-2 ID used to send the Solicitation message are specified in clause 5.9.1.2 and clause 5.9.1.3.

How the Discoveree UE determines the Destination Layer-2 ID for signalling reception is specified in clause 5.9.1.2.

2.	The Discoveree UE that matches the solicitation message responds to the Discoverer UE with the Response message. The Response message may include Type of Discovery Message (i.e. Response), ProSe Response Code, ProSe Application ID, User Info ID, security protection element, metadata information.

The Source Layer-2 ID used to send the Response message is specified in clause 5.9.1.3. The Destination Layer-2 ID is set to the Source Layer-2 ID of the received Solicitation message.

NOTE:	Details of security protection element will be defined by SA WG3.

6.3.2.2	Group Member Discovery

6.3.2.2.1	General

Group Member Discovery is applicable to public safety use and commercial services. To perform Group Member Discovery, the UE is configured with the related information as described in clause 5.2.

Group Member Discovery is a form of restricted discovery in that only users that are affiliated with each other can discover each other (e.g. only users sharing the same Discovery Group ID).

In the case of Public Safety use, the ProSe Restricted Code is not used for Group Member Discovery, and pre-configured or provisioned information for the Discovery procedures as defined in clause 5.2 is used.

NOTE:	The Group Member Discovery performed by Application Layer in coordination with Application Server is out of scope of this specification.

Both Model A and Model B discovery are supported:

-	Model A uses a single discovery protocol message (Announcement).

-	Model B uses two discovery protocol messages (Solicitation and Response).

6.3.2.2.2	Procedure for Group Member Discovery with Model A

Depicted in Figure 6.3.2.2.2-1 is the procedure for Group Member Discovery with Model A.

Figure 6.3.2.2.2-1: Group Member Discovery with Model A

1.	The announcing UE sends a Group Member Discovery Announcement message. The parameters contained in this message are described in clause 5.9.1.

NOTE:	A UE may send multiple Group Member Discovery Announcement messages (Model A) if the UE belongs to more than one discovery group.

6.3.2.2.3	Procedure for Group Member Discovery with Model B

Depicted in Figure 6.3.2.2.3-1 is the procedure for Group Member Discovery with Model B.

Figure 6.3.2.2.3-1: Group Member Discovery with Model B

1.	The discoverer UE sends a Group Member Discovery Solicitation message. The parameters contained in this message are described in clause 5.9.1.

2.	The discoveree UEs that match the values of the parameters contained in the solicitation message based on the Discovery Group ID, respond to the discoverer UE with a Group Member Discovery Response message. The parameters contained in this message are described in clause 5.9.1.

6.3.2.3	UE-to-Network Relay Discovery

6.3.2.3.1	General

UE-to-Network Relay Discovery is applicable to both Layer-3 and Layer-2 UE-to-Network relay discovery for public safety use and commercial services. To perform UE-to-Network Relay Discovery, the Remote UE and the UE-to-Network Relay are pre-configured or provisioned with the related information as described in clause 5.1.

In UE-to-Network Relay Discovery, the UEs use pre-configured or provisioned information for the relay discovery procedures as defined in clause 5.1.4.1.

The Relay Service Code (RSC) is used in the UE-to-Network Relay discovery, to indicate the connectivity service the UE-to-Network Relay provides to the Remote UE. The RSCs are configured on the UE-to-Network Relay and the Remote UE as defined in clause 5.1.4. The RSC can also indicate if the UE-to-Network Relay is a Layer-3 or Layer-2 UE-to-Network Relay.

Additional information used for the UE-to-Network Relay (re)selection and connection maintenance can be advertised using a separate discovery messages of type "Relay Discovery Additional Information". This may include for example the related system information of the UE-to-Network Relay's serving cell, as defined in TS 38.300 [12].

Both Model A and Model B discovery are supported:

-	Model A uses a single discovery protocol message (Announcement).

-	Model B uses two discovery protocol messages (Solicitation and Response).

For Relay Discovery Additional Information, only Model A discovery is used.

6.3.2.3.2	Procedure for UE-to-Network Relay Discovery with Model A

Depicted in Figure 6.3.2.3.2-1 is the procedure for UE-to-Network Discovery with Model A.

Figure 6.3.2.3.2-1: UE-to-Network Relay Discovery with Model A

1.	The UE-to-Network Relay sends a UE-to-Network Relay Discovery Announcement message. The parameters contained in this message are described in clause 5.9.3.

Remote UE (1 to 3) monitors announcement messages with the UE-to-Network RSC corresponding to the desired services.

Optionally, the ProSe UE-to-Network Relay may also send UE-to-Network Relay Discovery Additional Information messages. The parameters contained in this message are described in clause 5.9.3.

6.3.2.3.3	Procedure for UE-to-Network Relay Discovery with Model B

Depicted in Figure 6.3.2.3.3-1 is the procedure for UE-to-Network Relay Discovery with Model B.

Figure 6.3.2.3.3-1: UE-to-Network Relay Discovery with Model B

1.	The Remote UE sends a UE-to-Network Relay Discovery Solicitation message. The parameters contained in this message are described in clause 5.9.3. The Remote UE discovering a UE-to-Network Relay sends a solicitation message with the UE-to-Network RSC which is associated to the desired connectivity service.

2.	The UE-to-Network Relays (1 to 3) that match the values of the RSC contained in the solicitation message respond to the Remote UE with a UE-to-Network Relay Discovery Response message. The parameters contained in this message are described in clause 5.9.3.

The Remote UE selects the UE-to-Network Relay based on the information received in step 2.

6.3.2.4	UE-to-UE Relay Discovery

Editor's note:	This clause will document the detailed procedures of the UE-to-UE Relay discovery and base on the KI# 1&4 conclusions of TR 23.752 [21].

6.4	ProSe Direct Communication

6.4.1	Broadcast mode ProSe Direct Communication

To perform ProSe direct communication over PC5 reference point in broadcast mode operation, the UE is configured with the related information as described in clause 5.1.3.

Figure 6.4.1-1 shows the procedure for broadcast mode of ProSe direct communication over PC5 reference point.

Figure 6.4.1-1: Procedure for Broadcast mode of ProSe direct communication over PC5 reference point

1.	The receiving UE(s) determine the destination Layer-2 ID for broadcast reception as specified in clause 5.9.2. The destination Layer-2 ID is passed down to the AS layer of receiving UE(s) for the reception.

2.	The transmitting UE ProSe application layer provides data unit and may provide ProSe Application Requirements specified in clause 5.7.1 to ProSe layer.

3.	The transmitting UE determines the destination Layer-2 ID for broadcast as specified in clause 5.9.2. The transmitting UE self-assigns the source Layer-2 ID as specified in clause 5.9.2.

The transmitting UE determines the PC5 QoS parameters for this broadcast ProSe service as specified in clauses 5.7.2.

4.	The transmitting UE sends the ProSe data using the source Layer-2 ID and the destination Layer-2 ID.

NOTE:	In step 4, there is only one broadcast message from the transmitting UE.

6.4.2	Groupcast mode ProSe Direct Communication

Figure 6.4.2-1 shows the procedure for groupcast mode of ProSe Direct Communication.

Figure 6.4.2-1: Procedure for groupcast mode ProSe Direct communication

Steps 1 to 3 are optional, e.g. applicable for Application Layer managed group. Groupcast mode ProSe Direct communication can be performed without steps 1 to 3 based on the provisioned parameters as defined in clause 5.1.3.1.

1.	ProSe Group management is carried out at ProSe application layer. This may be performed in coordination with ProSe Application Server.

Group Member Discovery parameters as specified in clause 5.1.2.1 can be provisioned in ME from PCF or configured in UICC or provided from ProSe Application Server to the UE.

2.	Each UE may perform a ProSe group member discovery (similar to the procedures in clause 6.3.2.2) that is restricted only to users sharing the same Application Layer Group ID obtained in step 1.

The Application layer discovery messages including the Application Layer Group ID for Application Layer managed group may be exchanged between UEs to discover each other sharing the same Application Layer Group ID. When the group is formed, the Application layer discovery message including group size and member ID may be provided by one UE to all other UEs.

Editor's note:	Whether to transmit the Application layer discovery messages as metadata in a PC5 direct discovery message or as user traffic depends on the size of the PC5 direct discovery message. The size of the PC5 direct discovery message or no size limit will be determined by RAN WG.

3.	The ProSe Application layer may provide group identifier information (i.e. the Application Layer Group ID discovered based on discovery messages exchanged in step 2) as specified in clause 5.9.2.3.

The ProSe application layer may also provide ProSe Application Requirements for this communication.

If the ProSe application layer does not provide ProSe Application Requirements, the ProSe layer determines the PC5 QoS parameters based on the mapping of ProSe service to PC5 QoS parameters as specified in clause 5.1.3.1.

The ProSe application layer may provide a group size and a member ID (optionally based on discovery messages exchanged in step 2) as specified in clause 5.3.3 for Application Layer managed group.

4.	Transmitting UE self-assigns a source Layer-2 ID and determines a destination Layer-2 ID and Receiving UE(s) determine destination Layer-2 ID as specified in clause 5.9.2.

Transmitting UE determines the PC5 QoS parameters for this groupcast mode communication as specified in clause 5.7.1.

If the group size and the member ID for Application Layer managed group are provided by the ProSe application layer, the ProSe layer passes them to the AS layer as described in clause 5.3.3.

5.	Transmitting UE sends the ProSe data using the source Layer-2 ID and the destination Layer-2 ID.

NOTE:	In step 5, there is only one groupcast message from the transmitting UE. That is, all receiving UEs can receive the groupcast message directly from the transmitting UE.

6.4.3	Unicast mode ProSe Direct Communication

6.4.3.1	Layer-2 link establishment over PC5 reference point

To perform unicast mode of ProSe Direct communication over PC5 reference point, the UE is configured with the related information as described in clause 5.1.3.

Figure 6.4.3.1-1 shows the layer-2 link establishment procedure for the unicast mode of ProSe Direct communication over PC5 reference point.

Figure 6.4.3.1-1: Layer-2 link establishment procedure

1.	The UE(s) determine the destination Layer-2 ID for signalling reception for PC5 unicast link establishment as specified in clause 5.9.2. The destination Layer-2 ID is configured with the UE(s) as specified in clause 5.1.3.

2.	The ProSe application layer in UE-1 provides application information for PC5 unicast communication. The application information includes the ProSe Service Info, UE's Application Layer ID. The target UE's Application Layer ID may be included in the application information.

The ProSe application layer in UE-1 may provide ProSe Application Requirements for this unicast communication. UE-1 determines the PC5 QoS parameters and PFI as specified in clause 5.7.

If UE-1 decides to reuse the existing PC5 unicast link as specified in clause 5.3.4, the UE triggers the Layer-2 link modification procedure as specified in clause 6.4.3.4.

3.	UE-1 sends a Direct Communication Request message to initiate the unicast layer-2 link establishment procedure. The Direct Communication Request message includes:

-	Source User Info: the initiating UE's Application Layer ID (i.e. UE-1's Application Layer ID).

-	If the ProSe application layer provided the target UE's Application Layer ID in step 2, the following information is included:

-	Target User Info: the target UE's Application Layer ID (i.e. UE-2's Application Layer ID).

-	ProSe Service Info: the information about the ProSe service type(s) requesting Layer-2 link establishment.

-	Security Information: the information for the establishment of security.

NOTE 1:	The Security Information and the necessary protection of the Source User Info and Target User Info are defined by SA WG3.

The source Layer-2 ID and destination Layer-2 ID used to send the Direct Communication Request message are determined as specified in clause 5.9.2. The destination Layer-2 ID may be broadcast or unicast Layer-2 ID. When unicast Layer-2 ID is used, the Target User Info shall be included in the Direct Communication Request message.

UE-1 sends the Direct Communication Request message via PC5 broadcast or unicast using the source Layer-2 ID and the destination Layer-2 ID.

4.	Security with UE-1 is established as below:

4a.	If the Target User Info is included in the Direct Communication Request message, the target UE, i.e. UE-2, responds by establishing the security with UE-1.

4b.	If the Target User Info is not included in the Direct Communication Request message, the UEs that are interested in using the announced ProSe Service(s) over a PC5 unicast link with UE-1 responds by establishing the security with UE-1.

NOTE 2:	The signalling for the Security Procedure is defined by SA WG3.

When the security protection is enabled, UE-1 sends the following information to the target UE:

-	If IP communication is used:

-	IP Address Configuration: For IP communication, IP address configuration is required for this link and indicates one of the following values:

-	"DHCPv4 server " if only IPv4 address allocation mechanism is supported by the target UE, i.e., acting as a DHCPv4 server; or

-	"IPv6 Router" if only IPv6 address allocation mechanism is supported by the initiating UE, i.e., acting as an IPv6 Router; or

-	"DHCPv4 server & IPv6 Router" if both IPv4 and IPv6 address allocation mechanism are supported by the initiating UE; or

-	"address allocation not supported" if neither IPv4 nor IPv6 address allocation mechanism is supported by the initiating UE.

-	Link-Local IPv6 Address: a link-local IPv6 address formed locally based on RFC 4862 [17] if UE-1 does not support the IPv6 IP address allocation mechanism, i.e. the IP Address Configuration indicates " address allocation not supported ".

-	QoS Info: the information about PC5 QoS Flow(s). For each PC5 QoS Flow, the PFI and the corresponding PC5 QoS parameters (i.e. PQI and conditionally other parameters such as MFBR/GFBR, etc.).

The source Layer-2 ID used for the security establishment procedure is determined as specified in clause 5.9.2. The destination Layer-2 ID is set to the source Layer-2 ID of the received Direct Communication Request message.

Upon receiving the security establishment procedure messages, UE-1 obtains the peer UE's Layer-2 ID for future communication, for signalling and data traffic for this unicast link.

5.	A Direct Communication Accept message is sent to UE-1 by the target UE(s) that has successfully established security with UE-1:

5a.	(UE oriented Layer-2 link establishment) If the Target User Info is included in the Direct Communication Request message, the target UE, i.e. UE-2 responds with a Direct Communication Accept message if the Application Layer ID for UE-2 matches.

5b.	(ProSe Service oriented Layer-2 link establishment) If the Target User Info is not included in the Direct Communication Request message, the UEs that are interested in using the announced ProSe Service(s) respond to the request by sending a Direct Communication Accept message (UE-2 and UE-4 in Figure 6.3.3.1-1).

The Direct Communication Accept message includes:

-	Source User Info: Application Layer ID of the UE sending the Direct Communication Accept message.

-	QoS Info: the information about PC5 QoS Flow(s). For each PC5 QoS Flow, the PFI and the corresponding PC5 QoS parameters requested by UE-1 (i.e. PQI and conditionally other parameters such as MFBR/GFBR, etc).

-	If IP communication is used:

-	IP Address Configuration: For IP communication, IP address configuration is required for this link and indicates one of the following values:

-	" DHCPv4 server " if only IPv4 address allocation mechanism is supported by the target UE, i.e., acting as a DHCPv4 server; or

-	"IPv6 Router" if only IPv6 address allocation mechanism is supported by the target UE, i.e., acting as an IPv6 Router; or

-	"DHCPv4 server & IPv6 Router" if both IPv4 and IPv6 address allocation mechanism are supported by the target UE; or

-	"address allocation not supported" if neither IPv4 nor IPv6 address allocation mechanism is supported by the target UE.

-	Link-Local IPv6 Address: a link-local IPv6 address formed locally based on RFC 4862 [17] if the target UE does not support the IPv6 IP address allocation mechanism, i.e. the IP Address Configuration indicates "address allocation not supported", and UE-1 included a link-local IPv6 address in the Direct Communication Request message. The target UE shall include a non-conflicting link-local IPv6 address.

If both UEs (i.e. the initiating UE and the target UE) are selected to use link-local IPv6 address, they shall disable the duplicate address detection defined in RFC 4862 [17].

NOTE 3:	When either the initiating UE or the target UE indicates the support of IPv6 routing, the corresponding address configuration procedure would be carried out after the establishment of the layer 2 link, and the link-local IPv6 addresses are ignored.

The ProSe layer of the UE that established PC5 unicast link passes the PC5 Link Identifier assigned for the unicast link and the PC5 unicast link related information down to the AS layer. The PC5 unicast link related information includes Layer-2 ID information (i.e. source Layer-2 ID and destination Layer-2 ID). This enables the AS layer to maintain the PC5 Link Identifier together with the PC5 unicast link related information.

6.	ProSe service data is transmitted over the established unicast link as below:

The PC5 Link Identifier and PFI are provided to the AS layer, together with the ProSe service data.

Optionally in addition, the Layer-2 ID information (i.e. source Layer-2 ID and destination Layer-2 ID) is provided to the AS layer.

NOTE 4:	It is up to UE implementation to provide the Layer-2 ID information to the AS layer.

UE-1 sends the ProSe service data using the source Layer-2 ID (i.e. UE-1's Layer-2 ID for this unicast link) and the destination Layer-2 ID (i.e. the peer UE's Layer-2 ID for this unicast link).

NOTE 5:	PC5 unicast link is bi-directional, therefore the peer UE of UE-1 can send the ProSe service data to UE-1 over the unicast link with UE-1.

6.4.3.2	Link identifier update for a unicast link

Figure 6.4.3.2-1 shows the link identifier update procedure for a unicast link. When privacy requirements are configured for a ProSe service type associated with the unicast link, identifiers used for the unicast mode of ProSe communication over PC5 reference point (e.g. Application Layer ID, Source Layer-2 ID, and IP address/prefix) shall be changed over time as specified in clauses 5.9.2. A UE may decide to change the identifiers for other reasons, e.g. application layer requirement. This procedure is used to update and exchange new identifiers between the source and the peer UEs for a unicast link before using the new identifiers, to prevent service interruptions. When there are privacy requirements as indicated above, this procedure is executed over a security protected unicast link.

If a UE has multiple unicast links using the same Application Layer IDs or Layer-2 IDs, the UE needs to perform the link identifier update procedure over each of the unicast links.

Figure 6.4.3.2-1: Link identifier update procedure

0.	UE-1 and UE-2 have a unicast link established as described in clause 6.4.3.1.

1.	UE-1 decides to change its identifier(s), e.g. due to the Application Layer ID change or upon expiry of a timer. UE-1 generates its new Layer-2 ID and sends a Link Identifier Update Request message to UE-2 using the old identifiers.

The Link Identifier Update Request message includes the new identifier(s) to use (including the new Layer-2 ID, Security Information, optionally the new Application Layer ID, and optionally new IP address/prefix if IP communication is used). The new identifier(s) shall be cyphered to protect privacy if security is configured for the unicast link. After sending the Link Identifier Update Request message, if the UE-1 has data to send, UE-1 keeps sending data traffic to UE-2 with the old identifiers until UE-1 sends the Link Identifier Update Ack message to UE-2.

NOTE 1:	The timer is running on per Source Layer-2 ID.

NOTE 2:	When one of the two UEs acts as IPv6 router as described in clause 5.6 and the IP address/prefix also needs to be changed, the corresponding address configuration procedure would be carried out after the Link Identifier update procedure.

2.	Upon reception of the Link Identifier Update Request message, UE-2 changes its identifier(s). UE-2 responds with a Link Identifier Update Response message which includes the new identifier(s) to use (including the new Layer-2 ID, Security Information, optionally the new Application Layer ID, and optionally a new IP address/prefix if IP communication is used). The new identifier(s) shall be cyphered to protect privacy if security is configured for the unicast link. The Link Identifier Update Response message is sent using the old identifiers. UE-2 continues to receive traffic with the old Layer-2 ID from UE-1 until UE-2 receives traffic with the new Layer-2 ID from UE-1. After sending the Link Identifier Update Response message, UE-2 keeps sending data traffic to UE-1 with the old identifier, if UE-2 has data to send, until UE-2 receives the Link Identifier Update Ack message from UE-1.

3.	Upon reception of the Link Identifier Update Response message, UE-1 responds with a Link Identifier Update Ack message. The Link Identifier Update Ack message includes the new identifier(s) from UE-2, as received on the Link Identifier Update Response message. The Link Identifier Update Ack message is sent using the old identifiers. UE-1 continues to receive traffic with the old Layer-2 ID from UE-2 until UE-1 receives traffic with the new Layer-2 ID from UE-2.

4.	The ProSe layer of UE-1 passes the PC5 Link Identifier for the unicast link and the updated Layer-2 IDs (i.e. new Layer-2 ID for UE-1 for the source and new Layer-2 ID of UE-2 for the destination) down to the AS layer. This enables the AS layer to update the provided Layer-2 IDs for the unicast link.

UE-1 starts using its new identifiers and UE-2's new identifiers for this unicast link.

5.	Upon reception of the Link Identifier Update Ack message, the ProSe layer of UE-2 passes the PC5 Link Identifier for the unicast link and the updated Layer-2 IDs (i.e. new Layer-2 ID of UE-2 for the source and new Layer-2 ID for UE-1 for the destination) down to the AS layer. This enables the AS layer to update the provided Layer-2 IDs for the unicast link.

UE-2 starts using its new identifiers and UE-1's new identifiers for this unicast link.

NOTE 3:	The Security Information in the above messages also needs to be updated at the same time as the Layer-2 IDs. This is defined in TS 33.XXX [TBD].

6.4.3.3	Layer-2 link release over PC5 reference point

Figure 6.4.3.3-1 shows the layer-2 link release procedure over PC5 reference point.

Figure 6.4.3.3-1: Layer-2 link release procedure

0.	UE-1 and UE-2 have a unicast link established as described in clause 6.4.3.1.

1.	UE-1 sends a Disconnect Request message to UE-2 in order to release the layer-2 link and deletes all context data associated with the layer-2 link. The Disconnect Request message includes Security Information.

2.	Upon reception of the Disconnect Request message, UE-2 shall respond with a Disconnect Response message and deletes all context data associated with the layer-2 link. The Disconnect Response message includes Security Information.

The ProSe layer of each UE informs the AS layer that the unicast link has been released. The ProSe layer uses PC5 Link Identifier to indicate the released unicast link. This enables the AS layer to delete the context related to the released unicast link.

NOTE:	The Security Information in the above messages is defined in TS 33.YYY [TBD].

6.4.3.4	Layer-2 link modification for a unicast link

Figure 6.4.3.4-1 shows the layer-2 link modification procedure for a unicast link. This procedure is used to:

-	add new PC5 QoS Flow(s) in the existing PC5 unicast link.

-	This covers the case for adding new PC5 QoS Flow(s) to the existing ProSe service(s) as well as the case for adding new PC5 QoS Flow(s) to new ProSe service(s).

-	modify existing PC5 QoS Flow(s) in the existing PC5 unicast link.

-	This covers the case for modifying the PC5 QoS parameters for existing PC5 QoS Flow(s).

-	This also covers the case for removing the associated ProSe service(s) from existing PC5 QoS Flow(s) as well as the case for associating new ProSe service(s) with existing PC5 QoS Flow(s).

-	remove existing PC5 QoS Flow(s) in the existing PC5 unicast link.

Figure 6.4.3.4-1: Layer-2 link modification procedure

0.	UE-1 and UE-2 have a unicast link established as described in clause 6.4.3.1.

1.	The ProSe application layer in UE-1 provides application information for PC5 unicast communication. The application information includes the ProSe Service Info and the initiating UE's Application Layer ID. The target UE's Application Layer ID may be included in the application information. If UE-1 decides to reuse the existing PC5 unicast link as specified in clause 5.3.4, so decides to modify the unicast link established with UE-2, UE-1 sends a Link Modification Request to UE-2.

The Link Modification Request message includes:

a)	To add new PC5 QoS Flow(s) in the existing PC5 unicast link:

-	QoS Info: the information about PC5 QoS Flow(s) to be added. For each PC5 QoS Flow, the PFI, the corresponding PC5 QoS parameters (i.e. PQI and conditionally other parameters such as MFBR/GFBR, etc) and the associated ProSe service type(s).

b)	To modify PC5 QoS Flow(s) in the existing PC5 unicast link:

-	QoS Info: the information about PC5 QoS Flow(s) to be modified. For each PC5 QoS Flow, the PFI, the corresponding PC5 QoS parameters (i.e. PQI and conditionally other parameters such as MFBR/GFBR, etc.) and the associated ProSe service type(s).

c)	To remove PC5 QoS Flow(s) in the existing PC5 unicast link:

-	PFIs.

2.	UE-2 responds with a Link Modification Accept message.

The Link Modification Accept message includes:

-	For case a) and case b) described in step 1:

-	QoS Info: the information about PC5 QoS Flow(s) requested by UE-1. For each PC5 QoS Flow, the PFI, the corresponding PC5 QoS parameters (i.e. PQI and conditionally other parameters such as MFBR/GFBR, etc) and the associated ProSe service type(s).

The ProSe layer of each UE provides information about the unicast link modification to the AS layer. This enables the AS layer to update the context related to the modified unicast link.

6.4.3.5	Layer-2 link maintenance over PC5 reference point

The PC5 Signalling Protocol shall support keep-alive functionality that is used to detect if a particular PC5 unicast link is still valid. Either side of the PC5 unicast link can initiate the layer-2 link maintenance procedure (i.e. keep-alive procedure), based on for example triggers from the AS layer or internal timers. The UEs shall minimize the keep-alive signalling, e.g. cancel the procedure if data are successfully received over the PC5 unicast link.

Figure 6.4.3.5-1: Layer-2 link maintenance procedure

0.	UE-1 and UE-2 have a unicast link established as described in clause 6.4.3.1.

1.	Based on trigger conditions, UE-1 sends a Keep-alive message to UE-2 in order to determine the status of the PC5 unicast link.

NOTE 1:	It is left to Stage 3 to determine the exact triggers for the keep-alive messages. For example, the trigger can be based on a timer associated with the Layer-2 link. The timer can be reset with a successful reception event defined by TS 38.300 [12].

2.	Upon reception of the Keep-alive message, UE-2 responds with a Keep-alive Ack message.

The UE initiating the keep-alive procedure shall determine the follow-up actions based on the result of the signalling, e.g. proceed with implicit layer-2 link release.

NOTE 2:	It is left to Stage 3 to determine the follow-up actions. For example, a successful reception event can also cancel the layer-2 link release if received in time.

6.5	ProSe UE-to-Network Relay Communication

6.5.1	ProSe Communication via Layer-3 UE-to-Network Relay

6.5.2	ProSe Communication via Layer-2 UE-to-Network Relay

#### 6.5.2.1	Registration and Connection Management

##### 6.5.2.1.1	Registration Management

Registration Management for the Remote UE and the ProSe UE-to-Network Relay follows the principles and procedures defined in TS 23.501 [4] and TS 23.502 [5]. The Remote UE and the ProSe UE-to-Network Relay may be served by the same AMF or different AMFs.

##### 6.5.2.1.2	Connection Management

Connection Management for the Remote UE and the ProSe UE-to-Network Relay follows the principles and procedures defined in TS 23.501 [4] and TS 23.502 [5] with the following modifications.

The ProSe UE-to-Network Relay may only relay data/signalling for the Remote UE(s) when the ProSe UE-to-Network Relay is in CM-CONNECTED state. If the ProSe UE-to-Network Relay is in CM_IDLE state and receives a connection request from the Remote UE for relaying, the ProSe UE-to-Network Relay shall trigger Service Request procedure to enter CM_CONNECTED state before relaying the Remote UEs traffic.

-	If any Remote UE connected to the ProSe UE-to-Network Relay is in CM-CONNECTED, the ProSe UE-to-Network Relay should remain CM-CONNECTED state.

-	If all Remote UEs connected to the ProSe UE-to-Network Relay enter CM-IDLE, the ProSe UE-to-Network Relay may enter CM-IDLE state.

Editor’s Note:	The state handling may need updating once the AS layer aspects are defined.

When Remote UE is CM-IDLE or CM-CONNECTED, the ProSe UE-to-Network Relay and Remote UE keep the PC5 link.

For paging a Remote UE, it follows the principles and procedures defined in TS 23.501 [4] and TS 23.502 [5], and the paging message delivery from NG-RAN to Remote UE is specified in TS 38.XXX.

#### 6.5.2.2	Connection establishment

Figure 6.5.2.1-1: Connection Establishment for L2 UE-to-Network Relay

0.	If in coverage, the Remote UE and ProSe UE-to-Network Relay may independently perform the initial registration to the network according to registration procedures in TS 23.502 [5].

1.	If in coverage, the Remote UE and ProSe UE-to-Network Relay independently get the service authorization for ProSe UE-to-Network Relay operation from the network. Service authorization and parameters provisioning for ProSe UE-to-Network Relay operation are performed for the ProSe UE-to-Network Relay and Remote UE as specified in clause 5.1.4.

If the Remote UE is not in coverage, the pre-configured parameters are used, and the service authorization and parameters may be updated after step 6.

If the Remote UE has not performed Initial Registration, the Remote UE may perform the Initial Registration in step 6.

2.	The Remote UE and ProSe UE-to-Network Relay perform UE-to-Network Relay UE Discovery and selection, as specified in clause 6.3.2.3.

3.	The Remote UE initiates a one-to-one communication connection with the selected ProSe UE-to-Network Relay over PC5 using the procedure as described in clause 6.4.3.

4.	If the ProSe UE-to-Network Relay is in CM_IDLE state, triggered by the request received from the Remote UE, the ProSe UE-to-Network Relay performs Service Request procedure in the clause 4.2.3.2 of TS 23.502 [5].

Editor's Note:	How the ProSe UE-to-Network Relay is triggered to perform Service Request procedure requires cooperation with RAN2.

5.	The Remote UE establishes an RRC Connection with the same NG-RAN serving the selected ProSe UE-to-Network Relay, specified in TS 38.XXX.

6.	The Remote UE sends a NAS message to the serving AMF. The NAS message is encapsulated in an Uu RRC message that is sent over PC5 to the ProSe UE-to-Network Relay, and the ProSe UE-to-Network Relay forwards the Uu RRC message to the NG-RAN specified in TS 38.XXX. NG-RAN selects the Remote UE's serving AMF and forwards the NAS message to this AMF.

If Remote UE has not performed the initial registration, the NAS message is an initial Registration message. Otherwise, the NAS message is either a service request message, or a mobility or periodic Registration message.

7.	The Remote UE may trigger the PDU Session Establishment procedure as defined in clause 4.3.2.2 of TS 23.502 [5].

8.	The data is transferred between the Remote UE and UPF via the ProSe UE-to-Network Relay and NG-RAN. The ProSe UE-to-Network Relay forwards all the data messages between the Remote UE and NG-RAN, as specified in TS 38.XXX.

6.6	ProSe UE-to-UE Relay Communication

Editor's note:	This clause will document the detailed procedures of the direct communication via ProSe UE-to-UE Relay and base on the KI# 4 conclusions of TR 23.752 [21].

6.7	Procedures for Service Authorization to NG-RAN

6.7.1	General

In order to support PC5 radio resource control in NG-RAN, ProSe service Authorisation information and PC5 QoS parameters for ProSe need to be made available in NG-RAN. This clause describes the corresponding procedures and aspects.

6.7.2	Registration procedure

The Registration procedure for UE is performed as defined in TS 23.502 [5] clause 4.2.2.2 with the following additions:

-	The UE includes the 5G ProSe Capability as part of the "5GMM capability" in the Registration Request message. The AMF stores this information for ProSe operation. The 5G ProSe Capability indicates whether the UE is capable of one or more of the following ProSe capabilities: ProSe Direct Discovery, ProSe Direct Communication, ProSe UE-to-Network Relay (Layer-2 and/or Layer-3), and Remote UE (Layer-2 and/or Layer-3).

-	The AMF obtains the ProSe subscription data as part of the user subscription data from UDM during UE Registration procedure using Nudm_SDM service as defined in clause 4.2.2.2.2 of TS 23.502 [5].

-	The AMF determines whether the UE is authorised to use ProSe services based on UE's ProSe Capability and the ProSe Service Authorisation included in the subscription data received from UDM as specified in clause 5.8. ProSe NR UE-PC5-AMBR is also provided to the AMF as part of the subscription data for ProSe services.

-	The PCF provides the PC5 QoS parameters for ProSe to AMF. The AMF stores such information as part of the UE context.

-	If the UE is authorised to use ProSe services, then the AMF shall include in a NGAP message sent to NG-RAN:

-	"ProSe authorised" information, including one or more of the following ProSe services:

-	whether the UE is authorized to use ProSe Direct Discovery;

-	whether the UE is authorized to use ProSe Direct Communication;

-	whether the UE is authorized to act as a Layer-2 ProSe UE-to-Network Relay;

-	whether the UE is authorized to act as a Layer-3 ProSe UE-to-Network Relay;

-	whether the UE is authorized to act as a Layer-2 Remote UE.

-	ProSe NR UE-PC5-AMBR, used by NG-RAN for the resource management of UE's PC5 transmission for ProSe services in network scheduled mode.

-	the PC5 QoS parameters for ProSe used by the NG-RAN for the resource management of UE's PC5 transmission for ProSe services in network scheduled mode.

-	If the UE is authorised to use ProSe services, then the AMF should not initiate the release of the signalling connection after the completion of the Registration procedure. The release of the signalling connection relies on the decision of NG-RAN, as specified in TS 23.502 [5].

6.7.3	Service Request procedure

The Service Request procedures for UE in CM-IDLE state are performed as defined in TS 23.502 [5] clause 4.2.3.2 and clause 4.2.3.3 with the following additions:

-	If the UE is authorised to use ProSe services, then the AMF shall include "ProSe authorised" information in the NGAP message, indicating which of the ProSe services the UE is authorised to use as described in clause 6.7.2.

-	The AMF includes the ProSe NR UE-PC5-AMBR in the NGAP message to the NG-RAN as part of the UE context and NG-RAN may use in resource management of UE's PC5 transmission for ProSe services in network scheduled mode.

-	The AMF sends the PC5 QoS parameters for ProSe to NG-RAN via N2 signalling. The PC5 QoS parameters for ProSe may be stored in the UE context after the registration procedure. If the UE is authorised to use ProSe services but AMF does not have PC5 QoS parameters for ProSe available, the AMF fetches the PC5 QoS parameters for ProSe from the PCF.

6.7.4	N2 Handover procedure

The N2 based handover procedures for UE are performed as defined in TS 23.502 [5] clause 4.9.1.3 with the following additions:

-	If the UE is authorised to use ProSe services, then the target AMF shall send the "ProSe authorised" information, ProSe NR UE-PC5-AMBR, and PC5 QoS parameters for ProSe to the target NG-RAN in the NGAP Handover Request message.

6.7.5	Xn Handover procedure

The Xn based handover procedures for UE are performed as defined in TS 23.502 [5] clause 4.9.1.2 with the following additions:

-	If the "ProSe authorised" information is included in the UE context, then the source NG-RAN shall include a "ProSe authorised" information, ProSe NR UE-PC5-AMBR and PC5 QoS parameters for ProSe in the XnAP Handover Request message to the target NG-RAN.

-	If the "ProSe authorised" information is included in the UE context, then the AMF shall send the "ProSe authorised" information, the ProSe NR UE-PC5-AMBR, and PC5 QoS parameters for ProSe to the target NG-RAN in the Path Switch Request Acknowledge message.

6.7.6	Subscriber Data Update Notification to AMF

The procedure of Subscriber Data Update Notification to AMF is performed as defined in TS 23.502 [5] clause 4.5.1 with the following additions:

-	The Nudm_SDM_Notification service operation may contain the ProSe Service Authorisation or the ProSe NR UE-PC5-AMBR or any combination.

-	The AMF updates the UE Context with the above new ProSe subscription data.

-	When the AMF updates UE context stored at NG-RAN, the UE context contains the ProSe subscription data.

6.7.7	Delivery of PC5 QoS parameters for ProSe to NG-RAN

The procedure specified in the clause 6.5.7 of TS 23.287 [2] is used for the delivery of the PC5 QoS parameters for ProSe to NG-RAN, with the following changes:

-	The PC5 capability for ProSe is checked by the PCF during the Npcf_UEPolicyControl_Create request instead of the PC5 capability for V2X.

7	Network Function Services

7.1	5G DDNMF Services

7.1.1	General

The following table illustrates the 5G DDNMF Services.

Table 7.1.1-1: Services provided by 5G DDNMF

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| N5g-ddnmf_Discovery | AnnounceAuthorize | Request/Response | 5G DDNMF |
|  | AnnounceUpdate | Request/Response | 5G DDNMF |
|  | MonitorAuthorize | Request/Response | 5G DDNMF |
|  | MonitorUpdate | Request/Response | 5G DDNMF |
|  | MonitorUpdateResult | Notify | 5G DDNMF |
|  | DiscoveryAuthorize | Request/Response | 5G DDNMF |
|  | MatchReport | Request/Response | 5G DDNMF |
|  | MatchInformation | Notify | 5G DDNMF |

7.1.2	N5g-ddnmf_Discovery service

7.1.2.1	General

Service description: This service enables a 5G DDNMF to manage inter-PLMN ProSe Direct Discovery operations.

7.1.2.2	N5g-ddnmf_Discovery_AnnounceAuthorize service operation

Service operation name: N5g-ddnmf_Discovery_AnnounceAuthorize

Description: The consumer NF obtains the authorization from the 5G DDNMF for announcing in the PLMN.

Input, Required: Discovery type ("open" or " restricted ") and

-	(for "open" discovery type:) ProSe Application ID, ProSe Application Code, UE Identity, validity timer, Discovery Entry ID,

-	(for "restricted" discovery type:) RPAUID, Application ID, ProSe Restricted Code/Prefix, UE Identity, Discovery Entry ID,

Input, Optional: metadata, Restricted Code Suffix pool.

Output, Required: authorization result.

Output, Optional: None.

7.1.2.3	N5g-ddnmf_Discovery_AnnounceUpdate service operation

Service operation name: N5g-ddnmf_Discovery_AnnounceUpdate

Description: The consumer NF updates or revoke the authorization from the 5G DDNMF for announcing in the PLMN.

Input, Required: Discovery type = "open", UE Identity, validity timer, Discovery Entry ID

Input, Optional: ProSe Application Code

Output, Required: result.

Output, Optional: None.

7.1.2.4	N5g-ddnmf_Discovery_MonitorAuthorize service operation

Service operation name: N5g-ddnmf_Discovery_MonitorAuthorize

Description: The consumer NF obtains the authorization from the 5G DDNMF for monitoring in the PLMN.

Input, Required: Discovery type ("open" or " restricted ") and

-	(for "open" discovery type:) ProSe Application ID Name(s), UE Identity, Discovery Entry ID;

-	(for "restricted" discovery type:) RPAUID, UE Identity, Target PDUID, Application ID, Target RPAUID, Discovery Entry ID,

Input, Optional: None,

Output, Required: (for "open" discovery) ProSe Application Code(s)/Prefix, ProSe Application Mask(s), TTL; or (for "restricted" discovery) ProSe Restricted Code, validity timer

Output, Optional: None.

7.1.2.5	N5g-ddnmf_Discovery_MonitorUpdate service operation

Service operation name: N5g-ddnmf_Discovery_MonitorUpdate

Description: The consumer NF updates or revoke the authorization for the indicated UE to monitor in the PLMN.

Input, Required: Discovery type ("open" or " restricted ") and,

-	(for "open" discovery type:) ProSe Application ID name, UE Identity, TTL, Discovery Entry ID

-	(for "restricted" discovery type:) ProSe Restricted Code, Application ID, Banned RPAUID, Banned PDUID

Input, Optional: None

Output, Required: result

Output, Optional: None

7.1.2.6	N5g-ddnmf_Discovery_MonitorUpdateResult service operation

Service operation name: N5g-ddnmf_Discovery_MonitorUpdateResult

Description: The consumer NF informs the 5G DDNMF of the monitoring revocation results.

Input, Required: Discovery type = "restricted", ProSe Restricted Code, Application ID, Banned RPAUID, Banned PDUID, Result

Input, Optional: None

Output, Required: None

Output, Optional: None

7.1.2.7	N5g-ddnmf_Discovery_DiscoveryAuthorize service operation

Service operation name: N5g-ddnmf_Discovery_DiscoveryAuthorize

Description: The consumer NF obtains the authorization from the 5G DDNMF for a discoverer UE in the PLMN to operate Model B restricted discovery.

Input, Required: Discovery type = "restricted", Restricted ProSe App User ID, UE Identity, Target PDUID, Application ID, Target RPAUID, Discovery Entry ID,

Input, Optional: None

Output, Required: ProSe Query Code(s), ProSe Response Code, validity timer,

Output, Optional: None

7.1.2.8	N5g-ddnmf_Discovery_MatchReport service operation

Service operation name: N5g-ddnmf_Discovery_MatchReport

Description: The consumer NF obtains the information about the indicated discovery code from the 5G DDNMF.

Input, Required: Discovery type = "open", ProSe Application Code(s), UE identity, Monitored PLMN ID

Input, Optional: None

Output, Required: ProSe Application ID Name(s), validity timer(s).

Output, Optional: Metadata, Metadata Index Mask(s)

7.1.2.9	N5g-ddnmf_Discovery_MatchInformation service operation

Service operation name: N5g-ddnmf_Discovery_MatchInformation

Description: The consumer NF receives from the 5G DDNMF of a matching result, and the information can be used for charging purpose.

Input, Required: Discovery type ("open" or " restricted ") and,

-	(for "open" type:) ProSe Application ID(s), UE Identity

-	(for "restricted" type:) RPAUID, Target RPAUID, UE Identity, ProSe Restricted Code

Input, Optional: None

Output, Required: None

Output, Optional: None

Annex A (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2021-03 | SA2#143e |  |  |  |  | Skeleton for this TS (approved in S2-2101633) | 0.0.0 |
| 2021-03 | SA2#143e |  |  |  |  | Inclusion of documents approved in SA2#143E: S2-2100211, S2-2100403, S2-2100432, S2-2100454, S2-2100686, S2-2100738, S2-2100777, S2-2100779, S2-2100939, S2-2100944, S2-2100960, S2-2100963, S2-2100999, S2-2101634, S2-2101635, S2-2101636, S2-2101637, S2-2101638, S2-2101639, S2-2101640, S2-2101641, S2-2101642, S2-2101643, S2-2101644, S2-2101645, S2-2101646, S2-2101647, S2-2101648, S2-2101649, S2-2101650, S2-2101651, S2-2101652, S2-2101653, S2-2101654, S2-2101655, S2-2101656, | 0.1.0 |
| 2021-04 | SA2#144e |  |  |  |  | Inclusion of documents approved in SA2#144E: S2-2102200, S2-2102203, S2-2102241, S2-2102423, S2-2102431, S2-2102485, S2-2102585, S2-2102633, S2-2102812, S2-2102875,S2-2102877, S2-2103495, S2-2103496, S2-2103497, S2-2103498, S2-2103499, S2-2103500, S2-2103501, S2-2103502, S2-2103503, S2-2103504, S2-2103505, S2-2103506, S2-2103507, S2-2103509, S2-2103510, S2-2103511, S2-2103512, S2-2103513, S2-2103514, S2-2103515, S2-2103516, S2-2103517, S2-2103518, S2-2103519, S2-2103520, S2-2103521, S2-2103523, S2-2103524, S2-2103525, | 0.2.0 |