SA WG2 Meeting #144e	S2-2102162r03

# April 12 – 16, 2021; Elbonia             		 	(revision of S2-210)

Source: 	Nokia, Nokia Shanghai Bell

Title: 	 Issue that we cannot guarantee that the UE uses the EASDF's IP address for its subsequent DNS Query

Document for: 	Agreement (P-CR)

Agenda Item: 	8.3

Work Item / Release:	eEDGE_5GC / Rel-17

Abstract of the contribution: Issue that we cannot guarantee that the UE uses the EASDF's IP address for its subsequent DNS Query

# 1 Discussion

# 2 Proposal

It is proposed to update TS 23.548 as follows

FIRST CHANGE only the part of the clause 6.2.3.2.2 that is modified by the PCR is shown, the rest of the clause is unchanged by the PCR

5.	The SMF includes the IP address of the EASDF as DNS server in PDU Session Establishment Accept message as in step 11 of clause 4.3.2.2.1 of TS 23.502 [3]. The UE configures the EASDF as DNS server for that PDU Session.

NOTE: 	It is not possible to guarantee that all UE(s) use the EASDF's IP address provided by the SMF for the DNS Query from the UE (e.g. in step 8); some network-based mechanisms are nevertheless described in step 8 in order to deal with UE that do not use the EASDF's IP address provided by the SMF;

The SMF may thus carry out following action before sending the IP address of the EASDF as DNS server to the UE:

-	The SMF configures the UPF to detect DNS traffic sent by the UE to a different DNS server than the EASDF; this corresponds to a PDR referring to the ports used by DNS protocols (53, 853) and to the EASDF's IP address (that the SMF will send to the UE) and with a corresponding FAR allowing the SMF to carry out the corrective actions described in step 8

NOTE: 	this mechanism can’t deal with DNS requests not sent by the UE over well known port;

6.	The SMF may invoke Neasdf_DNSContext_Update Request (PDU Session Context ID, rules to handle DNS queries from the UE) to EASDF. The update may be triggered by UE mobility, e.g. when UE moves to a new location, or by a reporting by EASDF of a DNS Query with certain FQDN, or, the update may be triggered by insertion/removal of Local PSA, e.g. to update rules to handle DNS messages from the UE or by new PCC rule information.

Editor's note:	It is FFS whether and how the AF-triggered DNS server information change trigger DNS context update procedure.

7.	The EASDF responds with Neasdf_DNSContext_Update Response.

8.	The UE sends DNS Query message.

To handle the case where the UE would not send the DNS Query message to the EASDF: The SMF may, based on local policies, have instructed an UPF to detect (UL) DNS traffic from the UE that is not targeting the EASDF's IP address as described in step 5;

The SMF may, based on local policies, further request the UPF to forward such DNS traffic to the SMF in which case the SMF takes care itself of forwarding further the DNS traffic once it has applied corrective actions ; Or alternatively the UPF may be instructed by the SMF to buffer such DNS traffic and to report to the SMF; in this alternative case, once corrective actions have been carried out by the SMF, the SMF may ask the UPF to forward the buffered DNS traffic.

The local policies mentioned above may be based on the (DNN, S-NSSAI).

NOTE: 	The SMF or UPF can forward the DNS traffic once corrective actions have been carried out even if this DNS traffic from the UE is ciphered as they do not need to interpret it or modify it

The corrective actions carried out by the SMF may be

-	to insert offload capabilities (UL CL and local PSA) based on UE’s location for the PDU Session regardless of the detection (via DNS requests) of the UE willingness to reach an EAS. . Once the SMF has put the PDU Session in the mode where traffic offload capabilities are only inserted based on UE location (and thus not based on EASDF interactions) it can remove the UPF detection of the usage of another DNS server than the EASDF and release the EASDF context associated with the PDU Session;

-	to configure the EASDF to propagate (forward) the DNS requests of the UE towards the IP address used by the UE when sending DNS requests (the SMF provides this IP address to the EASDF) and to send the  DNS request of the UE (that it has received) to the EASDF .

NOTE: 	The latter mechanism is not useful if the UE is using ciphered DNS traffic.

The rest of the procedure assumes the UE has indeed used the EASDF as its network DNS resolver.

NEXT CHANGE (2)

End of changes