3GPP TSG-SA2 Meeting # 144E (e-meeting)	S2-2102176

# Elbonia, 12 April –  16 April, 2021										  		   (revision of S2-2100294)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 2490 | rev | 1 | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: |  |  | UICC apps |  |  |  |  | ME |  | Radio Access Network |  |  |  |  |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Title: | Application Identifier for NEF API ChargebleParty and AF Session with QoS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17, 5GMS3 |  |  |  |  |  |  |  |  |  | Date: |  |  | 2021-04-06 |  |  |  |
| Category: | C |  |  |  |  |  |  |  |  |  | Release: |  |  | Rel-17 |  |  |  |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |  |  |  |
| Reason for change: |  | Incoming LS S2-2100262/S4-210311 is questioning if Application Identifier can be used by AF in Chargeable Party and AF session with QoS APIs. For external parameter provisioning, application identifier information from AF is allowed for the PFD management (clause 4.18) and Service specific parameters (clause 4.15.6.7). For the other cases such as chargeable party and AF session with QoS, it’s indicated in stage 3 that NEF/SCEF can map the AF identifier into AF application identifier. However, in case there are multiple applications in the same AF and they need to be identified as different application, then it’s not possible to distinguish the different applications with the AF identifier only. It shall also be possible for the AF to provide application identifier in AF Session with QoS and Chargeable party APIs without flow description as long as it’s possible to identify the set of flow descriptions (either preconfigured or provisioned via PFD management) linked to the application. The application identifier is also missing in traffic influence flow in 4.3.6.2, though it is specified in clause 5.2.6.7.2 and stage 3 already. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce Application Identifier in AF requests to influence traffic routing flow, in clause 4.3.6.2. Introduce Application Identifier for Chargeble Party at AF session setup, in clause 4.15.6.4/5. Introduce Application ID for AF session with QoS procedures, in clause 4.15.6.6/6a. Introduce Application Identifier in ChargeableParty service operations in clause 5.2.6.8. Introduce Application Identifier in AFSessionWithQoS service operations in clause 5.2.6.9. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Wrong Application Identifier may be mapped by SCEF/NEF when the AF supporting more than one application identifiers, brings wrong application identifier problem in the related charging/billing handling and AF session with QoS. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3.6.2, 4.15.6.4, 4.15.6.5, 4.15.6.6, 4.15.6.6a, 5.2.6.8.2, 5.2.6.8.3, 5.2.6.9.2, 5.2.6.9.5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Y |  | N |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other specs |  |  |  | X | Other core specifications |  |  |  |  |  |  | TS/TR ... CR ... |  |  |  |  |  |
| affected: |  |  |  | X | Test specifications |  |  |  |  |  |  | TS/TR ... CR ... |  |  |  |  |  |
| (show related CRs) |  |  |  | x | O&M Specifications |  |  |  |  |  |  | TS/TR ... CR ... |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

4.15.6.4	Set a chargeable party at AF session setup

Figure 4.15.6.4-1: Set the chargeable party at AF session set-up

1.	When setting up the connection between an ASP sponsoring a session and the UE, the ASP may communicate with the AF to request to become the chargeable party for the session to be set up by sending a Nnef_ChargeableParty_Create request message (AF Identifier, UE address, Flow description(s) or External Application Identifier, Sponsor Information, Sponsoring Status, Background Data Transfer Reference ID,) to the NEF, The Sponsoring Status indicates whether sponsoring is started or stopped, i.e. whether the 3rd party service provider is the chargeable party or not. The Background Data Transfer Reference ID parameter identifies a previously negotiated transfer policy for background data transfer as defined in clause 4.16.7. The NEF assigns a Transaction Reference ID to the Nnef_ChargeableParty_Create request.

2.	The NEF authorizes the AF request to sponsor the application traffic and stores the sponsor information together with the AF Identifier and the Transaction Reference ID. If the authorisation is not granted, step 2 is skipped and the NEF replies to the AF with a Result value indicating that the authorisation failed.

NOTE:	Based on operator configuration, the NEF may skip this step. In this case the authorization is performed by the PCF in step 3.

3.	The NEF interacts with the PCF by triggering a Npcf_PolicyAuthorization_Create request message and provides IP filter information or Ethernet filter information, sponsored data connectivity information (as defined in TS 23.203 [24]), Background Data Transfer Reference ID (if received from the AF) and Sponsoring Status (if received from the AF) to the PCF.

4.	The PCF determines whether the request is allowed and notifies the NEF if the request is not authorized. If the request is not authorized, NEF responds to the AF in step 5 with a Result value indicating that the authorization failed.

5.	The NEF sends a Nnef_ChargeableParty_Create response message (Transaction Reference ID, Result) to the AF. Result indicates whether the request is granted or not.

# * * * * Next change * * * *

4.15.6.6	Setting up an AF session with required QoS procedure

Figure 4.15.6.6-1: Setting up an AF session with required QoS procedure

1.	The AF sends a request to reserve resources for an AF session using Nnef_AFsessionWithQoS_Create request message (UE address, AF Identifier, Flow description(s) or External Application Identifier, QoS reference, (optional) Alternative Service Requirements (containing one or more QoS reference parameters in a prioritized order) , ) to the NEF. Optionally, a period of time or a traffic volume for the requested QoS can be included in the AF request. The NEF assigns a Transaction Reference ID to the Nnef_AFsessionWithQoS_Create request. The AF may in addition provide the following parameters: Requested 5GS delay, Requested GFBR, Requested MFBR, flow direction, Burst Size (optional), Burst Arrival Time (optional) at UE (uplink) or UPF (downlink), Periodicity (optional), Time domain (optional).

2.	The NEF authorizes the AF request and may apply policies to control the overall amount of pre-defined QoS authorized for the AF. If the authorisation is not granted, steps 3 and 4 are skipped and the NEF replies to the AF with a Result value indicating that the authorisation failed.

3.	The NEF interacts with the PCF by triggering a Npcf_PolicyAuthorization_Create request and provides UE address, AF Identifier, Flow description(s), the QoS reference and the optional Alternative Service Requirements (containing one or more QoS reference parameters in a prioritized order). Any optionally received period of time or traffic volume is also included and mapped to sponsored data connectivity information (as defined in TS 23.203 [24]).

If a Requested 5GS delay is provided by the AF and if the UE-residence-time is provided by PCF, the NEF calculates a Requested PDB by subtracting the UE-residence-time from the Requested 5GS delay. The NEF sends the Requested PDB, Requested GFBR, Requested MFBR, Burst Size, and Time domain and the TSC Assistance Container (including flow direction, Periodicity, Burst Arrival Time) to the PCF.

4.	The PCF determines whether the request is authorized and notifies the NEF if the request is not authorized.

If the request is authorized, the PCF derives the required QoS parameters based on the information provided by the NEF and determines whether this QoS is allowed (according to the PCF configuration for this AF), and notifies the result to the NEF. In addition, if the Alternative Service Requirements are provided, the PCF derives the Alternative QoS parameter set(s) from the one or more QoS reference parameters contained in the Alternative Service Requirements in the same prioritized order (as defined in TS 23.503 [20]).

If the NEF provides Requested PDB, Requested GFBR, Requested MFBR or Burst Size, then the PCF sets the PDB and MDBV according to the received Requested PDB and Burst Size received from the NEF. It also sets the GFBR and MFBR according to requested values sent by the NEF. NEF specified parameter values are used to over-ride default values for the 5QI corresponding to the NEF provided QoS Reference.

If the PCF determines that the SMF needs updated policy information, the PCF issues a Npcf_SMPolicyControl_UpdateNotify request with updated policy information about the PDU Session as described in the PCF initiated SM Policy Association Modification procedure in clause 4.16.5.2.

If the request is not authorized, or the required QoS is not allowed, NEF responds to the AF in step 5 with a Result value indicating the failure cause.

5.	The NEF sends a Nnef_AFsessionWithQoS_Create response message (Transaction Reference ID, Result) to the AF. Result indicates whether the request is granted or not.

6.	The NEF shall send a Npcf_PolicyAuthorization_Subscribe message to the PCF to subscribe to notifications of Resource allocation status and may subscribe to other events described in clause 6.1.3.18 of TS 23.503 [20].

7.	When the event condition is met, e.g. that the establishment of the transmission resources corresponding to the QoS update succeeded or failed, the PCF sends Npcf_PolicyAuthorization_Notify message to the NEF notifying about the event.

8.	The NEF sends Nnef_AFsessionWithQoS_Notify message with the event reported by the PCF to the AF.

The AF may send Nnef_AFsessionWithQoS_Revoke request to NEF in order to revoke the AF request. The NEF authorizes the revoke request and triggers the Npcf_PolicyAuthorization_Delete and the Npcf_PolicyAuthorization_Unsubscribe operations for the AF request.

# * * * * Next change * * * *

5.2.6.8.2	Nnef_ChargeableParty_Create service operation

Service operation name: Nnef_ChargeableParty Create

Description: The consumer requests to become the chargeable party for a data session for a UE.

Inputs, Required: AF Identifier, UE address (i.e. IP address or MAC address), Flow description(s) or External Application Identifier, Sponsor Information, Sponsoring Status.

Inputs, Optional: Time period, traffic volume, Background Data Transfer Reference ID, .

Outputs, Required: Transaction Reference ID, result.

Output (optional): None.

# * * * * Next change * * * *

5.2.6.9.2	Nnef_AFsessionWithQoS_Create service operation

Service operation name: Nnef_AFsessionWithQoS Create

Description: The consumer requests the network to provide a specific QoS for an AF session.

Inputs, Required: AF Identifier, UE address (i.e. IP address or MAC address), Flow description(s) or External Application Identifier, QoS Reference.

Inputs, Optional: time period, traffic volume, Alternative Service Requirements (containing one or more QoS reference parameters in a prioritized order), QoS parameter(s) to be measured, Reporting frequency, Target of reporting as described in clause 6.1.3.21 of TS 23.503 [20], Requested 5GS delay, Requested GFBR, Requested MFBR, Flow Direction, Burst Size, Burst Arrival Time at UE (uplink) or UPF (downlink), Periodicity, Time domain.

Outputs, Required: Transaction Reference ID, result.

Output (optional): None.

# * * * * End of changes * * * *