3GPP TSG-WG SA2 Meeting #144E e-meeting 	S2-2102219

Elbonia, April 12 – April 16, 2021

# Title:	[DRAFT] LS Reply on Server Domain Name Usage for Application Traffic Detection

# Response to:	LS (S2-2102102/S4- 210311) on LS on Server Domain Name Usage for Application Traffic Detection from SA4

# Release:	Release 16

# Work Item:	5GMS3

Source:	[Huawei to be] SA2

To:	SA4

Cc:	CT3

Contact Person:

Name:	Haiyang, Sun

Tel. Number:

E-mail Address:	sunhaiyang3 AT huawei DOT com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	23.502 CR 2490/2491, 23.682 CR 0475

1. Overall Description:

SA2 thanks SA4 for the questions. SA2 would like to answer them as below:

1:	When invoking the Nnef_AFSessionWithQoS method, is it possible to use the application identifier (as a reference to a set of PFDs) instead of the flow description?

SA2 Answer:

In this release, Rel-16, as described in clause 5.2.6.8 and 5.2.6.9 of TS 23.502, application identifier is not an alternative. The NEF could map the AF Identifier to Application Identifier during the Nnef_AFSessionWithQoS/ Nnef_ChargeableParty service operation, if this is necessary.

For Rel-17, SA2 has agreed to introduce the support for AF to provide (external) Application Identifier to NEF/SCEF for the 2 service operations and the attached CRs, 23.502 CR 2490 and 23.682 CR 0475, are agreed.

3:	When invoking a particular policy using Nnef_AFSessionWithQoS, is it possible to provide the DNN, the S-NSSAI, and/or a reference to the server domain name for the purposes of traffic identification?

NOTE:	The same questions apply to Nnef_ChargeableThird Party API.

SA2 Answer:

In this release Rel-16, as described in clause 5.2.6.8 and 5.2.6.9 of TS 23.502, the DNN, the S-NSSAI, and/or a reference to the server domain name are not used as traffic identification, because a concrete description of the traffic (that is subject to the special QoS/charging treatment) is required to differentiate the flows within the PDU Session, which has a certain DNN/S-NSSAI. (For example, DNN/S-NSSAI cannot differentiate the flows within a PDU Session.) And this concrete description can only be achieved by using the Flow description(s) parameter (or the application identifier mapped from the AF identifier as described in the first answer).

In addition to the answers above, SA2 considers it’s beneficial for AF to provide DNN/S-NSSAI (e.g. to facilitate BSF/PCF discovery for the NEF in case of MAC address is used for the PDU session, or to deal with cases where 5GMS AF and 5GMS AS are in different Data Networks or different Network Slices) when it’s available for the two  service operations in discussion, thus 23.502 CR 2491 is agreed.

2. Actions:

To SA4 group.

ACTION: 	SA2 asks SA4 group to

Take the above information into account and provide feedback, if needed.

3. Date of Next TSG SA WG2 Meetings:

TSG-SA2 Meeting #145E		May 17 – 28, 2021			Elbonia