3GPP TSG-WG SA2 Meeting #144E e-meeting	S2-2102228

# Elbonia, April 12 – April 16, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0265 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Delete NSI ID via N7 interface |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_Ph1, TEI16 |  |  |  |  |  | Date: |  |  | 2021-03-24 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | NSI ID was introduced in N7 interface for PCF to retrieve slice load information from NWDAF. But as discussed in SA2#143, SMF cannot be aware of the NSI ID of a PDU Session. Besides, Stage 3 cannot supported it as mentioned in C3-210310. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Delete NSI ID in N7 interface. Clarify NSI ID is only applicable when the consumer is NSSF but not PCF. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The description is not algined with stage 3. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.1, 6.3.2A, 6.3.3A |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

## 6.3	Slice load level related network data analytics

### 6.3.1	General

The NWDAF provides slice load level information to a consumer NF on a Network Slice level or a Network Slice instance level or both. The NWDAF is not required to be aware of the current subscribers using the slice. The NWDAF notifies slice specific network status analytics information to the consumer NF that is subscribed to it. A consumer NF may collect directly slice specific network status analytics information from NWDAF. This information is not subscriber specific.

The NWDAF services as defined in the clause 7.2 and clause 7.3 are used to expose slice load level analytics from the NWDAF to the consumer NF (e.g. PCF, NSSF or AMF).

The consumer of these analytics shall indicate in the request or subscription:

-	Analytics Id set to "Load level information";

-	The following Analytics Filter Information:

-	S-NSSAI and NSI ID;

NOTE:	The use of NSI ID in the network is optional and depends on the deployment choices of the operator. If used, the NSI ID is associated with S-NSSAI. If applicable, it can only be provided by the NSSF or AMF.

-	an optional Area of Interest;

-	an optional list of NF types;

-	optionally, the list of analytics subsets that are requested among those specified in clause 6.3.4.

-	optionally, Load Level Threshold value; and

-	optionally, "maximum number of objects" indicating the maximum number of Network Slice instances expected in output, when the Analytics Filter Information does not indicate an NSI ID.

# * * * * Next change * * * *

### 6.3.2A	Input data

The detailed information collected by the NWDAF is listed in Table 6.3.2A-1.

Table 6.3.2A-1: Input data for slice load analytics

| Information | Source | Description |
| --- | --- | --- |
| Timestamps | 5GC NF | A time stamp associated with the collected information. |
| UE registered in a Network Slice/Network Slice instance | AMF, OAM | Current number of UEs registered in a NW slice or NW slice instance. |
| PDU session established on a Network Slice/Network Slice instance | SMF, OAM | Current number of established PDU sessions in a NW slice |
| Load of NFs associated to Network Slice instance | OAM, NRF | Resource utilization information of a Network Slice instance obtained from its constituent NF instances. NF instance load input data collection is described in clause 6.5, Table 6.5.2-1. |

# * * * * Next Change * * * *

### 6.3.3A	Output analytics

The NWDAF services as defined in the clause 7.2 and 7.3 are used to expose the following analytics:

-	Network Slice load statistics information as defined in Table 6.3.3A-1.

-	Network Slice instance load statistics information as defined in Table 6.3.3A-2.

-	Network Slice load predictions information as defined in Table 6.3.3A-3.

-	Network Slice instance load predictions information as defined in Table 6.3.3A-4.

Table 6.3.3A-1: Network Slice instance load statistics

| Information | Description |
| --- | --- |
| S-NSSAI | Identification of the Network Slice. |
| Network Slice instances (1,…,max) | List of Network Slice instance(s) within the S-NSSAI. |
| > NSI ID | Identification of the Network Slice instance. |
| > Number of UE Registrations (NOTE 1) | Number of UE registrations of the Network Slice instance (average, variance). |
| > Number of PDU Sessions establishment (NOTE 1) | Number of PDU Session establishments of the Network Slice instance (average, variance). |
| > Resource usage (NOTE 1) | The usage of assigned virtual resources currently in use for the NF instances (mean usage of virtual CPU, memory, disk) as defined in TS 28.552 [8] clause 5.7, belonging to a particular Network Slice instance. |
| > Resource usage threshold crossings (optional) (NOTE 1) | Number of resource usage threshold crossings on the Network Slice instance provided if threshold is provided by the consumer as Analytics Filter. |
| > Resource usage threshold crossings time period (1,…,max) (optional)(NOTE 1) | Resource usage threshold crossing vector including time elapsed between each threshold crossing on the Network Slice instance provided that a threshold value is provided by the consumer as Analytics Filter. |
| NOTE 1: Analytics subset that can be used in "list of analytics subsets that are requested". |  |

Editor's note:	Whether number of PDU Sessions per Network Slice instance can be provided as part of analytics without SMF knowledge of NSI ID is FFS.

Table 6.3.3A-2: Network Slice load statistics

| Information | Description |
| --- | --- |
| S-NSSAI | Identification of the Network Slice. |
| > Number of UE Registrations (NOTE 1) | Number of UE registrations at the Network Slice (average, variance). |
| > Number of PDU sessions establishments (NOTE 1) | Number of PDU Session establishments at the Network Slice (average, variance). |
| NOTE 1: Analytics subset that can be used in "list of analytics subsets that are requested". |  |

Table 6.3.3A-3: Network Slice instance load predictions

| Information | Description |
| --- | --- |
| S-NSSAI | Identification of the Network Slice. |
| Network Slice instances (1,…,max) | List of Network Slice instance(s) within the S-NSSAI. |
| > NSI ID | Identification of the Network Slice instance. |
| > Number of UE Registrations (NOTE 1) | Number of predicted UE registrations at the Network Slice instance. |
| > Number of PDU Sessions establishment (NOTE 1) | Number of predicted PDU Session establishments of the Network Slice instance. |
| > Resource usage (NOTE 1) | The predicted usage of assigned virtual resources for the NF instances (mean usage of virtual CPU, memory, disk) as defined in TS 28.552 [8] clause 5.7, belonging to a particular Network Slice instance. |
| > Resource usage threshold crossings (optional) (NOTE 1) | Number of predicted resource usage threshold crossings at the Network Slice instance provided that a threshold value is provided by the consumer as Analytics Filter. |
| > Resource usage threshold crossings time period (optional) (1,…,max) (NOTE 1) | Predicted Resource usage threshold crossing vector including predicted time elapsed between each threshold crossing on the Network Slice instance provided that a threshold value is provided by the consumer as Analytics Filter. |
| > Probability assertion | Confidence of this prediction. |
| NOTE 1: Analytics subset that can be used in "list of analytics subsets that are requested". |  |

Editor's note:	Whether number of PDU Sessions per Network Slice instance can be provided as part of analytics without SMF knowledge of NSI ID is FFS.

Table 6.3.3A-4: Network Slice load predictions

| Information | Description |
| --- | --- |
| S-NSSAI | Identification of the Network Slice. |
| > Number of UE Registrations (NOTE 1) | Predicted Number of UE registrations at the Network Slice. |
| > Number of PDU sessions establishments (NOTE 1) | Predicted Number of PDU Session establishments at the Network Slice. |
| > Probability assertion | Confidence of this prediction. |
| NOTE 1: Analytics subset that can be used in "list of analytics subsets that are requested". |  |

NOTE:	If no NSI ID is provided as analytics filter, slice load level related output analytics are provided according to Tables 6.3.3A-2 and 6.3.3A-4. Otherwise slice load level related output analytics are provided according to Tables 6.3.3A-1 and 6.3.3A-3.

# * * * * End of changes * * * *