3GPP TSG-WG SA2 Meeting #144E e-meeting 	S2-2102306r02

Elbonia, April 12 – April 16, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0272 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Removal of Editorial Note related to Namf/Nsmf_EventExposure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Spirent, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_Ph2 |  |  |  |  |  | Date: |  |  | 2021-04-05 |
| Category: | D |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Replacement of Editorial Note with a clarifying Note. Also correction of tables reference |  |  |  |  |  |  |  |  |
| Summary of change: |  | Removal of Editorial Note referencing a CR (S2-2101349) which was approved in #143E. The addition of a clarification note referncing Namf/NsmfEventExposure support of transaction information. Also a correction of tables reference incorrectly captured in the first version of subclause 6.10.2. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Editorial Note will not be addressed as required. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.10.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | x |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  | x |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### 6.10.2	Input Data

The NWDAF shall be able to collect UE dispersion information from NF(s) and AFs. The information collected by the NWDAF is network data from 5GC NFs and service data from AFs:

-	Data related to UE dispersion and bound by location collected from 5GC are defined in the tables 6.10.2-1 and 6.10.2-2.

Table 6.10.2-1: Location based UE transaction dispersion information collected from serving AMF

| Information | Source | Description |
| --- | --- | --- |
| UE ID | AMF | SUPI. |
| Type Allocation Code | AMF | Terminal model and vendor information of the UE. |
| UE locations (1..max) | AMF | UE locations. |
| > UE location | AMF | Area of Interest (TA or cells that the UE had entered). |
| > Timestamp | AMF | Time stamp when the AMF detected that the UE had entered this location. |
| > Transactions | AMF | Either all transactions for this UE in this location or, optionally, if subscription with periodic notification is requested, the amount of UE transactions exchanged at the location in the notified time period. |

Table 6.10.2-2: UE transaction dispersion information collected from serving SMF

| Information | Source | Description |
| --- | --- | --- |
| UE IP address | SMF | UE IP address. |
| UE ID | SMF | SUPI. |
| > Timestamp | SMF | Time stamp of the collected information. |
| > Transactions | SMF | Either all transaction for this UE or, optionally, if subscription with periodic notification is requested, the amount of UE transactions exchanged in the notified time period. |

Data volume can be collected from the AF per UE across all the UE applications provided by a particular AF or per UE per application when application IDs are requested.

Data related to UE data dispersion, bound by a location or slice, collected from the AF and UPF are defined in tables 6.10.2-3 through 6.10.2-6.

-	In table 6.10.2-3, the data volume is collected per UE from the AF. The collected UE information is applicable across all the applications used by the UE during the period.

Table 6.10.2-3: UE Data dispersion information collected from the AF

| Information | Source | Description |
| --- | --- | --- |
| UE ID | AF | External UE ID (i.e. GPSI). |
| Timestamp | AF | Time stamp of the collected information. |
| > Data Volume UL/DL | AF | Sum of UE data volume exchanged at the AF during the period of observation. |

-	In table 6.10.2-4, the AF reports volume per UE per application in relation to the start and stop of the application as indicated by the application duration.

Table 6.10.2-4: UE data dispersion collected from the AF

| Information | Source | Description |
| --- | --- | --- |
| UE IP address | AF | UE IP address. |
| IP 5-tuple | AF | IP address 5-tuple. |
| Timestamp | AF | Timestamp of the collected information. |
| Application ID | AF | Identifier of the application at the AF. |
| Location of Application | AF/NEF | Location of application represented by a list of DNAI(s). The NEF may map the AF-Service identifier information to a list of DNAI(s) when the DNAI(s) being used by the application are statically defined. |
| Data Volume UL/DL | AF | Sum of UE data volume exchanged per application during the period. |
| Application duration | AF | Duration for the application (e.g. Voice talk time). |
| NOTE 1: Application ID and IP 5-tuple are mutually exclusive. NOTE 2: Multiple outputs are provided by the AF when multiple applications are running at the AF for the same UE and time period. |  |  |

Data volume can also be collected from the UPF per UE IP address across all applications or per UE for specific application(s).

-	In table 6.10.2-5, the data volume is collected per UE from the UPF. The collected UE information is applicable across all applications used by the UE between start and stop of the PDU session. The UPF reports volume per UE IP address across all applications.

Table 6.10.2-5: UE data dispersion collected from serving UPF

| Information | Source | Description |
| --- | --- | --- |
| UE IP address | UPF | UE IP address. |
| Timestamp | UPF | Time stamp of the collected information. |
| Data Volume UL/DL | UPF | Sum of UE data volume exchanged per UE across all applications. |
| NOTE: The Data volume can be reported either as total volume of the PDU session or periodically. It refers to the Data volume exchanged between the start and stop of the PDU session. When reported periodically, the period can be specified in the requested analytic target period or configured as a default value in the UPF. |  |  |

-	In table 6.10.2-6, the UPF reports volume per UE for specific application(s) in relation to the start and stop of the application as indicated by the application duration.

There are two modes of data collection:

-	Non periodical: A mode where the data volume is requested and consequently provided for the total volume of a PDU session.

-	Periodical: A mode where data volume is provided periodically between the start and stop of a PDU session. The period can be specified in the requested analytic target period or configured as a default value in the UPF.

For both modes of data collection, if there are multiple application sessions for the UE PDU session, the NWDAF aggregates the volume across all applications to obtain per UE information.

Table 6.10.2-6: UE data dispersion collected from serving UPF

| Information | Source | Description |
| --- | --- | --- |
| UE IP address | UPF | UE IP address. |
| Timestamp | UPF | A timestamp of the collected information. |
| Application ID | UPF | Identify the application at the UPF |
| IP 5-tuple | UPF | IP 5-tuple. |
| Location of Application | UPF | List of Internet applications represented by DNAI(s). |
| Data Volume UL/DL | UPF | Sum of UE data volume exchanged per application during the period. |
| Application duration | UPF | Duration for the application (e.g. Voice talk time). |
| NOTE 1: Application ID and IP 5-tuple are mutually exclusive. NOTE 2: Multiple outputs are provided by the UPF when multiple applications are running at the UPF for the same UE and time period. NOTE 3: The Data volume can be reported either as total volume of the PDU session or periodically. It refers to the Data volume exchanged between the start and stop of the PDU session. When reported periodically, the period can be specified in the requested analytic target period or configured as a default value in the UPF. |  |  |

The task of the NWDAF is to calculate the volume of data and amount of transactions dispersed by the UE at each visited location during a period of interest. The entry time and exit time to a location is provided by the AMF.

Collecting volume per UE and per specific application is optional and such information can be provided by the UPF or AF.

The Correlation Information as defined in table 6.2.4-1 is used to correlate the input data from AMF and SMF, the input data from SMF and UPF, and the input data from UPF and AF.

-	Slice information related to UE transaction dispersion from NF(s) are defined in tables 6.10.2-7 and 6.10.2-8.

-	The Slice data volume collection is the same as listed in tables 6.10.2-3 through 6.10.2-6.

Table 6.10.2-7: Slice based UE transaction dispersion information collected from serving AMF

| Information | Source | Description |
| --- | --- | --- |
| UE ID | AMF | SUPI. |
| Type Allocation Code | AMF | Terminal model and vendor information of the UE. |
| Slice (1..max) | AMF | UE assigned slice. |
| >S-NSSAI | AMF | Identifier of the slice. |
| >Timestamp | AMF | Time stamp when the slice was assigned. |
| >Transactions | AMF | Either all transactions for this UE or, optionally, if subscription with periodic notification is requested, an amount of UE transactions exchanged at the slice in the notified time period. |

Table 6.10.2-8: Slice based UE transaction dispersion information collected from serving SMF

| Information | Source | Description |
| --- | --- | --- |
| UE IP | SMF | UE IP address. |
| UE ID | SMF | SUPI. |
| Slice (1..max) | SMF | UE assigned slice. |
| >S-NSSAI | SMF | Identifier of the slice. |
| >Timestamp | SMF | Time stamp associated with the collected information. |
| >Application ID | SMF | Identifier of the application for the UE in the slice. |
| >Transactions | SMF | Either all transactions for this UE or, optionally, if subscription with periodic notification is requested, an amount of UE transactions exchanged at the slice in the notified time period. |

The task of the NWDAF is to calculate the volume of data and amount of transactions dispersed by the UE at each slice during a period of interest. A time stamp of a slice assignment is provided by the AMF.

The Correlation Information as defined in table 6.2.4-1 is used to correlate the input data from AMF and SMF, the input data from SMF and UPF, and the input data from UPF and AF.

Data collection may be provided on samples (e.g. spatial subsets of UEs or UE group, temporal subsets of UE location information).

NOTE 1:	How the data from UPF is retrieved is not specified in this release.

NOTE 2: The list of monitored transactions (MM and SM messages) is the subset of the messages listed in clauses 8.2 and 8.3 of TS 24.501 [23].

NOTE 3: The Namf_EventExposure and Nsmf_EventExposure services in clauses 5.2.2.3 and 5.2.8.3, TS 23.502 [3] provide the required transactions information.