3GPP TSG-WG SA2 Meeting #144E e-meeting 	S2-2102538r07

April 12 – 16, 2021, Electronic

Source:	China Mobile, vivo,Xiaomi

Title:	Update for ECS address configuration information

Document for:	Approval

Agenda Item: 	8.3

Work Item / Release:	eEDGE_5GC/Rel-17

Abstract of the contribution: This contribution proposes to include how to decide the ECS address configuration information in TS 23.548.

# Discussion

There maybe multiple ECS(s) deployed in data network. Currently it is not clear how to decide which ECS address configuration information should be sent to UE and also the criteria is not clear. For currently 6.5, the ECS address configuration information may be locally configured in SMF or provided by AF through NEF, which one has the high priority is not clear.

In this paper, we propose the ECS address configuration information may additionally include ECS service area, ECS selection priority and ECS property information. The SMF may decide the specific ECS based on the UE location, ECS service area, and/or the selection priority. For the ECS service area, it is clear that only when the UE is located within the ECS service area, it is reasonable for 5GC sending the ECS address configuration information through NAS signal

The ECS selection priority maybe provided by AF or configured in SMF. The selection priority can help 5GC decide which ECS servers have the high priority to be sent to UE based on local operator policy. And the UE can also use the ECS selection priority to select the proper ECS when the two ECS(s) has almost the same ECS property information.

The ECS property information includes the ECS capability e.g. which applications on the EES can be provided.

# Proposal

It is proposed to add the following update to TS 23.548.

************************ 1st Change, all new text************************

### 6.5.2	ECS Address Provisioning

Editor's note:	This clause describes here the procedure agreed in conclusions in 9.1.3.

If the UE hosts an EEC and supports transferring the ECS address received from the 5GC to the EEC, the UE indicates in the PCO at PDU Session establishment or modification that it supports the ability to receive ECS address(es) via NAS and to transfer the ECS Address(es) to the EEC(s).

The ECS Address Configuration Information consists of one or more FQDN(s) and/or IP Address(es) of Edge Configuration Server(s), ECS Provider ID and ECS ID. As described in clause 4.3.2 in TS 23.502 [3], if the UE supports the ability to receive ECS Address Configuration Information via NAS and to transfer the ECS Address(es) to the EEC(s), the UE may receive ECS Address Configuration Information from the SMF via PCO during PDU Session Establishment and/or during PDU Session Modification procedures. The SMF may derive the ECS Address Configuration Information based on local configuration, the UE's location, ECS Provider ID and/or UE subscription information from UDM. The SMF may decide to send updated ECS Address Configuration Information to the UE based on locally configured policy, updated UE subscription information, or a change of UE location. The PDU Session Modification procedure is used to send updated ECS Address Configuration Information to the UE as described in clause 4.3.3 in TS 23.502 [3].

NOTE:	In home routed sessions, the ECS Address Configuration Information comes from the H-SMF.

#### 6.5.2.1	ECS Address Provisioning by a 3rd Party AF

As described in TS 23.558 [5], the Edge Configuration Server can be deployed in the MNO domain or can be deployed in a 3rd party domain by a service provider. If the ECS is deployed in a 3rd party domain by a service provider, a 3rd party AF can use Nnef_ParameterProvision to provide, update, or delete ECS Address Configuration Information for a UE (See TS 23.502 [3], clause 4.15.6.2).

When the AF uses Nnef_ParameterProvision to send a new ECS Address Configuration Information to the UDM for a UE (e.g. because on Application layer activity, change of UE location, etc.), the UDM may notify the SMF of the updated ECS Address Configuration Information and the new ECS Address Configuration Information will be sent to the UE in a PDU Session Modification procedure.

Editor's note:	It is FFS whether the 3rd party AF would know the GPSI; the possibility to configure the ECS address for all UE in one operation should be considered.

Editor's note:	Potential signalling overload is FFS When the AF uses Nnef_ParameterProvision to send a new ECS Address Information to the UDM for many UE.

********************************* End Change ****************************