3GPP TSG-WG SA2 Meeting #144E e-meeting 	S2-2102576r02

Elbonia, 12 - 16 April, 2021	(revision of S2-210xxxx)

Source:	Huawei, HiSilicon

Title:	Mobility Procedures for 5MBS

Document for:	Approval

Agenda Item:	8.9

Work Item / Release:	5MBS / Rel-17

Abstract: This document adds Mobility Procedure for 5MBS to the new TS. This include the mobility among the homogenous and non-homogenous 5MBS capability NG-RAN node.

- Background and Introduction

This contribution proposes the related procedures as per the conclusion in clause 8.7 of TR 23.757.

- Proposal

It is proposed to capture the following changes vs. TS23.247.

# * * * * First change* * * *

## 7.1.5	Mobility Procedures for 5MBS

## 7.1.5.1 General

UE may move from one NG-RAN node to another NG-RAN node after UE has joined the MB Session. Depending on whether the NG-RAN node support 5G MBS, the following scenarios are possible:

- Both source and target NG-RAN support 5G MBS.

- Source NG-RAN supports 5G MBS, target NG-RAN does not support 5G MBS.

- Source NG-RAN does not support 5G MBS, target NG-RAN supports 5G MBS.

- Both source and target NG-RAN don’t support 5G MBS.

When the UE moves from a NG-RAN node that supports 5G MBS to a NG-RAN node that does not support 5G MBS, during the handover procedure the delivery method is switched from 5GC Shared MBS traffic delivery method to 5GC Individual MBS traffic delivery method.

When the UE moves from a NG-RAN node that does not support 5G MBS to a NG-RAN node that supports 5G MBS, during the handover procedure the delivery method is kept as the 5GC Individual MBS traffic delivery. After the handover procedure, the delivery method is switched from 5GC Individual MBS traffic delivery method to 5GC Shared MBS traffic delivery method.

## 7.1.5.2 Xn based handover with MBS Session

Editor’s Note: Details for Xn based handover should be aligned with RAN WG.

This clause describes the Xn based handover with the MBS Session established at the source NG-RAN side.

Figure 7.15.2-1: Xn based handover with MBS Session

Comparing with the existing Xn based handover procedure (see clause 4.9.1.2 of TS 23.502 [X]), the following enhancements apply:

0.	Handover Preparation phase.

The source NG RAN notify the MBS Session information (including the MBS Session ID and multicast QoS flow information) and the associated mapping PDU session information to the target NG-RAN. The associated mapping PDU Session information is the information of the mapped unicast QoS flow(s) correlated with the multicast QoS flow(s) of the MBS Session.

Based on the received the MBS Session information and its associated mapping PDU Session information:

- If the target NG-RAN does not support 5G MBS, normal Xn based handover preparation is performed.

- If the target NG-RAN supports 5G MBS, the target NG-RAN uses the MBS session information to allocate resources to deliver MBS service. If the indicated MBS Session has not been established in target NG-RAN,the shared tunnel is established.

1.	Target NG-RAN to AMF: the target NG-RAN sends N2 Path Switch Request to AMF.

- If the target NG-RAN has accepted the multicast QoS flow, when the indicated MBS Session has not been established in target NG-RAN, the N2 SM message included in the N2 Path Switch Request contains the MBS Session ID and the accepted multicast QoS flow(s) information.

- If the target NG-RAN does not support 5G MBS and if the target NG-RAN has accepted the mapped unicast QoS flow(s), the N2 SM message includes the accepted QoS flow information of the mapped unicast QoS flow(s) correlated with the MBS Session..

Based on the received N2 SM message, the SMF can differentiate two cases:

Case A) The target NG-RAN supports 5G MBS. Step 3 applies and steps 4~8 are skipped.

- SMF to UPF: The SMF invokes N4 Session Modification procedure with the UPF (PSA) only for unicast PDU Session. The SMF instructs the UPF to send the end market packet towards the source NG-RAN.

Case B) The target NG-RAN does not support 5G MBS and the UPF is not yet configured to forward multicast data via unicast, steps 4 to 8 apply.

- SMF to UPF (PSA): The SMF invokes N4 Session Modification procedure with UPF (PSA), the SMF instructs the UPF (PSA) to forward the multicast data received from the MB-UPF via the mapped unicast QoS flow(s) of the PDU Session. The SMF provides the mapping information between the multicast QFI and the corresponding unicast QFI of the multicast QFI in the PDU Session to the UPF (PSA), the UPF (PSA) forwards the multicast data via the PDU session based on the mapping information. If the delivery tunnel for the MBS session from MB-SMF to UPF is not established yet, the SMF instructs the UPF (PSA) to allocate a tunnel endpoint for the reception of multicast data from the MB-UPF.

If delivery of the multicast data from MB-UPF to UPF needs to be configured, steps 5 to 8 apply.

- SMF to MB-SMF: The SMF invokes a Nmbsmf_MBSSession_Update (MBS session ID, SMF ID, DL tunnel info) request service operation to MB-SMF to establish the shared tunnel between the UPF(PSA) and MB-UPF.

- MB-SMF to MB-UPF: The MB-SMF configures the MB-UPF with the received DL tunnel Info and instructs the MB-UPF to forward data of the MBS session to the UPF (PSA) via the tunnel. The MB-UPF starts to forward data of the MBS session to the UPF (PSA).

- MB-SMF to SMF: The MB-SMF responds to SMF through Nmbsmf_MBSSession_Update response. If multicast data are transported via multicast, the MB-SMF provides endpoint information (e.g., the Common-TEID) including the transport multicast address.

- SMF to UPF (PSA): The SMF invokes an N4 Session Modification procedure with the UPF (PSA). If multicast data are transported via multicast, the SMF provides endpoint information including the transport multicast address to the UPF (PSA) and the UPF (PSA) sends IGMP Join in order to receive data from the MB-UPF.

- UPF (PSA) to Source NG-RAN: The Source NG-RAN forwards the end marker to target NG-RAN. The target NG-RAN starts to send the buffered packets for the PDU Session including buffered MBS data if any to the UE.

During handover preparation phase, if the source NG-RAN knows no other served UEs participate in the MBS session, step 11 applies, which release the shared tunnel.

## 7.1.5.3 N2 based handover with MBS Session

Editor’s Note: Details for N2 based handover should be aligned with RAN WG.

Editor’s Note: Procedure for multicast delivery resource establishment is FFS.

This clause describes the N2 based handover with the MBS Session established at the source NG-RAN side.

Figure 7.15.3-1: N2 based handover with MBS Session

Comparing with the existing N2 based handover procedure (see clause 4.9.1.3 of TS 23.502 [8]), the following enhancements apply:

- Source NG-RAN to S-AMF: Handover Required (RAN container (associated mapping PDU session information, [MBS session information])).

The RAN container contains both the MBS session information and the associated mapping PDU Session information. The associated mapping PDU Session information is the information of the mapped unicast QoS flows correlated with the multicast QoS flow of the MBS Session.

- SMF to T-AMF: The SMF includes both MBS Session information (including MBS session ID and multicast QoS flow information) and the associated mapped unicast QoS flows information to the target NG-RAN.

An indication of whether forwarding is possible is also included in the N2 SM message.

- T-AMF to Target NG-RAN: The Target NG-RAN prepares the radio resource based on the received information.

- If the target NG-RAN does not support 5G MBS, the MBS Session information is not used. The target NG-RAN uses the associated mapping PDU Session information to allocate resource to deliver MBS data. The MBS data are transmitted as one of the QoS flows within the unicast PDU Session.

- If the target NG-RAN supports 5G MBS, the target NG-RAN use the MBS Session information allocate resource to deliver the MBS data. The mapped unicast QoS flow information in the unicast PDU Session information is not used.

If Target NG-RAN support 5G MBS and the MBS delivery for the indicated MBS Session has not been established towards target NG-RAN, the target NG-RAN allocates the shared downlink tunnel information for receiving the MBS data from 5GC and steps 6 to 10 apply:

- Target NG-RAN to AMF: Target NG-RAN node selects the AMF to reach MB-SMF and signals a multicast session distribution request towards AMF via the N2 Message (MBS Session ID). If the RAN node is configured to use a unicast transport for multicast distribution sessions, it allocates a downlink tunnel ID (an IP address and a GTP-U TEID) for the reception of the multicast distribution session and indicates the downlink tunnel information in the request.

- AMF to MB-SMF: AMF invokes the Nmbsmf_MBSSession_Create Request (MBS session ID, [DL tunnel info]) Request towards the MB-SMF to establish the multicast distribution towards AMF.

- MB-SMF to MB-UPF: MB-SMF invokes the N4 Session Modification procedure with MB-UPF. For unicast transport of the multicast distribution session, MB-SMF configures MB-UPF to transmit the multicast distribution session towards target NG-RAN node (using the received IP address and a GTP-U TEID).

- MB-SMF to AMF: MB-SMF responds to AMF through the Nmbsmf_MBSSession_Create Response. For multicast transport of the multicast distribution, it indicates in the downlink tunnel information and the multicast address for the multicast session.

- AMF to Target NG-RAN: AMF notify multicast session distribution response to Target NG-RAN node via the N2 Message.

- Target NG-RAN to T-AMF: The target NG-RAN sends Handover Request Ack to T-AMF.

- If the target NG-RAN support MBS, the N2 SM message included in the Handover Request Ack includes the MBS Session ID and the accepted multicast QoS flow(s).

- If the target NG-RAN does not support 5G MBS, the N2 SM message included in the Handover Request Ack includes the QoS flow info of the mapped unicast QoS flow(s) correlated with the multicast QoS flow.

- SMF to UPF: If the target NG-RAN does not support MBS, and indirect forwarding is needed, the forwarding tunnel for unicast PDU Session is used for the MBS data. The indirect forwarding tunnel is established as in existing N2 based handover procedure in TS 23.502 [8].

- Source NG-RAN to Target NG-RAN: The source NG-RAN forwards data to the target NG-RAN either directly or indirectly via the UPF.

- T-AMF to SMF: The AMF invokes Nsmf_PDUSession_UpdateSMContext request towards SMF, the message includes the received N2 SM message.

Based on the received N2 SM message, the SMF can differentiate two cases:

Case A) The Target NG-RAN supports 5G MBS. Step 22 applies and steps 23~29 are skipped.

- SMF to UPF (PSA): The SMF invokes N4 Session Modification procedure with the UPF (PSA) only for unicast PDU Session. The SMF instructs the UPF to send the end marker packet towards the source NG-RAN and to send subsequent packets towards the target NG-RAN within the unicast PDU Session.

Case B) The target NG-RAN does not support 5G MBS. If the UPF (PSA) is not yet configured to forward multicast data via unicast, steps 23 to 29 apply.

- SMF to UPF: The SMF may invokes N4 Session Modification procedure with UPF (PSA), the SMF instructs the UPF (PSA) to forward the multicast data received from the MB-UPF via the mapped unicast QoS flow(s) of the PDU Session within the unicast PDU(i.e., 5GC Individual MBS traffic delivery method will be used). The SMF provides the mapping information between the multicast QFI and the corresponding mapped unicast QFI to the UPF (PSA), the UPF (PSA) forwards the multicast data via the PDU session based on the mapping information. If the delivery tunnel for the MBS session from MB-SMF to UPF is not established yet, the SMF instructs the UPF (PSA) to allocate a tunnel endpoint for the reception of multicast data from the MB-UPF.

If delivery of the multicast data from MB-UPF to UPF needs to be configured, steps 24 to 27 apply.

- SMF to MB-SMF: The SMF invokes a Nmbsmf_MBSSession_Update (MBS session ID, SMF ID, DL tunnel info) request service operation to MB-SMF to establish the shared tunnel between the UPF(PSA) and MB-UPF.

- MB-SMF to MB-UPF: The MB-SMF configures the MB-UPF with the received DL tunnel Info and instructs the MB-UPF to forward data of the MBS session to the UPF (PSA) via the tunnel. The MB-UPF starts to forward data of the MBS session to the UPF (PSA).

- MB-SMF to SMF: The MB-SMF responds to SMF through Nmbsmf_MBSSession_Update response. If multicast data are transported via multicast, the MB-SMF provides endpoint information (e.g., the Common-TEID) including the transport multicast address.

- SMF to UPF (PSA): The SMF invokes an N4 Session Modification procedure with the UPF (PSA). If multicast data are transported via multicast, the SMF provides endpoint information including the transport multicast address to the UPF (PSA) and the UPF (PSA) sends IGMP Join in order to receive data from the MB-UPF.

During handover preparation phase, if the source NG-RAN knows no other served UEs participate in the MBS session, steps 30 applies, which release the shared tunnel.

## 7.1.5.3 X2/N2 based handover without MBS Session

If the 5GC Individual MBS traffic delivery method is used at the source NG-RAN node for the MBS Session, the existing X2 /N2 based handover procedure defined in TS23.502[6] is used to support the UE move to Target NG-RAN node.

If the target NG-RAN supports 5G MBS, the MBS Traffic delivery method can be switched from 5GC Individual MBS traffic delivery method to 5GC Shared MBS traffic delivery method.

## 7.1.5.4 Lossless packet transferring

Editor’s Note: Details for how to achieve the lossless packets transfer between the source NG-RAN node and the target NG-RAN node should be aligned with 3GPP RAN WG.

# * * * * End of changes * * * *