3GPP TSG-SA WG2 Meeting #144E 	S2-2102578r02

Elbonia, 12 - 16 April, 2021

Source:	Huawei, HiSilicon

Title:	MBS procedures for inter System Mobility

Document for:	Agreement

Agenda Item: 	8.9

Work Item / Release:	5MBS/Rel-17

Abstract of the contribution: This paper add the MBS procedures for inter System Mobility for MBS session.

- Proposal

This contribution proposes procedures to the new TS for the inter System Mobility for MBS session.

It has agreed that if the MBS service is not provided via eMBMS, the 5MBS service is delivered to the UE via individual delivery mode when the UE moves to the EPS network. One issue needs to discuss is, after UE handover to the EPS, if the UE further moves to an area, in which the eMBMS is provided via EPS, how would the resource allocated for individual delivery of the service be removed.

There are 2 possible ways to remove resources for individual delivery of the service:

Alternative 1: The AS indicate the 5GC to remove the resources for individual delivery of the service.

According to TS 23.468, when UE moves to area in which the service is available via eMBMS, the UE will send a report to the AS. After receiving the report from the UE, the AS can indicate the 5GC to remove the resource for individual delivery.

In this alternative, the interface between AF and PCF and the interface between PCF and SMF need to be enhanced for such purpose, since for individual delivery mode establishment, the AF has not provide service information to allocate the resource. For example, for interface between AF and PCF, the AppSessionContextReqData or AppSessionContextUpdateData may need to add a new IE for delivery of the indication. For interface between PCF and SMF, the SmPolicyDecision need to be enhanced to add a new IE for delivery of the indication. Similar to TSN, the indication can be encapsulated into a container, and is transparently sent to SMF via PCF (and possibly NEF).

The disadvantage for this alternative is that it depends on the deployment of PCF.

Alternative 2: The UE indicate the 5GC to remove the resources for individual delivery of the service

After the UE moves to the area in which the service is available via eMBMS, the UE sends an indication to PGW-C+SMF if the service is delivered via a dedicated EPS bearer, e.g. the UE may send a leave indication and MBS session ID via PCO to SMF+PGW-C. The UE knows that it is receiving the service via 5G MBS session, since it has successfully joined the 5G MBS session.

This option does not depend on the PCF. It need enhance the NAS PCO to send an indication from UE to SMF+PGW-C.

Comparing the 2 options, it seems the Alternative 2 has less impacts, and it does not require the deployment of PCF. In addition, after UE moved to EPS, even if the UE does not move to eMBMS service area, it may want to leave the MBS service. So it should also be possible for UE to leave the MBS session when the UE camps at the EPS network. Hence, Alternative 2 is recommended.

C1: it is suggested to use the UE leave proposal to leave the MBS session in case of the eMBMS service is available.

**************************** 1st Change ***************************

7.2.2.3	MBS Session leave via EPS

If the UE has joined a MBS session, 5GC Individual MBS traffic delivery method may still be used when it moves to EPS. In this case, it shall be possible for UE leave the MBS session when it camps at the EPS network.

Figure 7.2.2.3-1 MBS Session leave via EPS

The UE leaves the MBS session by triggering UE requested bearer resource modification procedure as defined in clause 5.4.5 of TS 23.401 [x] with the following enhancement:

1.	When the UE camps at the EPS network and want to leave MBS Session, the UE initiates UE requested bearer resource modification with the following change:

In step 1 of UE requested bearer resource modification procedure in clause 5.4.5 of TS 23.401 [x], the UE includes a leave indication and MBS session ID in the PCO to indicate that it wants to stop receiving the indicated MBS service via the EPS bearer.

2. After the SMF+PGW-C receives the leave indication, it performs steps 3 to 6 as defined in MBS Session leave procedure in clause 7.2.2.2.

3. The SMF+PGW-C releases the bearer resources for the MBS Session as in steps 4 to 6 of clause 5.4.5 in TS 23.401[x].

The SMF+PGW-C removes the UE from the MBS Session context and removes the MBS Session ID from the associated UE SM context.

**************************** 2nd  Change ***************************

7.4	MBS procedures for inter System Mobility

7.4.2	Mobility between 5GS and EPS when the same Service is not provided via 5MBS and eMBMS

7.4.2.1 General

During 5GS to EPS connected mode handover or idle mode mobility procedure, if the MBS service is not provided via eMBMS in target location, interworking is supported at transport layer.

7.4.2.2 Mobility from 5GS to EPS using N26

Figure 7.4.2.2-1 Mobility from 5GS to EPS using N26

2.	UE joins the 5G MBS Session to receive the service as described in clause 7.2.1.

3.	During 5GS to EPS mobility procedure, the EPS bearer(s) context is sent to MME if the MBS service is provided via eMBMS in target location.

For connected mode mobility, the EPS bearer(s) for the service is established in EPS as part of the intersystem handover procedure as described in clause 4.11.1.2.1 of TS 23.502 [6] with the following enhancement:

In step 15: The SMF+PGW-C provides the mapping between the multicast QFI and DL tunnel info of the EPS bearer(s) associated to the MBS Service to UPF+PGW-U. The UPF+PGW-U maps packets received from the MB-UPF to the DL tunnel(s) of the EPS bearer(s) according to the mapping. If the N9 tunnel between UPF+PGW-U and MB-UPF for the MBS Service has not established yet, the SMF+PGW-C requests the UPF+PGW-U to allocate DL tunnel information of the N9 tunnel for the MBS Service.

For Idle mode mobility, the EPS bearer(s) for the service is established in EPS as part of the intersystem idle mode mobility procedure as described in clause 4.11.1.3.2 of TS 23.502 [6] with the following enhancement:

In step 11: The SMF+PGW-C provides the mapping between the multicast QFI and DL tunnel info of the EPS bearer(s) associated to the MBS Service to UPF+PGW-U. The UPF+PGW-U maps packets received from the MB-UPF to the DL tunnel(s) of the EPS bearer(s) according to the mapping. If the N9 tunnel between UPF+PGW-U and MB-UPF for the MBS Service has not established yet, the SMF+PGW-C requests the UPF+PGW-U to allocate DL tunnel information of the N9 tunnel for the MBS Service.

4.	The PGW-C+SMF sends Nsmf_MBSession_Update Request (DL Tunnel info for the MBS Service) to the MB-SMF.

5.	The MB-SMF sends the DL tunnel info for the MBS Service to the MB-UPF. The MB-UPF starts to send packets to the UPF+PGW-U, and the UPF+PGW-U starts to send the DL packets of the MBS Service to UE via corresponding EPS bearer(s).

6.	The MB-SMF sends Nsmf_ MBSession_Update Response to SMF+PGW-C.

7. 	For connected mode handover procedure, steps 16 to 21 of 5GS to EPS handover using N26 interface procedure in clause 4.11.1.2.1 of TS 23.502 [6] are performed.

For idle mode mobility procedure, steps 12 to 19 of 5GS to EPS Idle mode mobility using N26 interface procedure in clause 4.11.1.3.2 of TS 23.502 [6] are performed.

8.	The UE reports that eMBMS coverage of the MBS Service is available for the service to AS, and the AS indicates the UE to receive the MBS Service via MBMS bearer.

9.	The UE initiates MBS leave procedure via EPS as defined in clause 7.2.2.3.

7.4.2.3 Mobility from EPS to 5GS using N26

Figure 7.4.2.3-1 Mobility from EPS to 5GS using N26

1.

The UE receiving data over unicast bearer moves to 5G NG-RAN cell, the inter system handover or Idle mode mobility procedure from EPS to 5GS as described in TS 23.502 [6] is triggered. The service may be mapped to QoS flow of the PDU Session.

2.	If the UE wants to receive the service via 5G MBS Session, the UE joins the MBS Session as described in clause 7.2.1.

3.	If the MBS service is mapped to QoS flow of the PDU Session, after the UE joins the 5G MBS Session, it sends a report to the AS as described in TS 23.468 [5], and indicates that it moved to MBS service area.

4.	The AS stops sending the MBS Service data via unicast delivery. If the AS has sent MBS service info for unicast delivery to PCF in step 1, the AS requests the 5GC to delete the service info for the MBS service. Upon reception of the request from AS, the 5GC releases the QoS flow(s) for the MBS Service as described in TS 23.502 [6].

********************************* End Change ****************************