3GPP TSG-WG SA2 Meeting #144E e-meeting	S2-2102588r02

Elbonia, April 12 – 16, 2021	(revision of S2-200xxxx)

Source:	Huawei, HiSilicon, Samsung

Title:	Clarification on parameters for group member discovery

Document for:	Approval

Agenda Item:	8.8

Work Item / Release:	5G_ProSe / Rel-17

Abstract: This contribution proposes to clarify the parameters for group member discovery.

# 1. Introduction/Discussion

The parameters defined in clause 5.9.1 covers both ProSe Direct Discovery and Group Member Discovery. It is not clear which parameter is included in Group Member Discovery message by only reference to clause 5.9.1. Thus, this contribution proposes to clarify the parameters for group member discovery.

# 2. Text Proposal

It is proposed to capture the following changes vs. TS 23.304.

# * * * * First change * * * *

##### 6.3.2.2.2	Procedure for Group Member Discovery with Model A

Depicted in Figure 6.3.2.2.2-1 is the procedure for Group Member Discovery with Model A.

Figure 6.3.2.2.2-1: Group Member Discovery with Model A

1.	The announcing UE sends a Group Member Discovery Announcement message. The parameters contained in this message are as described in clause 5.9.1.x

NOTE:	A UE may send multiple Group Member Discovery Announcement messages (Model A) if the UE belongs to more than one discovery group.

##### 6.3.2.2.3	Procedure for Group Member Discovery with Model B

Depicted in Figure 6.3.2.2.3-1 is the procedure for Group Member Discovery with Model B.

Figure 6.3.2.2.3-1: Group Member Discovery with Model B

1.	The discoverer UE sends a Group Member Discovery Solicitation message. The parameters contained in this message are as described in clause 5.9.1.x.

2.	The discoveree UEs that match the values of the parameters contained in the solicitation message based on the Discovery Group ID, respond to the discoverer UE with a Group Member Discovery Response message. The parameters contained in this message are as described in clause 5.9.1.x.

# * * * * Second change * * * *

## 5.9	Identifiers

### 5.9.1	Identifiers for ProSe Direct Discovery

#### 5.9.1.1	ProSe Application ID

ProSe Application ID is defined in TS 23.303 [3].

#### 5.9.1.2	Destination Layer 2 ID

Destination Layer 2 ID is defined in TS 23.287 [2].

#### 5.9.1.3	Source Layer 2 ID

Source Layer 2 ID is defined in TS 23.287 [2].

#### 5.9.1.4	Discovery Group ID

Discovery Group ID is defined in TS 23.303 [3].

#### 5.9.1.5	ProSe Application Code

ProSe Application Code is defined in TS 23.303 [3].

#### 5.9.1.6	ProSe Restricted Code

ProSe Restricted Code is defined in TS 23.303 [3].

#### 5.9.1.7	ProSe Query Code

ProSe Query Code is defined in TS 23.303 [3].

#### 5.9.1.8	ProSe Response Code

ProSe Response Code is defined in TS 23.303 [3].

#### 5.9.1.9	User Info ID

User Info ID (including Announcer Info, Discoverer Info, Target Info, Discoveree Info) is defined in TS 23.303 [3].

#### 5.9.1.10	ProSe Discovery UE ID

ProSe Discovery UE ID is defined in TS 23.303 [3].

#### 5.9.1.11	Restricted ProSe Application User ID

Restricted ProSe Application User ID is defined in TS 23.303 [3].

#### 5.9.1.x	Identifiers for Group Member Discovery

The following parameters are used in the Group Member Discovery Announcement message (Model A):

-	Announcer Info as defined in clause 5.9.1.9.

-	Discovery Group ID as defined in clause 5.9.1.4.

The following parameters are used in the Group Member Discovery Solicitation message (Model B):

-	Discoverer Info as defined in clause 5.9.1.9.

-	Discovery Group ID as defined in clause 5.9.1.4.

-	Target Info as defined in clause 5.9.1.9

The following parameters are used in the Group Member Discovery Response message (Model B):

-	Discoveree Info as defined in clause 5.9.1.9.

-	Discovery Group ID as defined in clause 5.9.1.4.

# * * * * End of changes * * * *