SA WG2 Meeting #S2-144E	S2-2102785r02

12 April - 16 April 2021, Electronic, Elbonia	(revision of S2-210xxxx)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 2817 | rev |  | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Termnology on the TSC MIC and Bridge ID |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE, Nokia, Nokia Shanghai Bell, Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA WG2 |  |  |  |  |  |  |  |  |  |
| Work item code: | IIoT |  |  |  |  |  | Date: |  |  | 2021-04-02 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In the Rel-16, the term BMIC(Bridge management information container) and Bridge ID is used between NW-TT and TSN AF But in the Rel1-17, there is no Bridge concept for the IP PDU session. The more generic term is need for this case. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Replace the BMIC with NMIC, and Bridge ID with NW-TT instance ID |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The Bridge concept is not applied for IP PDU session. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.8.2.11.1, 5.8.2.11.9, 5.8.2.11.14, 5.27.1.4, 5.28.3.1, 5.28.3.2, K.1, K.2, K.2.1, K.2.2.1, K.2.2.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

5.8.2.11	Parameters for N4 session management

5.8.2.11.1	General

These parameters are used by SMF to control the functionality of the UPF as well as to inform SMF about events occurring at the UPF.

The N4 session management procedures defined in clause 4.4.1 of TS 23.502 [3] will use the relevant parameters in the same way for all N4 reference points: the N4 Session Establishment procedure as well as the N4 Session Modification procedure provide the control parameters to the UPF, the N4 Session Release procedure removes all control parameters related to an N4 session, and the N4 Session Level Reporting procedure informs the SMF about events related to the PDU Session that are detected by the UPF.

The parameters over N4 reference point provided from SMF to UPF comprises an N4 Session ID and may also contain:

-	Packet Detection Rules (PDR) that contain information to classify traffic (PDU(s)) arriving at the UPF;

-	Forwarding Action Rules (FAR) that contain information on whether forwarding, dropping or buffering is to be applied to a traffic identified by PDR(s);

-	Multi-Access Rules (MAR) that contain information on how to handle traffic steering, switching and splitting for a MA PDU Session;

-	Usage Reporting Rules (URR) contains information that defines how traffic identified by PDR(s) shall be accounted as well as how a certain measurement shall be reported;

-	QoS Enforcement Rules (QER), that contain information related to QoS enforcement of traffic identified by PDR(s);

-	Session Reporting Rules (SRR) that contain information to request the UP function to detect and report events for a PDU session that are not related to specific PDRs of the PDU session or that are not related to traffic usage measurement.

-	Trace Requirements;

-	Port Management Information Container in 5GS;

-	Bridge Information.

The N4 Session ID is assigned by the SMF and uniquely identifies an N4 session.

If the UPF indicated support of Trace, the SMF may activate a trace session during a N4 Session Establishment or a N4 Session Modification procedure. In that case it provides Trace Requirements to the UPF. The SMF may deactivate an on-going trace session using a N4 Session Modification procedure. There shall be at most one trace session activated per N4 Session at a time.

For the MA PDU Session, the SMF may add an additional access tunnel information during an N4 Session Modification procedure by updating MAR with addition of an FAR ID which refers to an FAR containing the additional access tunnel information for the MA PDU session for traffic steering in the UPF. For the MA PDU Session, the SMF may request Access Availability report per N4 Session, during N4 Session Establishment procedure or N4 Session Modification procedure.

A N4 Session may be used to control both UPF and NW-TT behaviour in the UPF. A N4 session support and enable exchange of TSN bridge configuration between the SMF and the UPF:

-	Information that the SMF needs for bridge management (clause 5.8.2.11.9);

-	Information that 5GS transparently relays between the TSN AF or NEF and the NW-TT: transparent Port Management Information Container along with the associated NW-TT port number.

-	Information that 5GS transparently relays between the TSN AF or NEF and the NW-TT: transparent NW-TT Management Information Container (clause 5.8.2.11.13).

When a N4 Session related with bridge management is established, the UPF allocates a dedicated port number for the DS-TT side of the PDU Session. The UPF then provides to the SMF following configuration parameters for the N4 Session:

-	DS-TT port number.

-	NW-TT instance ID.

To support TSN, the NW-TT instance ID is Bridge ID.

After the N4 session has been established, the SMF and UPF may at any time exchange transparent NW-TT and  Port Management Information Container over a N4 session.

Next CHANGE

5.8.2.11.9	Bridge Information

The following table describes the NW-TT Information (NI) that includes the information required to configure a 5GS logical bridge for TSC PDU Sessions.

Table 5.8.2.11.9-1: NW-TT Management Information

| Attribute | Description | Comment |
| --- | --- | --- |
| DS-TT Port Number | Port Number allocated by the UPF for the DS-TT for a given PDU Session |  |
| NW-TT ID | NW-TT instance ID which is for TSN identical to the Bridge identifier of the 5GS TSN bridge |  |

Next CHANGE

5.8.2.11.14	TSC Management Information

The following table describes the TSC Management Information Container (TSC MIC) that includes NMIC, PMIC and the associated NW-TT port number.

Table 5.8.2.11.13-1: TSC Management Information Container

| Attribute | Description | Comment |
| --- | --- | --- |
| NW-TTManagement Information Container | 5GS TSN Bridge information exchanged transparently between NW-TT and TSN AF or NEF via 5GS (as in Table 5.28.3.1-2). |  |
| Port Management Information Container | Information exchanged transparently between NW-TT and TSN AF or NEF via 5GS (as in Table 5.28.3.1-1). |  |
| NW-TT Port Number | NW-TT Port Number related to the PMIC. | Included when the PMIC information is present. |

Next CHANGE

5.27.1.4	DS-TT and NW-TT Time Synchronization functionality

DS-TT and NW-TT may support the following PTP instance types:

-	Boundary Clock as defined in IEEE Std 1588 [126];

Editor's note:	Whether DS-TT and NW-TT support Boundary Clock also if 5GS is not acting as grandmaster is FFS.

-	End-to-End Transparent Clock as defined in IEEE Std 1588 [126];

-	Peer-to-Peer Transparent Clock as defined in IEEE Std 1588 [126];

-	PTP Relay instance (i.e. time-aware system) as defined in IEEE Std 802.1AS [104].

Editor's note:	Support for external networks operating with previous version of IEEE Std 1588 [107] is for further study.

DS-TT and NW-TT may support the following transports for PTP:

-	IPv4 as defined in IEEE Std 1588 [126] Annex C;

-	IPv6 as defined in IEEE Std 1588 [126] Annex D;

IEEE 802.3 (Ethernet) as defined in IEEE Std 1588 [126] Annex E.

For operation as a Boundary clock or as a Transparent Clock, DS-TT and NW-TT may support the following path and link delay measurement methods:

-	Delay request-response mechanism as described in IEEE  Std 1588 [126] clause 11.3;

-	Peer-to-peer delay mechanism as defined in IEEE Std 1588 [126] clause 11.4.

DS-TT and NW-TT may support acting as a PTP grandmaster, i.e. may support generating (g)PTP Announce, Sync and Follow_Up messages.

DS-TT and NW-TT may also support PTP profiles as described in IEEE Std 1588 [126] clause 20.3,  i.e.:

-	IEEE Std 802.1AS [104] PTP profile for transport of timing as defined in IEEE Std 802.1AS [104] Annex F;

-	SMPTE Profile for Use of IEEE Std 1588 [126] Precision Time Protocol in Professional Broadcast Applications [127].

TSN AF and NEF may determine the PTP functionalities supported by DS-TT and NW-TT and may configure PTP instances in DS-TT and NW-TT using port and NW-TT management information exchange as described in Annex K, clause K.2.

Next CHANGE

### 5.28.3	Port and NW-TT management information exchange in 5GS

#### 5.28.3.1	General

Port number of the DS-TT for the PDU Session is assigned by the UPF during PDU session establishment. The port number of the DS-TT port for a PDU Session shall be reported to the SMF from the UPF and further stored at the SMF. The SMF provides the DS-TT port number via PCF to the TSN AF or NEF. TSN AF or NEF maintains an association between the DS-TT port number and the MAC address (with Ethernet type PDU session) or IP address (with IP type PDU Session) of the UE. If a PDU session for which SMF has reported a DS-TT port number to TSN AF or NEF is released, then SMF informs TSN AF or NEF accordingly.

NOTE:	Port number can refer either to Ethernet port or PTP port. In Ethernet type PDU Sessions, it is assumed that the PTP port number is the same as the associated Ethernet port number.

5GS shall support transfer of standardized and deployment-specific port management information transparently between TSN AF or NEF and DS-TT or NW-TT, respectively inside a Port Management Information Container. NW-TT may support one or more ports. In this case, each port uses separate Port Management Information Container. 5GS shall also support transfer of standardized and deployment-specific NW-TT management information transparently between TSN AF or NEF and NW-TT, respectively inside a NW-TT Management Information Container. Table 5.28.3.1-1 and Table 5.28.3.1-2 list standardized port management information and bridge management information, respectively.

If TSN AF is deployed, i.e. if 5GS is integrated with an IEEE TSN network, the port and NW-TT management information is exchanged between CNC and TSN AF. The port management information, is related to ports located in DS-TT or NW-TT.

If TSN AF is not deployed, the port and NW-TT management information is exchanged between NEF and DS-TT/NW-TT.

Editor's note:	It is FFS which of the information elements in the tables below are exchanged between NEF and DS-TT/NW-TT.

Table 5.28.3.1-1: Standardized port management information

| Port management information | Applicability (see NOTE 6) |  | Supported operations by TSN AF | Reference |
| --- | --- | --- | --- | --- |
|  | DS-TT | NW-TT | (see NOTE 1) |  |
| General |  |  |  |  |
| Port management capabilities (see NOTE 2) | X | X | R |  |
| Bridge delay related information |  |  |  |  |
| txPropagationDelay | X | X | R | IEEE Std 802.1Qcc [95] clause 12.32.2.1 |
| Traffic class related information |  |  |  |  |
| Traffic class table | X | X | RW | IEEE Std 802.1Q [98] clause 12.6.3 and clause 8.6.6. |
| Gate control information |  |  |  |  |
| GateEnabled | X | X | RW | IEEE Std 802.1Q [98] Table 12-29 |
| AdminBaseTime | X | X | RW | IEEE Std 802.1Q [98] Table 12-29 |
| AdminControlList | X | X | RW | IEEE Std 802.1Q [98] Table 12-29 |
| AdminCycleTime (see NOTE 3) | X | X | RW | IEEE Std 802.1Q [98] Table 12-29 |
| AdminControlListLength (see NOTE 3) | X | X | RW | IEEE Std 802.1Q [98] Table 12-28 |
| Tick granularity | X | X | R | IEEE Std 802.1Q [98] Table 12-29 |
| General Neighbor discovery configuration (NOTE 4) |  |  |  |  |
| adminStatus | D | X | RW | IEEE Std 802.1AB [97] clause 9.2.5.1 |
| lldpV2LocChassisIdSubtype | D | X | RW | IEEE Std 802.1AB [97] Table 11-2 |
| lldpV2LocChassisId | D | X | RW | IEEE Std 802.1AB [97] Table 11-2 |
| lldpV2MessageTxInterval | D | X | RW | IEEE Std 802.1AB [97] Table 11-2 |
| lldpV2MessageTxHoldMultiplier | D | X | RW | IEEE Std 802.1AB [97] Table 11-2 |
| NW-TT port neighbor discovery configuration |  |  |  |  |
| lldpV2LocPortIdSubtype |  | X | RW | IEEE Std 802.1AB [97] Table 11-2 |
| lldpV2LocPortId |  | X | RW | IEEE Std 802.1AB [97] Table 11-2 |
| DS-TT port neighbor discovery configuration |  |  |  |  |
| lldpV2LocPortIdSubtype | D |  | RW | IEEE Std 802.1AB [97] Table 11-2 |
| lldpV2LocPortId | D |  | RW | IEEE Std 802.1AB [97] Table 11-2 |
| Neighbor discovery information for each discovered neighbor of NW-TT |  |  |  |  |
| lldpV2RemChassisIdSubtype |  | X | R | IEEE Std 802.1AB [97] Table 11-2 |
| lldpV2RemChassisId |  | X | R | IEEE Std 802.1AB [97] Table 11-2 |
| lldpV2RemPortIdSubtype |  | X | R | IEEE Std 802.1AB [97] Table 11-2 |
| lldpV2RemPortId |  | X | R | IEEE Std 802.1AB [97] Table 11-2 |
| TTL |  | X | R | IEEE Std 802.1AB [97] clause 8.5.4 |
| Neighbor discovery information for each discovered neighbor of DS-TT (NOTE 5) |  |  |  |  |
| lldpV2RemChassisIdSubtype | D |  | R | IEEE Std 802.1AB [97] Table 11-2 |
| lldpV2RemChassisId | D |  | R | IEEE Std 802.1AB [97] Table 11-2 |
| lldpV2RemPortIdSubtype | D |  | R | IEEE Std 802.1AB [97] Table 11-2 |
| lldpV2RemPortId | D |  | R | IEEE Std 802.1AB [97] Table 11-2 |
| TTL | D |  | R | IEEE Std 802.1AB [97] clause 8.5.4.1 |
| Stream Parameters (NOTE 11) |  |  |  |  |
| MaxStreamFilterInstances | X |  | R | IEEE Std 802.1Q [98] clause 12.31.1.1 |
| MaxStreamGateInstances | X |  | R | IEEE Std 802.1Q [98] clause 12.31.1.2 |
| MaxFlowMeterInstances | X |  | R | IEEE Std 802.1Q [98] clause 12.31.1.3 |
| SupportedListMax | X |  | R | IEEE Std 802.1Q [98] clause 12.31.1.4 |
| Per-Stream Filtering and Policing information (NOTE 10) |  |  |  |  |
| Stream Filter Instance Table (NOTE 8) |  |  |  | IEEE Std 802.1Q [98] Table 12-32 |
| > StreamFilterInstanceIndex | X | X | RW | IEEE Std 802.1Q [98] Table 12-32 |
| > Stream Identification type | X | X | RW | IEEE Std 802.1CB [83] clause 9.1.1.6 |
| > Stream Identification Controlling Parameters | X | X | RW | IEEE Std 802.1CB [83] clauses 9.1.2, 9.1.3, 9.1.4 (NOTE 12) |
| > PrioritySpec | X | X | RW | IEEE Std 802.1Q [98] Table 12-32 |
| > StreamGateInstanceID | X | X | RW | IEEE Std 802.1Q [98] Table 12-32 |
| Stream Gate Instance Table (NOTE 9) |  |  |  | IEEE Std 802.1Q [98] Table 12-33 |
| StreamGateInstanceIndex | X | X | R | IEEE Std 802.1Q [98] Table 12-33 |
| PSFPAdminBaseTime | X | X | RW | IEEE Std 802.1Q [98] Table 12-33 |
| PSFPAdminControlList | X | X | RW | IEEE Std 802.1Q [98] Table 12-33 |
| PSFPAdminCycleTime | X | X | RW | IEEE Std 802.1Q [98] Table 12-33 |
| PSFPTickGranularity | X | X | R | IEEE Std 802.1Q [98] Table 12-33 |
| Time Synchronization Information |  |  |  |  |
| Supported PTP instance types (NOTE 13) | X |  | R | IEEE Std 1588 [126] clause 8.2.1.5.5 |
| Supported transport types (NOTE 14) | X |  | R |  |
| Supported delay mechanisms (NOTE 15) | X |  | R | IEEE Std 1588 [126] clause 8.2.15.4.4 |
| PTP grandmaster capable (NOTE 16) | X |  | R |  |
| gPTP grandmaster capable (NOTE 17) | X |  | R |  |
| Supported PTP profiles (NOTE 18) | X |  | R |  |
| Number of supported PTP instances | X |  | R |  |
| PTP Instance ID | X | X | RW |  |
| > PTP profile (NOTE 19) | X | X | RW |  |
| > Transport type (NOTE 20) | X | X | RW |  |
| > Grandmaster enabled (NOTE 21) | X |  | RW |  |
| IEEE Std 1588 [126] data sets (NOTE 22) |  |  |  |  |
| > defaultDS.clockIdentity | X | X | RW | IEEE Std 1588 [126] clause 8.2.1.2.2 |
| > defaultDS.clockQuality.clockClass | X | X | RW | IEEE Std 1588 [126] clause 8.2.1.3.1.2 |
| > defaultDS.clockQuality.clockAccuracy | X | X | RW | IEEE Std 1588 [126] clause 8.2.1.3.1.3 |
| > defaultDS.clockQuality.offsetScaledLogVariance | X | X | RW | IEEE Std 1588 [126] clause 8.2.1.3.1.4 |
| > defaultDS.priority1 | X | X | RW | IEEE Std 1588 [126] clause 8.2.1.4.1 |
| > defaultDS.priority2 | X | X | RW | IEEE Std 1588 [126] clause 8.2.1.4.2 |
| > defaultDS.domainNumber | X | X | RW | IEEE Std 1588 [126] clause 8.2.1.4.3 |
| > defaultDS.sdoId | X | X | RW | IEEE Std 1588 [126] clause 8.2.1.4.5 |
| > defaultDS.instanceEnable | X | X | RW | IEEE Std 1588 [126] clause 8.2.1.5.2 |
| > defaultDS.externalPortConfigurationEnabled |  | X | RW | IEEE Std 1588 [126] clause 8.2.1.5.3 |
| > defaultDS.instanceType | X | X | RW | IEEE Std 1588 [126] clause 8.2.1.5.5 |
| > portDS.portIdentity | X | X | RW | IEEE Std 1588 [126] clause 8.2.15.2.1 |
| > portDS.portState | X | X | RW | IEEE Std 1588 [126] clause 8.2.15.3.1 |
| > portDS.logMinDelayReqInterval | X | X | RW | IEEE Std 1588 [126] clause 8.2.15.3.2 |
| > portDS.logAnnounceInterval | X | X | RW | IEEE Std 1588 [126] clause 8.2.15.4.1 |
| > portDS.announceReceiptTimeout |  | X | RW | IEEE Std 1588 [126] clause 8.2.15.4.2 |
| > portDS.logSyncInterval | X | X | RW | IEEE Std 1588 [126] clause 8.2.15.4.3 |
| > portDS.delayMechanism | X | X | RW | IEEE Std 1588 [126] clause 8.2.15.4.4 |
| > portDS.logMinPdelayReqInterval | X | X | RW | IEEE Std 1588 [126] clause 8.2.15.4.5 |
| > portDS.versionNumber | X | X | RW | IEEE Std 1588 [126] clause 8.2.15.4.6 |
| > portDS.minorVersionNumber | X | X | RW | IEEE Std 1588 [126] clause 8.2.15.4.7 |
| > portDS.delayAssymetry | X | X | RW | IEEE Std 1588 [126] clause 8.2.15.4.8 |
| > portDS.portEnable | X | X | RW | IEEE Std 1588 [126] clause 8.2.15.5.1 |
| > timePropertiesDS.currentUtcOffset | X | X | RW | IEEE Std 1588 [126] clause 8.2.4.2 |
| > timePropertiesDS.timeSource | X | X | RW | IEEE Std 1588 [126] clause 8.2.4.9 |
| > externalPortConfigurationPortDS.desiredState |  | X | RW | IEEE Std 1588 [126] clause 15.5.3.7.15.1 |
| IEEE Std 802.1AS [104] data sets (NOTE 22) |  |  |  |  |
| > defaultDS.clockIdentity | X | X | RW | IEEE Std 802.1AS [104] clause 14.2.2 |
| > defaultDS.clockQuality.clockClass | X | X | RW | IEEE Std 802.1AS [104] clause 14.2.4.2 |
| > defaultDS.clockQuality.clockAccuracy | X | X | RW | IEEE Std 802.1AS [104] clause 14.2.4.3 |
| > defaultDS.clockQuality.offsetScaledLogVariance | X | X | RW | IEEE Std 802.1AS [104] clause 14.2.4.4 |
| > defaultDS.priority1 | X | X | RW | IEEE Std 802.1AS [104] clause 14.2.5 |
| > defaultDS.priority2 | X | X | RW | IEEE Std 802.1AS [104] clause 14.2.6 |
| > defaultDS.timeSource | X | X | RW | IEEE Std 802.1AS [104] clause 14.2.15 |
| > defaultDS.domainNumber | X | X | RW | IEEE Std 802.1AS [104] clause 14.2.16 |
| > defaultDS.sdoId | X | X | RW | IEEE Std 802.1AS [104] clause 14.2.4.3 |
| > defaultDS.externalPortConfigurationEnabled |  | X | RW | IEEE Std 802.1AS [104] clause 14.2.18 |
| > defaultDS.instanceEnable | X | X | RW | IEEE Std 802.1AS [104] clause 14.2.19 |
| > portDS.portIdentity | X | X | RW | IEEE Std 802.1AS [104] clause 14.8.2 |
| > portDS.portState |  | X | R | IEEE Std 802.1AS [104] clause 14.8.3 |
| > portDS.ptpPortEnabled | X | X | RW | IEEE Std 802.1AS [104] clause 14.8.4 |
| > portDS.delayMechanism | X | X | RW | IEEE Std 802.1AS [104] clause 14.8.5 |
| > portDS.isMeasuringDelay | X | X | R | IEEE Std 802.1AS [104] clause 14.8.6 |
| > portDS.asCapable | X | X | R | IEEE Std 802.1AS [104] clause 14.8.7 |
| > portDS.meanLinkDelay | X | X | R | IEEE Std 802.1AS [104] clause 14.8.8 |
| > portDS.meanLinkDelayThresh | X | X | RW | IEEE Std 802.1AS [104] clause 14.8.9 |
| > portDS.delayAssymetry | X | X | RW | IEEE Std 802.1AS [104] clause 14.8.10 |
| > portDS.neighborRateRatio | X | X | R | IEEE Std 802.1AS [104] clause 14.8.11 |
| > portDS.initialLogAnnounceInterval | X | X | RW | IEEE Std 802.1AS [104] clause 14.8.12 |
| > portDS.currentLogAnnounceInterval | X | X | R | IEEE Std 802.1AS [104] clause 14.8.13 |
| > portDS.useMgtSettableLogAnnounceInterval | X | X | RW | IEEE Std 802.1AS [104] clause 14.8.14 |
| > portDS.mgtSettableLogAnnounceInterval | X | X | RW | IEEE Std 802.1AS [104] clause 14.8.15 |
| > portDS.announceReceiptTimeout |  | X | RW | IEEE Std 802.1AS [104] clause 14.8.16 |
| > portDS.initialLogSyncInterval | X | X | RW | IEEE Std 802.1AS [104] clause 14.8.17 |
| > portDS.currentLogSyncInterval | X | X | R | IEEE Std 802.1AS [104] clause 14.8.18 |
| > portDS.useMgtSettableLogSyncInterval | X | X | RW | IEEE Std 802.1AS [104] clause 14.8.19 |
| > portDS.mgtSettableLogSyncInterval | X | X | RW | IEEE Std 802.1AS [104] clause 14.8.20 |
| > portDS.syncReceiptTimeout |  | X | RW | IEEE Std 802.1AS [104] clause 14.8.21 |
| > portDS.syncReceiptTimeoutTimeInterval |  | X | RW | IEEE Std 802.1AS [104] clause 14.8.22 |
| > portDS.initialLogPdelayReqInterval | X | X | RW | IEEE Std 802.1AS [104] clause 14.8.23 |
| > portDS.currentLogPdelayReqInterval | X | X | R | IEEE Std 802.1AS [104] clause 14.8.24 |
| > portDS.useMgtSettableLogPdelayReqInterval | X | X | RW | IEEE Std 802.1AS [104] clause 14.8.25 |
| > portDS.mgtSettableLogPdelayReqInterval | X | X | RW | IEEE Std 802.1AS [104] clause 14.8.26 |
| > portDS.initialLogGptpCapableMessageInterval | X | X | RW | IEEE Std 802.1AS [104] clause 14.8.27 |
| > portDS.currentLogGptpCapableMessageInterval | X | X | R | IEEE Std 802.1AS [104] clause 14.8.28 |
| > portDS.useMgtSettableLogGptpCapableMessageInterval | X | X | RW | IEEE Std 802.1AS [104] clause 14.8.29 |
| > portDS.mgtSettableLogGptpCapableMessageInterval | X | X | RW | IEEE Std 802.1AS [104] clause 14.8.30 |
| > portDS.initialComputeNeighborRateRatio | X | X | RW | IEEE Std 802.1AS [104] clause 14.8.31 |
| > portDS.currentComputeNeighborRateRatio | X | X | R | IEEE Std 802.1AS [104] clause 14.8.32 |
| > portDS.useMgtSettableComputeNeighborRateRatio | X | X | RW | IEEE Std 802.1AS [104] clause 14.8.33 |
| > portDS.mgtSettableComputeNeighborRateRatio | X | X | RW | IEEE Std 802.1AS [104] clause 14.8.34 |
| > portDS.initialComputeMeanLinkDelay | X | X | RW | IEEE Std 802.1AS [104] clause 14.8.35 |
| > portDS.currentComputeMeanLinkDelay | X | X | R | IEEE Std 802.1AS [104] clause 14.8.36 |
| > portDS.useMgtSettableComputeMeanLinkDelay | X | X | RW | IEEE Std 802.1AS [104] clause 14.8.37 |
| > portDS.mgtSettableComputeMeanLinkDelay | X | X | RW | IEEE Std 802.1AS [104] clause 14.8.38 |
| > portDS.allowedLostResponses | X | X | RW | IEEE Std 802.1AS [104] clause 14.8.39 |
| > portDS.allowedFaults | X | X | RW | IEEE Std 802.1AS [104] clause 14.8.40 |
| > portDS.gPtpCapableReceiptTimeout | X | X | RW | IEEE Std 802.1AS [104] clause 14.8.41 |
| > portDS.versionNumber | X | X | RW | IEEE Std 802.1AS [104] clause 14.8.42 |
| > portDS.nup | X | X | RW | IEEE Std 802.1AS [104] clause 14.8.43 |
| > portDS.ndown | X | X | RW | IEEE Std 802.1AS [104] clause 14.8.44 |
| > portDS.oneStepTxOper | X | X | R | IEEE Std 802.1AS [104] clause 14.8.45 |
| > portDS.oneStepReceive | X | X | R | IEEE Std 802.1AS [104] clause 14.8.46 |
| > portDS.oneStepTransmit | X | X | R | IEEE Std 802.1AS [104] clause 14.8.47 |
| > portDS.initialOneStepTxOper | X | X | RW | IEEE Std 802.1AS [104] clause 14.8.48 |
| > portDS.currentOneStepTxOper | X | X | RW | IEEE Std 802.1AS [104] clause 14.8.49 |
| > portDS.useMgtSettableOneStepTxOper | X | X | RW | IEEE Std 802.1AS [104] clause 14.8.50 |
| > portDS.mgtSettableOneStepTxOper | X | X | RW | IEEE Std 802.1AS [104] clause 14.8.51 |
| > portDS.syncLocked | X | X | R | IEEE Std 802.1AS [104] clause 14.8.52 |
| > portDS.pdelayTruncatedTimestampsArray | X | X | RW | IEEE Std 802.1AS [104] clause 14.8.53 |
| > portDS.minorVersionNumber | X | X | RW | IEEE Std 802.1AS [104] clause 14.8.54 |
| > timePropertiesDS.currentUtcOffset | X | X | RW | IEEE Std 802.1AS [104] clause 14.5.2 |
| > externalPortConfigurationPortDS.desiredState |  | R | RW | IEEE Std 802.1AS [104] clause 14.12.2 |
| NOTE 1: R = Read only access; RW = Read/Write access. NOTE 2: Indicates which standardized and deployment-specific port management information is supported by DS-TT or NW-TT. NOTE 3: AdminCycleTime and AdminControlListLength are optional for gate control information. NOTE 4: If DS-TT supports neighbor discovery, then TSN AF sends the general neighbor discovery configuration for DS-TT Ethernet ports to DS-TT. If DS-TT does not support neighbor discovery, then TSN AF sends the general neighbor discovery configuration for DS-TT Ethernet ports to NW-TT using the Bridge Management Information Container (refer to Table 5.28.3.1-2) and NW-TT performs neighbor discovery on behalf on DS-TT. When a parameter in this group is changed, it is necessary to provide the change to every DS-TT and the NW-TT that belongs to the 5GS TSN bridge. It is mandatory that the general neighbor discovery configuration is identical for all DS-TTs and the NW-TTs that belongs to the bridge. NOTE 5: If DS-TT supports neighbor discovery, then TSN AF retrieves neighbor discovery information for DS-TT Ethernet ports from DS-TT. If DS-TT does not support neighbor discovery, then TSN AF retrieves neighbor discovery information for DS-TT Ethernet ports from NW-TT, using the Bridge Management Information Container (refer to Table 5.28.3.1-2), the NW-TT performing neighbor discovery on behalf on DS-TT. NOTE 6: X = applicable; D = applicable when validation and generation of LLDP frames is processed at the DS-TT. NOTE 7: Void. NOTE 8: There is a Stream Filter Instance Table per Stream. NOTE 9: There is a Stream Gate Instance Table per Gate. NOTE 10: TSN AF indicates the support for PSFP to the CNC only if each DS-TT and NW-TT of the 5GS bridge has indicated support of PSFP. DS-TT indicates support of PSFP using port management capabilities, i.e. by indicating support for the Per-Stream Filtering and Policing information and by setting higher than zero values for MaxStreamFilterInstances, MaxStreamGateInstances, MaxFlowMeterInstances, SupportedListMax parameters. When available, TSN AF uses the PSFP information for determination of the traffic pattern information as described in Annex I. The PSFP information can be used at the DS-TT (if supported) and at the NW-TT (if supported) for the purpose of per-stream filtering and policing as defined in IEEE Std 802.1Q [98] clause 8.6.5.1. NOTE 11: TSN AF composes a Stream Parameter Table towards the CNC. It is up to TSN AF how it composes the Stream Parameter Table based on the numerical values as received from DS-TT and NW-TT port(s) and for the bridge for each individual parameter. NOTE 12: The set of Stream Identification Controlling Parameters depends on the Stream Identification type value as defined in IEEE Std 802.1CB [83] Table 9-1 and clauses 9.1.2, 9.1.3, 9.1.4. NOTE 13: Enumeration of supported PTP instance types. Allowed values as defined in IEEE Std 1588 [126] clause 8.2.1.5.5. NOTE 14: Enumeration of supported transport types. Allowed values: IPv4 (as defined in IEEE Std 1588 [126] Annex C), IPv6 (as defined in IEEE Std 1588 [126] Annex D), Ethernet (as defined in IEEE Std 1588 [126] Annex E). NOTE 15: Enumeration of supported PTP delay mechanisms. Allowed values as defined in IEEE Std 1588 [126] clause 8.2.15.4.4. NOTE 16: Indicates whether DS-TT supports acting as a PTP grandmaster. NOTE 17: Indicates whether DS-TT supports acting as a gPTP grandmaster. NOTE 18: Enumeration of supported PTP profiles, each identified by PTP profile ID, as defined in IEEE Std 1588 [126] clause 20.3.3. NOTE 19: PTP profile to apply, identified by PTP profile ID, as defined in IEEE Std 1588 [126] clause 20.3.3. NOTE 20: Transport type to use. Allowed values: IPv4 (as defined in IEEE Std 1588 [126] Annex C), IPv6 (as defined in IEEE Std 1588 [126] Annex D), Ethernet (as defined in IEEE Std 1588 [126] Annex E). NOTE 21: Indicates whether to act as grandmaster or not, i.e. whether to send Announce, Sync and optionally Follow_Up messages. NOTE 22: The IEEE Std 802.1AS [104] data sets apply if the IEEE 802.1AS PTP profile is used; otherwise the IEEE Std 1588 [126] data sets apply. |  |  |  |  |

Table 5.28.3.1-2: Standardized NW-TT management information

| Bridge management information | Supported operations by TSN AF (see NOTE 1) | Reference |
| --- | --- | --- |
| Information for 5GS Bridge |  |  |
| NW-TT Address | R |  |
| NW-TT instance ID | R |  |
| NW-TT port numbers | R |  |
| Traffic forwarding information |  |  |
| Static Filtering Entry (NOTE 3) | RW | IEEE Std 802.1Q [98] clause 8.8.1 |
| General Neighbor discovery configuration (NOTE 2) |  |  |
| adminStatus | RW | IEEE Std 802.1AB [97] clause 9.2.5.1 |
| lldpV2LocChassisIdSubtype | RW | IEEE Std 802.1AB [97] Table 11-2 |
| lldpV2LocChassisId | RW | IEEE Std 802.1AB [97] Table 11-2 |
| lldpV2MessageTxInterval | RW | IEEE Std 802.1AB [97] Table 11-2 |
| lldpV2MessageTxHoldMultiplier | RW | IEEE Std 802.1AB [97] Table 11-2 |
| DS-TT port neighbor discovery configuration for DS-TT ports (NOTE 4) |  |  |
| >DS-TT port neighbor discovery configuration for each DS-TT port |  |  |
| >> DS-TT port number | RW |  |
| >> lldpV2LocPortIdSubtype | RW | IEEE Std 802.1AB [97] Table 11-2 |
| >> lldpV2LocPortId | RW | IEEE Std 802.1AB [97] Table 11-2 |
| Discovered neighbor information for DS-TT ports (NOTE 4) |  |  |
| >Discovered neighbor information for each DS-TT port (NOTE 4) |  |  |
| >> DS-TT port number | R |  |
| >> lldpV2RemChassisIdSubtype | R | IEEE Std 802.1AB [97] Table 11-2 |
| >> lldpV2RemChassisId | R | IEEE Std 802.1AB [97] Table 11-2 |
| >> lldpV2RemPortIdSubtype | R | IEEE Std 802.1AB [97] Table 11-2 |
| >> lldpV2RemPortId | R | IEEE Std 802.1AB [97] Table 11-2 |
| >> TTL | R | IEEE Std 802.1AB [97] clause 8.5.4.1 |
| Stream Parameters (NOTE 5) |  |  |
| MaxStreamFilterInstances | R | IEEE Std 802.1Q [98] |
| MaxStreamGateInstances | R | IEEE Std 802.1Q [98] |
| MaxFlowMeterInstances | R | IEEE Std 802.1Q [98] |
| SupportedListMax | R | IEEE Std 802.1Q [98] |
| Time synchronization information |  |  |
| Supported PTP instance types (NOTE 6) | R |  |
| Supported transport types (NOTE 7) | R |  |
| Supported delay mechanisms (NOTE 8) | R |  |
| PTP grandmaster capable (NOTE 9) | R |  |
| gPTP grandmaster capable (NOTE 10) | R |  |
| Supported PTP profiles (NOTE 11) | R |  |
| Number of supported PTP instances | R |  |
| Grandmaster candidate enabled | RW |  |
| Time synchronization information for DS-TT ports |  |  |
| > Time synchronization information for each DS-TT port |  |  |
| > DS-TT port number | RW |  |
| >> Time synchronization information for each PTP Instance |  |  |
| >> PTP Instance ID | RW |  |
| >> PTP profile (NOTE 12) | RW |  |
| >> Transport type (NOTE 13) | RW |  |
| >> Grandmaster on behalf of DS-TT enabled (NOTE 14) | RW |  |
| IEEE Std 1588 [126] data sets (NOTE 15) |  |  |
| >> defaultDS.clockIdentity | RW | IEEE Std 1588 [126] clause 8.2.1.2.2 |
| >> defaultDS.clockQuality.clockClass | RW | IEEE Std 1588 [126] clause 8.2.1.3.1.2 |
| >> defaultDS.clockQuality.clockAccuracy | RW | IEEE Std 1588 [126] clause 8.2.1.3.1.3 |
| >> defaultDS.clockQuality.offsetScaledLogVariance | RW | IEEE Std 1588 [126] clause 8.2.1.3.1.4 |
| >> defaultDS.priority1 | RW | IEEE Std 1588 [126] clause 8.2.1.4.1 |
| >> defaultDS.priority2 | RW | IEEE Std 1588 [126] clause 8.2.1.4.2 |
| >> defaultDS.domainNumber | RW | IEEE Std 1588 [126] clause 8.2.1.4.3 |
| >> defaultDS.sdoId | RW | IEEE Std 1588 [126] clause 8.2.1.4.5 |
| >> defaultDS.instanceEnable | RW | IEEE Std 1588 [126] clause 8.2.1.5.2 |
| >> defaultDS.externalPortConfigurationEnabled | RW | IEEE Std 1588 [126] clause 8.2.1.5.3 |
| >> defaultDS.instanceType | RW | IEEE Std 1588 [126] clause 8.2.1.5.5 |
| >> portDS.portIdentity | RW | IEEE Std 1588 [126] clause 8.2.15.2.1 |
| >> portDS.portState | R | IEEE Std 1588 [126] clause 8.2.15.3.1 |
| >> portDS.logMinDelayReqInterval | RW | IEEE Std 1588 [126] clause 8.2.15.3.2 |
| >> portDS.logAnnounceInterval | RW | IEEE Std 1588 [126] clause 8.2.15.4.1 |
| >> portDS.announceReceiptTimeout | RW | IEEE Std 1588 [126] clause 8.2.15.4.2 |
| >> portDS.logSyncInterval | RW | IEEE Std 1588 [126] clause 8.2.15.4.3 |
| >> portDS.delayMechanism | RW | IEEE Std 1588 [126] clause 8.2.15.4.4 |
| >> portDS.logMinPdelayReqInterval | RW | IEEE Std 1588 [126] clause 8.2.15.4.5 |
| >> portDS.versionNumber | RW | IEEE Std 1588 [126] clause 8.2.15.4.6 |
| >> portDS.minorVersionNumber | RW | IEEE Std 1588 [126] clause 8.2.15.4.7 |
| >> portDS.delayAssymetry | RW | IEEE Std 1588 [126] clause 8.2.15.4.8 |
| >> portDS.portEnable | RW | IEEE Std 1588 [126] clause 8.2.15.5.1 |
| >> timePropertiesDS.currentUtcOffset | RW | IEEE Std 1588 [126] clause 8.2.4.2 |
| >> timePropertiesDS.timeSource | RW | IEEE Std 1588 [126] clause 8.2.4.9 |
| >> externalPortConfigurationPortDS.desiredState | RW | IEEE Std 1588 [126] clause 15.5.3.7.15.1 |
| IEEE Std 802.1AS [104] data sets (NOTE 15) |  |  |
| >> defaultDS.clockIdentity | RW | IEEE Std 802.1AS [104] clause 14.2.2 |
| >> defaultDS.clockQuality.clockClass | RW | IEEE Std 802.1AS [104] clause 14.2.4.2 |
| >> defaultDS.clockQuality.clockAccuracy | RW | IEEE Std 802.1AS [104] clause 14.2.4.3 |
| >> defaultDS.clockQuality.offsetScaledLogVariance | RW | IEEE Std 802.1AS [104] clause 14.2.4.4 |
| >> defaultDS.priority1 | RW | IEEE Std 802.1AS [104] clause 14.2.5 |
| >> defaultDS.priority2 | RW | IEEE Std 802.1AS [104] clause 14.2.6 |
| >> defaultDS.timeSource | RW | IEEE Std 802.1AS [104] clause 14.2.15 |
| >> defaultDS.domainNumber | RW | IEEE Std 802.1AS [104] clause 14.2.16 |
| >> defaultDS.externalPortConfigurationEnabled | RW | IEEE Std 802.1AS [104] clause 14.2.18 |
| >> defaultDS.sdoId | RW | IEEE Std 802.1AS [104] clause 14.2.4.3 |
| >> defaultDS.instanceEnable | RW | IEEE Std 802.1AS [104] clause 14.2.19 |
| >> portDS.portIdentity | RW | IEEE Std 802.1AS [104] clause 14.8.2 |
| >> portDS.portState | R | IEEE Std 802.1AS [104] clause 14.8.3 |
| >> portDS.ptpPortEnabled | RW | IEEE Std 802.1AS [104] clause 14.8.4 |
| >> portDS.delayMechanism | RW | IEEE Std 802.1AS [104] clause 14.8.5 |
| >> portDS.isMeasuringDelay | R | IEEE Std 802.1AS [104] clause 14.8.6 |
| >> portDS.asCapable | R | IEEE Std 802.1AS [104] clause 14.8.7 |
| >> portDS.meanLinkDelay | R | IEEE Std 802.1AS [104] clause 14.8.8 |
| >> portDS.meanLinkDelayThresh | RW | IEEE Std 802.1AS [104] clause 14.8.9 |
| >> portDS.delayAssymetry | RW | IEEE Std 802.1AS [104] clause 14.8.10 |
| >> portDS.neighborRateRatio | R | IEEE Std 802.1AS [104] clause 14.8.11 |
| >> portDS.initialLogAnnounceInterval | RW | IEEE Std 802.1AS [104] clause 14.8.12 |
| >> portDS.currentLogAnnounceInterval | R | IEEE Std 802.1AS [104] clause 14.8.13 |
| >> portDS.useMgtSettableLogAnnounceInterval | RW | IEEE Std 802.1AS [104] clause 14.8.14 |
| >> portDS.mgtSettableLogAnnounceInterval | RW | IEEE Std 802.1AS [104] clause 14.8.15 |
| >> portDS.announceReceiptTimeout | RW | IEEE Std 802.1AS [104] clause 14.8.16 |
| >> portDS.initialLogSyncInterval | RW | IEEE Std 802.1AS [104] clause 14.8.17 |
| >> portDS.currentLogSyncInterval | R | IEEE Std 802.1AS [104] clause 14.8.18 |
| >> portDS.useMgtSettableLogSyncInterval | RW | IEEE Std 802.1AS [104] clause 14.8.19 |
| >> portDS.mgtSettableLogSyncInterval | RW | IEEE Std 802.1AS [104] clause 14.8.20 |
| >> portDS.syncReceiptTimeout | RW | IEEE Std 802.1AS [104] clause 14.8.21 |
| >> portDS.syncReceiptTimeoutTimeInterval | RW | IEEE Std 802.1AS [104] clause 14.8.22 |
| >> portDS.initialLogPdelayReqInterval | RW | IEEE Std 802.1AS [104] clause 14.8.23 |
| >> portDS.currentLogPdelayReqInterval | R | IEEE Std 802.1AS [104] clause 14.8.24 |
| >> portDS.useMgtSettableLogPdelayReqInterval | RW | IEEE Std 802.1AS [104] clause 14.8.25 |
| >> portDS.mgtSettableLogPdelayReqInterval | RW | IEEE Std 802.1AS [104] clause 14.8.26 |
| >> portDS.initialLogGptpCapableMessageInterval | RW | IEEE Std 802.1AS [104] clause 14.8.27 |
| >> portDS.currentLogGptpCapableMessageInterval | R | IEEE Std 802.1AS [104] clause 14.8.28 |
| >> portDS.useMgtSettableLogGptpCapableMessageInterval | RW | IEEE Std 802.1AS [104] clause 14.8.29 |
| >> portDS.mgtSettableLogGptpCapableMessageInterval | RW | IEEE Std 802.1AS [104] clause 14.8.30 |
| >> portDS.initialComputeNeighborRateRatio | RW | IEEE Std 802.1AS [104] clause 14.8.31 |
| >> portDS.currentComputeNeighborRateRatio | R | IEEE Std 802.1AS [104] clause 14.8.32 |
| >> portDS.useMgtSettableComputeNeighborRateRatio | RW | IEEE Std 802.1AS [104] clause 14.8.33 |
| >> portDS.mgtSettableComputeNeighborRateRatio | RW | IEEE Std 802.1AS [104] clause 14.8.34 |
| >> portDS.initialComputeMeanLinkDelay | RW | IEEE Std 802.1AS [104] clause 14.8.35 |
| >> portDS.currentComputeMeanLinkDelay | R | IEEE Std 802.1AS [104] clause 14.8.36 |
| >> portDS.useMgtSettableComputeMeanLinkDelay | RW | IEEE Std 802.1AS [104] clause 14.8.37 |
| >> portDS.mgtSettableComputeMeanLinkDelay | RW | IEEE Std 802.1AS [104] clause 14.8.38 |
| >> portDS.allowedLostResponses | RW | IEEE Std 802.1AS [104] clause 14.8.39 |
| >> portDS.allowedFaults | RW | IEEE Std 802.1AS [104] clause 14.8.40 |
| >> portDS.gPtpCapableReceiptTimeout | RW | IEEE Std 802.1AS [104] clause 14.8.41 |
| >> portDS.versionNumber | RW | IEEE Std 802.1AS [104] clause 14.8.42 |
| >> portDS.nup | RW | IEEE Std 802.1AS [104] clause 14.8.43 |
| >> portDS.ndown | RW | IEEE Std 802.1AS [104] clause 14.8.44 |
| >> portDS.oneStepTxOper | R | IEEE Std 802.1AS [104] clause 14.8.45 |
| >> portDS.oneStepReceive | R | IEEE Std 802.1AS [104] clause 14.8.46 |
| >> portDS.oneStepTransmit | R | IEEE Std 802.1AS [104] clause 14.8.47 |
| >> portDS.initialOneStepTxOper | RW | IEEE Std 802.1AS [104] clause 14.8.48 |
| >> portDS.currentOneStepTxOper | RW | IEEE Std 802.1AS [104] clause 14.8.49 |
| >> portDS.useMgtSettableOneStepTxOper | RW | IEEE Std 802.1AS [104] clause 14.8.50 |
| >> portDS.mgtSettableOneStepTxOper | RW | IEEE Std 802.1AS [104] clause 14.8.51 |
| >> portDS.syncLocked | R | IEEE Std 802.1AS [104] clause 14.8.52 |
| >> portDS.pdelayTruncatedTimestampsArray | RW | IEEE Std 802.1AS [104] clause 14.8.53 |
| >> portDS.minorVersionNumber | RW | IEEE Std 802.1AS [104] clause 14.8.54 |
| >> timePropertiesDS.currentUtcOffset | RW | IEEE Std 802.1AS [104] clause 14.5.2 |
| >> externalPortConfigurationPortDS.desiredState | RW | IEEE Std 802.1AS [104] clause 14.12.2 |
| NOTE 1: R = Read only access; RW = Read/Write access. NOTE 2: General neighbor discovery information is included only when NW-TT performs neighbor discovery on behalf of DS-TT. When a parameter in this group is changed, it is necessary to provide the change to every DS-TT and the NW-TT that belongs to the 5GS TSN bridge. NOTE 3: If the Static Filtering Entry information is present for uplink traffic, UPF/NW-TT uses Static Filtering Entry information to determine the NW-TT egress port for forwarding UL TSC traffic. The user plane forwarding is further specified in clause 5.8.2.5.3. NOTE 4: DS-TT discovery configuration and DS-TT discovery information are used only when DS-TT does not support LLDP and NW-TT performs neighbor discovery on behalf of DS-TT. NOTE 5: TSN AF indicates the support for PSFP to the CNC only if each DS-TT and NW-TT of the 5GS bridge have indicated support of PSFP. The support of PSFP at the NW-TT ports is expressed by setting higher than zero values for MaxStreamFilterInstances, MaxStreamGateInstances, MaxFlowMeterInstances, SupportedListMax parameters. NOTE 6: Enumeration of supported PTP instance types. Allowed values as defined in IEEE Std 1588 [126] clause 8.2.1.5.5. NOTE 7: Enumeration of supported transport types. Allowed values: IPv4 (as defined in IEEE Std 1588 [126] Annex C), IPv6 (as defined in IEEE Std 1588 [126] Annex D), Ethernet (as defined in IEEE Std 1588 [126] Annex E). NOTE 8: Enumeration of supported PTP delay mechanisms. Allowed values as defined in IEEE Std 1588 [126] clause 8.2.15.4.4. NOTE 9: Indicates whether NW-TT supports acting as a PTP grandmaster. NOTE 10: Indicates whether NW-TT supports acting as a gPTP grandmaster. NOTE 11: Enumeration of supported PTP profiles, each identified by PTP profile ID, as defined in IEEE Std 1588 [126] clause 20.3.3. NOTE 12: PTP profile to apply, identified by PTP profile ID, as defined in IEEE Std 1588 [126] clause 20.3.3. NOTE 13: Transport type to use. Allowed values: IPv4 (as defined in IEEE Std 1588 [126] Annex C), IPv6 (as defined in IEEE Std 1588 [126] Annex D), Ethernet (as defined in IEEE Std 1588 [126] Annex E). NOTE 14: Indicates whether to act as grandmaster on behalf of a DS-TT port or not if 5GS is determined to be the grandmaster clock, i.e. whether to send Announce, Sync and optionally Follow_Up messages on behalf of DS-TT. NOTE 15: The IEEE Std 802.1AS [104] data sets apply if the IEEE 802.1AS PTP profile is used; otherwise the IEEE Std 1588 [126] data sets apply. |  |  |

Exchange of port and NW-TTmanagement information between TSN AF or NEF and NW-TT or between TSN AF or NEF and DS-TT allows TSN AF or NEF to:

1)	retrieve port management information for a DS-TT or NW-TT port or NW-TT management information;

2)	send port management information for a DS-TT or NW-TT port or NW-TTmanagement information;

3)	subscribe to and receive notifications if specific port management information for a DS-TT or NW-TT port changes or NW-TT management information changes.

Exchange of port management information between TSN AF or NEF and NW-TT or DS-TT is initiated by DS-TT or NW-TT to:

-	notify TSN AF if port management information has changed that TSN AF has subscribed for.

Exchange of NW-TT management information between TSN AF and NW-TT is initiated by NW-TT to:

-	notify TSN AF or NEF if NW-TT management information has changed that TSN AF or NEF has subscribed for.

Exchange of port management information is initiated by DS-TT to:

-	provide port management capabilities, i.e. provide information indicating which standardized and deployment-specific port management information is supported by DS-TT.

TSN AF or NEF indicates inside the Port Management Information Container or NW-TT Management Information Container whether it wants to retrieve or send port or NW-TT management information or intends to (un-)subscribe for notifications.

Next CHANGE

5.28.3.2	Transfer of port or NW-TT management information

Port management information is transferred transparently via 5GS between TSN AF or NEF and DS-TT or NW-TT, respectively, inside a Port Management Information Container (PMIC). NW-TTmanagement information is transferred transparently via 5GS between TSN AF or NEF and NW-TT inside a NW-TT Management Information Container (NMIC). The transfer of port or NW-TT management information is as follows:

-	To convey port management information from DS-TT or NW-TT to TSN AF or NEF:

-	DS-TT provides a PMIC and the DS-TT port MAC address (if available) to the UE, which includes the PMIC as an optional Information Element of an N1 SM container and triggers the UE requested PDU Session Establishment procedure or PDU Session Modification procedure to forward the PMIC to the SMF. SMF forwards the PMIC and the port number of the related DS-TT Ethernet port to TSN AF or NEF as described in TS 23.502 [3] clause 4.3.3.2;

-	NW-TT provides PMIC(s) and/or NMIC to the UPF, which triggers the N4 Session Level Reporting Procedure to forward the PMIC(s) and/or NMIC to SMF. UPF selects an N4 session corresponding to any of the N4 sessions for this NW-TT. SMF in turn forwards the PMIC(s) and the port number(s) of the related NW-TT port(s), or the NMIC, to TSN AF or NEF as described in TS 23.502 [3] clause 4.16.5.1.

NOTE 1:	There has to be at least one established PDU session for DS-TT port before the UPF can report PMIC/NMIC information towards the AF.

-	To convey port management information from TSN AF or NEF to DS-TT:

-	TSN AF or NEF provides a PMIC, MAC address or UE IP address reported for a PDU Session (i.e. MAC address of the DS-TT port or IP address related to the PDU session) and the port number of the Ethernet port to manage to the PCF by using the AF Session level Procedure, which forwards the information to SMF based on the MAC or IP address using the PCF initiated SM Policy Association Modification procedure as described in TS 23.502 [3] clause 4.16.5.2. SMF determines that the port number relates to a DS-TT Ethernet port and based on this forwards the PMIC to DS-TT using the network requested PDU Session Modification procedure as described in TS 23.502 [3] clause 4.3.3.2.

-	To convey port or NW-TT management information from TSN AF or NEF to NW-TT:

-	TSN AF or NEF selects a PCF-AF session corresponding to any of the DS-TT MAC or IP addresses for the related PDU sessions of this bridge and provides a PMIC(s) and the related NW-TT port number(s) and/or NMIC to the PCF. The PCF uses the PCF initiated SM Policy Association Modification procedure to forward the information received from TSN AF or NEF to SMF as described in TS 23.502 [3] clause 4.16.5.2. SMF determines that the included information needs to be delivered to the NW-TT either by determining that the port number(s) relate(s) to a NW-TT Ethernet port(s) or based on the presence of NMIC, and forwards the container(s) and/or related port number(s) to NW-TT using the N4 Session Modification procedure described in TS 23.502 [3] clause 4.4.1.3.

Next CHANGE

Annex K (normative):
Port and NW-TT management information exchange

K.1	Standardized port and NW-TT management information

Editor's note:	Table 5.28.3.1-1 and Table 5.28.3.1-2 will moved here once the tables are stable.

K.2	Port and NW-TT management information exchange for time synchronization

K.2.1	Capability exchange

DS-TT and NW-TT indicate time synchronization information they support inside the Port management capabilities (see Table 5.28.3.1-1).

TSN AF and NEF may determine the PTP functionalities supported by DS-TT and NW-TT by retrieving the following port management information or NW-TT management information, respectively:

-	Supported PTP instance types;

-	Supported transport types;

-	Supported PTP delay mechanisms;

-	Grandmaster capability;

-	Supported PTP profiles;

-	Number of supported PTP instances.

NOTE:	If NW-TT or DS-TT do not indicate support for any of the PTP profiles, then TSN AF or NEF assume that the NW-TT or DS-TT only support acting as a PTP Relay instance with the gPTP GM connected on N6.

If DS-TT and NW-TT support the PTP Relay instance type as defined by IEEE Std 802.1AS [104] then DS-TT and NW-TT shall include the IEEE Std 802.1AS [104] PTP profile in the "Supported PTP profiles" in PMIC and NMIC, respectively.

The TSN AF or NEF may retrieve the "Number of supported PTP instances" from NW-TT via NMIC and from DS-TT via PMIC.

K.2.2	PTP Instance configuration

K.2.2.1	General

Based on input received from external applications (CNC in case of TSN AF or any AF in case of NEF), TSN AF and NEF may configure PTP instances (identified by PTP Instance ID) in a DS-TT or NW-TT by sending PTP port management information (see Table 5.28.3.1-1) to DS-TT and NW-TT, respectively. TSN AF or NEF may also configure PTP instances for DS-TT ports in NW-TT by sending NW-TT Management information (see Table 5.28.3.1-2) to NW-TT to enable NW-TT to operate as a grandmaster on behalf of DS-TT (see clause K.2.2.4 for more details).

For each PTP instance TSN AF or NEF may provide individual PTP configuration parameters or may provide a PTP profile ID to DS-TT or NW-TT.

NOTE:	Even if PTP profiles are used to configure DS-TT or NW-TT, individual PTP parameters can still be configured in addition, e.g. domain numbers, transport to use, etc.

To configure DS-TT and NW-TT to operate as a PTP relay instance, TSN AF or NEF shall set the PTP profile (see Table 5.28.3.1-1) to IEEE Std 802.1AS [104].

To initialize a PTP instance in 5GS, TSN AF or NEF retrieve the "defaultDS.clockIdentity" of the PTP instance in NW-TT via NMIC. Upon detection of a DS-TT port, the TSN AF or NEF initializes the PTP instance in the DS-TT by setting the "defaultDS.clockIdentity" via PMIC to the same value as retrieved from the NW-TT. The TSN AF or NEF also enables the PTP instance by setting the defaultDS.instanceEnable = TRUE to DS-TT via PMIC and to NW-TT via NMIC. The TSN AF or NEF can initialize any number of PTP instances up to the maximum number of supported PTP instances by the NW-TT and DS-TT(s), whichever is the lowest.

Next CHANGE

K.2.2.4	Configuration for PTP grandmaster function

The following options may be supported (per DS-TT) for the 5GS to generate the Sync, Follow_Up and Announce messages for the Master ports on the DS-TT:

a)	NW-TT generates the Sync, Follow_Up and Announce messages on behalf of DS-TT (e.g. if DS-TT does not support this).

b)	DS-TT generates the Sync, Follow_Up and Announce messages in this DS-TT.

TSN AF and NEF may use the elements in port and NW-TT management information container to determine the PTP grandmaster functionality supported by DS-TT and NW-TT and may configure the DS-TT and NW-TT ports to operate as in option a) or b) as follows:

-	The "PTP grandmaster capable" element and the "gPTP grandmaster capable" element in PMIC are used to indicate the support for PTP or gPTP grandmaster capability, respectively, in each DS-TT. If the TSN AF or NEF determines the DS-TT supports grandmaster capability (PTP or gPTP grandmaster capable is TRUE), then either option a) or b) can be used for the PTP instance(s) in the DS-TT. Otherwise, only option a) can be used for the PTP instance(s) in the DS-TT.

-	To enable option a) for PTP ports in DS-TT, the TSN AF or NEF sets the element "Grandmaster on behalf of DS-TT enabled" TRUE (per PTP instance per DS-TT) in NMIC for the respective DS-TT port, and the TSN AF or NEF sets the element "Grandmaster enabled" FALSE (per PTP instance per DS-TT) in PMIC to the respective DS-TT port.

-	To enable option b) for PTP ports in DS-TT, the TSN AF or NEF sets the element "Grandmaster on behalf of DS-TT enabled" FALSE in NMIC (per PTP instance per DS-TT) for the respective port, and the TSN AF or NEF sets the element "Grandmaster enabled" TRUE (per PTP instance per DS-TT) in PMIC to the respective DS-TT port.

-	To enable either option a) or option b) for a PTP instance, the TSN AF or NEF sets the element "Grandmaster candidate enabled" TRUE (per PTP instance) in NMIC.

-	When option b) is used for one or more PTP ports in DS-TT(s), the TSN AF or NEF shall use the elements in defaultDS in PMIC for the respective DS-TT(s) and in NMIC for NW-TT to ensure that all PTP ports in the DS-TT(s) and NW-TT in particular PTP instance are distributing the same values of grandmasterPriority1, grandmasterClockQuality, grandmasterPriority2, grandmasterIdentity, and timeSource message fields in Announce messages.

END OF CHANGES