SA WG2 Meeting #144E (e-meeting)	S2-2102843r02

# Feb 24 – Mar 09, 2021 									                         (revision of S2-2100XXXX)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 2833 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of architecture for AF requested support of Time Sensitive Communication and Time Synchronization |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Vivo, Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA WG2 |  |  |  |  |  |  |  |  |  |
| Work item code: | IIOT |  |  |  |  |  | Date: |  |  | 2021-04-01 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | A lefover EN need to fixed for the architecture for AF requested support of Time Sensitive Communication and/or Architecture for Time Synchronization Editor's note: The architecture for AF requested support of Time Sensitive Communication and/or Time Synchronization is FFS. To support the gPTP/PTP based Time Synchronization service, the DS-TT, NW-TT and NEF are required. When the gPTP/PTP based Time Synchronization service is terminated at the UE, the required DS-TT is collocated with UE. For the other scenario (e.g. non gPTP and/or PTP based Time Synchronization, AF requested support of Time Sensitive Communication without the need of gPTP/PTP), the DS-TT and NW-TT may not present. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify and correct the AF requested support of Time Sensitive Communication and/or Architecture for Time Synchronization. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The architectures for AF requested support of Time Sensitive Communication and/or Architecture for Time Synchronization, is not clear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.4.8.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

#### 4.4.8.3	Architecture for AF requested Time Synchronization

This clause describes the architecture to support AF requested time synchronization.

As shown in Figure 4.4.8.3-1, to support time synchronization , DS-TT, NW-TT and NEF are required in order to support the features in IEEE Std 802.1AS [104] or IEEE Std 1588-2019 [126] as described in clause 5.27. The NEF exposes 5GS capability to support Time Synchronization service as described in clause 5.27.1.8.

Figure 4.4.8.3-1: Architecture for AF requested Time Synchronization

Editor's note:	The architecture for AF requested support of Time Sensitive Communication is FFS.

END OF CHANGES