3GPP TSG-SA2 Meeting # 144E (e-meeting)	S2-2102910r01

# Online, April 12 – 16, 2021										  				   (revision of)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 2723 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Common NSAC service for both Number of UE and Number of PDU Session |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNS_Ph2 |  |  |  |  |  | Date: |  |  | 2021-04-02 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | It is expected that same service and service operation can be used for network slice admission control for both Number of UE and Number of PDU Session |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update the service name and service operations to cover network slice admission control for both Number of UE and Number of PDU Session |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Too many services for network slice admission control |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.11.2.1, 4.2.11.2.2, 4.2.11.2.3, 5.2.21.1, 5.2.21.2.1, 5.2.21.2.2, 5.2.21.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR …CR … |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

****************** START CHANGE ***************

4.2.11	Network Slice Admission Control Function (NSACF) procedures

4.2.11.1	General

The Network Slice Admission Control Function procedures are performed for an S-NSSAI which is subject to Network Slice Admission Control (NSAC) as described in TS 23.501 [2].

Editor's note:	It is FFS how the NSAC works in the roaming case.

4.2.11.2	Network Slice Admission Control for Number of UEs per network slice

4.2.11.2.1	General

4.2.11.2.2	Network Slice Admission Control check for Number of UEs procedure

The number of UEs per network slice availability check and update procedure is to update (i.e. increase or decrease) the number of UEs registered with a S-NSSAI which is subject to NSAC. The AMF is configured with the information indicating which network slice is subject to NSAC.

Figure 4.2.11.2.2-1: NSAC check for Number of UEs per network slice procedure

1.	The AMF triggers the Number of UEs per network slice availability check and update procedure to update the number of UEs registered with a network slice when a network slice subject to NSAC is included or removed from the Allowed NSSAI for a UE. The procedure is triggered in the following cases:

-	At UE Registration procedure, as per clause 4.2.2.2.2:

-	before the Registration Accept in step 21 if the EAC mode is active; or

-	after the Registration Accept message if the EAC mode is not active;

-	At UE Deregistration procedure, as per clause 4.2.2.3, after the Deregistration procedure is completed;

-	At UE Configuration Update procedure (which may result from NSSAA procedure):

-	before the UE Configuration Update message if the EAC mode is active; or

-	after the UE Configuration Update message if the EAC mode is not active;

NOTE:	Depending on the deployment, there may be different NSACF for different S-NSSAI subject to NSAC, and hence, during the registration, AMF triggers the Number of UEs per network slice availability check and update procedure to multiple NSACF.

Editor's note:	If one S-NSSAI the number of the UE/PDU session is managed by several different NSACF, it is FFS how to assure the AMF reaches the same NSACF and how to coordinate the number of UE among the NSACF(s)

2.	If the S-NSSAI is verified that it can be included in the Allowed NSSAI, and the S-NSSAI is subject to NSAC, the AMF sends Nnsacf_NSAC_NumOfUE Request message to the NSACF. The AMF includes in the message the UE ID, the S-NSSAI(s) and the update flag which indicates whether the number of UEs registered with the S-NSSAI(s) is to be increased or the number of UEs registered with the S-NSSAI(s) is to be decreased.

3.	The NSACF updates the current number of UEs registered for the S-NSSAI, i.e. increases or decrease the number of UEs registered per network slice based on the information provided by the AMF in the update flag parameter.

If the update flag parameter indicates increase and the NSACF finds the UE ID is already in the list of UEs registered with the network slice, the number of the UEs registered with the network slice is not increased as the UE has already been counted as registered with the network slice. If the UE ID is not in the list of UE IDs registered with the network slice and the maximum number of UEs registered with the network slice has not been reached yet, the NSACF adds the UE ID in the list of UEs registered with the network slice and increases the current number of the UEs registered with the network slice. If the UE ID is not in the list of UEs registered with that S-NSSAI and the maximum number of UEs per network slice for that S-NSSAI has already been reached, then the NSACF returns a result parameter indicating that the maximum number of UEs registered with the network slice has been reached.

If the update flag parameter from the AMF indicates decrease, the NSACF removes the UE ID from the list of UEs registered with the network slice for each of the S-NSSAI(s) indicated in the request from the AMF and also the NSACF decreases the number of UEs per network slice .

4.	The NSACF returns the Nnsacf_NSAC_NumOfUE_ Response in which the NSACF includes the S-NSSAI(s) for which the maximum number of UEs per network slice has already been reached along with a result parameter indicating that the maximum number of UEs registered with the network slice has been reached.

If for all the S-NSSAI(s) the NSACF returned the maximum number of UEs per network slice has been reached, the AMF rejects the UE request for registration. In the Registration Reject message the AMF includes the rejected S-NSSAI(s) in the rejected NSSAI parameter, a reject cause set to 'maximum number of UEs per network slice reached' for each S-NSSAI and optionally a back-off timer.

Editor's note:	It is FFS whether the AMF shall send the Registration Reject message when for all the S-NSSAI(s) that the UE requested to register for and were subject to NSAC the NSACF returned the maximum number of UEs per network slice has been reached.

Otherwise, the AMF returns Registration Accept message in which the AMF includes the rejected S-NSSAI(s) in the rejected NSSAI list for which the NSACF has indicated that the maximum number of UEs per network slice has been reached, a reject cause set to 'maximum number of UEs per network slice reached' for each rejected S-NSSAI and optionally a back-off timer.

Editor's note:	It is FFS whether and how to restrict the signalling sent from the AMFs to the NSACF in case the maximum number of UEs has been reached for prolonged time.

4.2.11.2.3	Early Admission Control (EAC) for Number of UEs notification procedure

The Early Admission Control (EAC) notification procedure is used by NSACF to indicate the activation or the deactivation of the EAC mode for the S-NSSAI subject to NSAC. For Number of UE, the EAC mode means that the AMF is required to perform the NSAC check procedure for number of UEs before the S-NSSAI subject to NSAC is included in the Allowed NSSAI and sent to the UE.

Editor's note:	It is FFS if an O&M procedure can be used to configure EAC procedure as opposed to the SUBSCRIBE/NOTIFY method.

Figure 4.2.11.2.3-1: Early Admission Control (EAC) notification procedure

1)	The number of UEs registered with a network slice subject to NSAC crosses a certain operator defined threshold. The NSACF determines whether to activate or deactivate the EAC mode.

2)	The NSACF triggers Nnsacf_NSAC_EarlyAdminControlNotification operation including the S-NSSAI, and a EAC flag set to activated if the number of UEs registered with the network slice is above certain threshold or set to deactivated if the number of the UEs registered with the network slice is below certain threshold.

Editor's note:	It is FFS how to trigger the notification without subscription. Also how to determine which AMF is to be notified need be clarified.

3)	The AMF uses the EAC flag to decide when to trigger the NSAC check for number of UEs procedure for this S-NSSAI so that delays to the registration procedure and impact to the already allowed network slices are avoided.

If the EAC flag indicates EAC mode activated, the AMF triggers the NSAC check for number of UEs procedure for this S-NSSAI before the Registration Accept step of the registration procedure or before the UE Configuration Update message as in EAC activated mode the NSACF is to check whether the maximum number of UEs per network slice is reached which may impact the allowed S-NSSAI(s) confirmed by the Registration Accept message and the UE Configuration Update message.

If the EAC flag indicates EAC mode deactivated, the AMF triggers the NSAC check for number of UEs procedure for this S-NSSAIafter Registration Accept step of the registration procedure or after the UE Configuration Update message as in EAC deactivated mode the NSACF does not check whether the maximum number of UEs per network slice is reached, i.e. no impact is expected to the allowed S-NSSAI(s) confirmed by the Registration Accept message and the UE Configuration Update message.

****************** Next Change ***************

5.2.21	Network Slice Admission Control Function (NSACF) services

5.2.21.1	General

The following table illustrates the NSACF services.

Table 5.2.21.1-1: List of NSACF services

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Nnsacf_NSAC | NumOfUE | Request/Response | AMF |
|  | EarlyAdminControlNotification | Notification | AMF |

5.2.21.2	Nnsacf_NSAC services

5.2.21.2.1	General

Service Description: The Nnsacf_NSAC services provides network slice admission control(NSAC) for  for the network slices subject to NSAC. The NSAC include the check of number of UEs and check of number of PDU Sessions in the network slice..

5.2.21.2.2	Nnsacf_NSAC_NumOfUE service operation

Service Operation name: Nnsacf_NSAC_NumOfUE

Description: This service operation enables the AMF to perform network slice admission control for number of UE.

Inputs, Required: S-NSSAI(s), UE ID (SUPI), update flag.

The S-NSSAI(s) input parameter are the network slices subject NSAC.

The UE ID is used by the NSACF to maintain a list of UE IDs registered with the network slice.

The update flag input parameter indicates whether the number of UEs registered with a network slice or number of the PDU session within the network slice is to be increased or decreased:

The NSACF may optionally return the current status of the network slice availability (e.g. a percentage out of the max number of UEs registered with a network slice) in the availability status parameter. This information may be used for NSACF signalling and load balancing in case multiple NSACFs are serving the same network slice.

Editor's note:	It is FFS how to support in case multi NSACF is supported, e.g. discover the same NSACF, coordination of the local maximum number among NSACF.

Inputs, Optional: None

Outputs, Required: Result.

Outputs, Optional: maximum number of UEs per network slice reached, availability status.

The NSACF may optionally return the current status of the network slice availability (e.g. a percentage out of the max number of UEs registered with a network slice) in the availability status parameter. This information may be used for NSACF signalling and load balancing in case multiple NSACFs are serving the same network slice.

5.2.21.2.3	Nnsacf_NSAC_EarlyAdminControlNotification service operation

Service Operation name: Nnsacf_NSAC_EarlyAdminControlNotification

Description: This service operation enables the NSACF to activate or deactivate the EAC mode.

Inputs, Required: S-NSSAI, EAC flag.

The S-NSSAI input parameter is the network slice for which the NSACF activates or deactivates the EAC mode.

The EAC flag input parameter indicates whether the Slice EAC mode is activated or deactivated.

Inputs, Optional: None

Output, Required: None

Output, Optional: None

****************** END CHANGES ***************