# 3GPP TSG-SA WG2 Meeting #145E	S2-2104343

Elbonia, 17 - 28 May, 2021	(Revision of S2-210xxxx)

Title:	[Draft] Reply LS on introducing extended DRX for RedCap UEs

Response to:	LS on introducing extended DRX for RedCap UEs (S2-2103769/R2-2104374)

Release:	Release 17

Work Item:	NR_redcap-Core

Source:	SA2

To:	RAN2, CT1

Cc:	RAN3

Contact Person:

Name:	Aihua Li

E-mail Address:	liaihua at chinamobile dot com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

1. Overall Description:

SA2 thanks RAN2 for the LS on introducing extended DRX for RedCap UEs in S2-2103769 (R2-2104374). SA2 discussed the support of eDRX for RedCap UEs and the following agreements are achieved:

- It is feasible for eDRX for RRC Idle with eDRX cycle up to 10485.76 s, while further evaluation is needed for extending too long eDRX cycle for RRC Inactive.

- The different handling of eDRX cycle up to 10.24s for RedCap (without PTW/PH) and WB-E-UTRAN (with PTW/PH) needs to be considered.

- It is feasible that the 5GC provides the necessary assistance information to RAN for eDRX configuration.

2. Actions:

To RAN2 and CT1:

SA2 kindly asks RAN2 and CT1 take the above information into consideration.

3. Date of Next TSG-SA2 Meetings:

TSG SA WG2 Meeting 146E		16th – 27th August 2021	Electronic meeting