3GPP TSG-SA2 # 145E (e-meeting)	S2-2104876

# May 17-28, 2021                               										(Revision of S2-2104740)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0370 | rev | 1 | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Analytics Data Repository procedures and Historical Data Handling procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell, Spirent, China Mobile |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_Ph2 |  |  |  |  |  | Date: |  |  | 2021-05-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | This CR takes contents agreed in S2-2103286 for procedure for storing historical data of NWDAF, DCCF, MFAF into ADRF. Addiitonal changes are proposed compared to SA2#144E agreed version. Also, it is proposed to introduce a specific level 2 clause for Analytics Data Repository procedures where this procedure for storing historical data of NWDAF, DCCF, MFAF into ADRF would be specified, along with procedure to store historical data and analytics via notifications (see S2-2104746) and procedure to delete data from ADRF (see S2-2104742). |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding general clause for Analytics Data Repository procedures. Enhancing further the description as agreed in S2-2103286. In particular, it is proposed to leave to ADRF implementation whether ADRF checks if data is already stored or being stored. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Procedures for historical data handling not specified. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | (NEW) 6.x, (NEW) 6.x.1, (NEW) 6.x.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 23.288 CR 0371, TS 23.288 CR 0374 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

## 6.x	Analytics Data Repository procedures

### 6.x.1	General

Collected data and analytics may be stored in ADRF, using procedure as specified in clause 6.x.2 and clause 6.x.3. Collected data and analytics may be deleted from ADRF, using procedure as specified in clause 6.x.4.

NEXT CHANGE

##### 6.x.2	Historical Data and Analytics storage

The procedure depicted in figure 6.x.2-1 is used by consumers (e.g. NWDAF, DCCF or MFAF) to store historical data and/or analytics, i.e. data and/or analytics related to past time period that has been obtained by the consumer. After the consumer obtains data and/or analytics, consumer may store historical data and/or analytics in an ADRF. Whether the consumer directly contacts the ADRF or goes via the DCCF or via the Messaging Framework is based on configuration.

Figure 6.x.2-1: Historical Data and Analytics storage

1.	The consumer sends data and/or analytics to the ADRF by invoking the Nadrf_DataManagement_StorageRequest(collected data, analytics) service operation.

2.	The ADRF stores the data and/or analytics sent by the consumer. The ADRF may, based on implementation, determines whether the same data and/or analytics is already stored or being stored based on the information sent in step 1 by the consumer NF and, if the data and/or analytics is already stored or being stored in the ADRF, the ADRF decides to not store again the data and/or analytics sent by the consumer.

3. 	The ADRF sends Nadrf_DataManagement_StorageRequest Response message to the consumer indicating that data and/or analytics is stored, including when the ADRF may have determined at step 2 that data or analytics is already stored.

END OF CHANGES