3GPP TSG-WG SA2 Meeting #145E e-meeting 	S2-2104435r01

Elbonia, May 17 – May 28, 2021	(revision of S2-210xxxx)

Source:	Huawei, HiSilicon

Title:	NF service and service operations for UAS

Document for:	Approval

Agenda Item:	8.7

Work Item / Release:	ID_UAS / Rel-17

Abstract: This contribution introduces NF services relevant to UAS.

# 1. Introduction/Discussion

New NF services are introduced for UAS use.

# 2. Text Proposal

It is proposed to capture the following changes vs. TS 23.256, all new texts.

# * * * * First change * * * *

### 4.4.2	Network Function Services

#### 4.4.2.X	NEF Services

##### 4.4.2.X.1	General

In addition to those defined in TS 23.501[2] clause 7.2.8 and TS 23.502[3] clause 5.2.6, the following table illustrates additional NEF services to support UAS.

Table 4.4.2.X.1-1: NF Services provided by NEF

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Nnef_Authentication | authenticate | Request/Response | AMF, SMF |
|  | revoke | Request/Response | AF |

##### 4.4.2.X.2	Nnef_Authentication service

###### 4.4.2.X.2.1	General

Service Description: This service enables the consumer to authenticate and authorise the Service Level Device Identity. In case of UAS, the service is used to authenticate and authorize the UAV identified by a CAA-Level UAV ID.

###### 4.4.2.X.2.2	Nnef_Authentication_authenticate service operation

Service operation name: Nnef_Authentication_authenticate.

Description: Provides the authentication and authorization result of the Service Level device Identity.

Input, Required: Service Level Device Identity, GPSI.

Input, Optional: Authorization Server Address, PEI, UE IP address (in case the consumer NF is SMF), authentication container provided by UE.

Output, Required: Success/Failure indication, Authorization Data container

Output, Optional: none.

###### 4.4.2.X.2.3	Nnef_Authentication_revoke service operation

Service operation name: Nnef_Authentication_revoke

Description: Revoke the UUAA authorization of a UAV

Input, Required: Service Level Device Identity, 3GPP UAV ID, cause of revocation

Input, Optional: null

Output, Required: acknowledge indication

Output, Optional: none.

# * * * * Second change * * * *

#### 4.4.2.Y	AF Services

##### 4.4.2.Y.1	General

In addition to the AF services defined in TS 23.501[2] clause 7.2.19 and TS 23.502[3] clause 5.2.19, the following table shows the AF services to support UAS.

Table 4.4.2.Y.1-1: NF Services provided by AF

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Naf_Authentication | authenticate | Request/Response | UAS NF/NEF |

##### 4.4.2.Y.2	Naf_Authentication service

###### 4.4.2.Y.2.1	General

Service Description: This service enables the consumer to authenticate and authorize the Service Level Device Identity. In case of UAS, the service is used to authenticate and authorize the UAV identified by a CAA-Level UAV ID.

###### 4.4.2.Y.2.2	Naf_Authentication_authenticate service operation

Service operation name: Naf_Authentication_authenticate

Description: Provides the Authentication and Authorization result of the Service Level Device Identity (i.e. CAA-Level UAV ID for UAS).

Input, Required: Service Level Device Identity, GPSI.

Input, Optional: PEI, UE IP address, authentication container provided by UE.

Output, Required: Success/Failure indication, Authorization Data container

Output, Optional: none.

# * * * * Third change * * * *

#### 4.4.2.a	AMF Services

AMF services related to UAS are defined in TS 23.502 [X] clause 5.2.2.

In addition, when SMF invokes Namf_Communication_N1N2MessageTransfer service operation, it may provide the UUAA result to the UAV.

# * * * * Fourth change * * * *

#### 4.4.2.b	SMF Services

SMF services related to UAS are defined in TS 23.502 [X] clause 5.2.8.

In addition, when AMF invokes Nsmf_PDUSession_CreateSMContext service operation, AMF may provide an indication to indicate “UAV has been authenticated by the USS”, if the UUAA has taken place during registration procedure by the AMF.

Editor’s Note: Whether an indication is needed or if UAS NF or NEF can decide since these entities will maintain the UUAA operation status is FFS.

# * * * * Fifth change * * * *

#### 4.4.2.c	UDM Services

UDM services related to UAS are defined in TS 23.502 [X] clause 5.2.3.

# * * * * Sixth change * * * *

#### 4.4.2.d	LMF Services

LMF services related to UAS are defined in TS 23.273 [Y] clause 8.3.

# * * * * Seventh change * * * *

#### 4.4.2.e	GMLC Services

GMLC services related to UAS are defined in TS 23.273 [Y] clause 8.4.

# * * * * Eighth change * * * *

#### 4.4.2.f	UDR Services

UDR services related to UAS are defined in TS 23.502 [X] clause 5.2.12.

# * * * * Ninth change * * * *

#### 4.4.2.g	PCF Services

PCF services related to UAS are defined in TS 23.502 [X] clause 5.2.5.

# * * * * End of changes * * * *