SA WG2 Meeting #S2-145E	S2-2103792

17 - 28 May, 2021, Electronic meeting

Title:	Reply LS on NAS-based busy indication

Response to:	S2-2103768 / R2-2104354

Release:	Release 17

Work Item:	MUSIM / LTE_NR_MUSIM-Core

Source:	SA2

To:	RAN2, CT1, RAN3

Cc:	SA3

Contact Person:

Name:	Sašo Stojanovski

E-mail Address:	saso stojanovski intel com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	TS 23.502 CR2558

1. Overall Description:

SA2 thanks RAN2 for their LS on NAS-based busy indication for Idle and RRC Inactive state (S2-2103768 / R2-2104354).

SA2 has discussed the RAN2 decision and has made the following working assumption :

-	When UE in RRC Inactive state decides to reject RAN paging, the UE initiates the 5GS NAS Leaving procedure to request connection release.

-	When rejecting RAN paging from RRC Inactive state, it is assumed that some UEs may discard any data or NAS PDUs due to UE’s implementation that it receives during the transition before being released to RRC Idle state.

SA2 would like to bring to RAN2’s attention that several companies in SA2 have expressed concern with the proposed working assumption (e.g. when the DL data or NAS PDUs sent over Uu is dropped by the UE, it is a waste of Uu resources; the NAS based Reject Paging will release the UE to RRC-Idle) and preferred to request RAN2 to reconsider their decision for using NAS-level rejection of RAN paging.

In the meantime, some companies in SA2 have expressed concerns about the feasibility of RRC-based Paging Reject since the AS layer of the UE has no service information in another USIM and AS layer cannot decide to accept or reject the received RAN paging.

Given that SA2 work on MUSIM is due for completion in June 2021, for the sake of progress SA2 agreed to proceed with the agreed TS 23.502 CR2558 including an Editor’s note that a final confirmation from RAN2 is expected to reconfirm the SA2’s working assumption.

Regarding the specific RAN2 questions in the LS:

- Question 1: Are the impacts identified by RAN2 valid?

- Question 2: Are there any other impacts beyond those identified by RAN2?

- Question 3: If the ANS to Q1 and/or to Q2 is yes, can they be specified within Rel-17 timeframe?

SA2 would like to offer the following answers:

-	A1: As indicated above, SA2 has tentatively decided to use the 5GS NAS Leaving procedure when UE in RRC Inactive state rejects RAN paging. Therefore, there are no additional system impacts.

-	A2: SA2 acknowledges that when initiating the 5GS NAS Leaving procedure for the purpose of rejecting RAN paging, the UE may receive some downlink data packets or NAS PDUs before being released to RRC Idle state. As indicated above, SA2 is of the opinion that the UE may discard any such data or NAS PDUs due to UE implementation, which would be waste of Uu resources.

-	A3: Based on the previous answers, SA2 see no issues for completion of the “busy indication” feature in Rel-17 timeframe.

SA2 would also like to bring to RAN2’s attention the SA2’s assumption that at the end of the 5GS NAS Leaving procedure the UE is always put in RRC Idle state.

SA2 assumes that for performing short activity in the other network (e.g. performing mobility update or periodic update or sending Busy indication) the UE would use the RAN2-defined procedure for “switching without leaving RRC Connected state”, noting that this would require the UE to be absent from the current network for 1-2 seconds.

SA2 would like to check with RAN2 whether the RAN2-defined procedure for “switching without leaving RRC Connected state” is compatible with a UE absence duration on the order of 1-2 seconds.

2. Actions:

To RAN2 group.

ACTION: 	SA2 respectfully asks RAN2 to reply whether the RAN2-defined procedure for “switching without leaving RRC Connected state” is compatible with an absence duration on the order of 1-2 seconds.

To RAN2, CT1 and RAN3 groups.

ACTION: 	SA2 respectfully asks CT1 and RAN3 to take the feedback above into consideration for their work on MUSIM.

3. Date of Next TSG-SA WG2 Meetings:

3GPP SA2#146-e			16 - 27 August 2021			Electronic Meeting

3GPPSA2#147-e	18 - 22 October 2021	Electronic Meeting