3GPP TSG-WG SA2 Meeting #145 e-meeting 	S2-2103796r02

17-28 May 2021, Elbonia

Title:	[DRAFT] Reply LS on introducing extended DRX for RedCap UEs

Reply to:	LS on introducing extended DRX for RedCap UEs (S2-2103769/R2-2104374)

Release:	Rel-17

Work Item:	NR_redcap-Core

Source:	SA2

To:	RAN2, RAN3, CT1

Cc:

Contact Person:

Name:	Miguel Griot

Tel. Number:

E-mail Address:	mgriot@qti.qualcomm.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	S2-210xxyy (TS 23.501 CR#2869)

1.	Overall Description:

SA2 would like to thank RAN2 for their LS on introducing extended DRX for RedCap UEs (S2-2103769/R2-2104374).

SA2 would like to provide the following feedback to RAN2:

- 5G System currently supports extended DRX in RRC/CM idle mode in the 5GC/NAS over E-UTRA/NB-IoT (since Rel-16) for eDRX cycles > 10.24s. SA2 has aligned with RAN2 request to support extended DRX in RRC/CM idle mode also for NR for eDRX cycles up to 10485.76 seconds.

- 5G System currently only supports eDXR cycles of up to 10.24s for CM-Connected with RRC inactive state. The extension of eDRX beyond 10.24s for CM-Connected with RRC inactive was studied in Rel-16 in the context of 5G CIoT over E-UTRA/NB-IoT, and impacts to the 5GC were identified. Based on the conclusion form this study, 5G System currently only supports eDXR cycles of up to 10.24s for CM-Connected with RRC inactive state.

- Extending eDRX beyond 10.24s for CM-Connected with RRC inactive is considered technically feasible from SA2 point of view, however, SA2 needs to evaluate whether it is necessary to support the new feature before developing further solutions.

The SA2 agreed CR is attached in this LS.

2.	Actions:

To RAN2, RAN3 and CT1:

ACTION:

SA WG2 kindly requests RAN2, RAN3 and CT1 to take the above information into account.

3.	Date of Next TSG SA WG2 Meetings:

3GPPSA2#146-e	TBD	Electronic Meeting

3GPPSA2#147-e	TBD	Electronic Meeting