3GPP TSG-WG SA2 Meeting #145E e-meeting 	S2-2103816

# Elbonia, May 17th - 28th, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0337 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Mapping UE IP address and GPSI |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_Ph2 |  |  |  |  |  | Date: |  |  | 2021-05-06 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | CR 258 rev 1 to 23.288 was agree to allow the AF or NWDAF to map the SUPI to a UE IP address that is used in the data collection from the UE Application. Given the flexibility above, and not having to place additional complexity to the AF in the untrusted domain, the proposal is to have the same procedure for untrusted AF. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Th AF in the untrusted may map the GPSI to a UE IP address or the NWDAF may do the mapping instead and provide the UE IP address to the AF. Here we refer to approved CR 258 rev.1 clause 6.2.8.2.4.x. for the NWDAF providing the UE IP address to the AF. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete specification of the conclusions in KI#8. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.8.2.4.z (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

---------------------------------First Change---------------------------------------

-

-

---------------------------------Second Change-----------------------------------

6.2.8.2.4.z	AF in untrusted domain correlates UE data collection and NWDAF request

If the AF receives the Naf_EventExposure_Subscribe from NWDAF, via NEF, including Target for Event Reporting set to GPSI, the AF request the NEF to provide the allocated IPv4 address or IPv6 prefix or both as described in Figure 6.2.8.2.4.z-1.  Note that the NWDAF can also provide the UE IP address to the AF as described in 6.2.8.2.4.x.

Figure 6.2.8.2.4.z-1.: AF in untrusted domain correlates UE data collection and NWDAF request

- Same step as step 0 in figure 6.2.8.2.4.z-1.

- The AF receives a request to retrieve input data as described in clause 6.2.8.2.3 including a GPSI. The AF requests NEF to provide the IPv4address or IPv6 prefix or both serving the PDU session for this GPSI towards the AF using Nnef_UEid_Get_Request.

- The NEF is configured with the DNN,S-NSSAI to access this AF, the NEF finds the SMF serving the PDU session(s) for this GPSI, DNN, S-NSSAI using Nudm_UECM_Get_Request including type of requested information set to SMF Registration Info and the S-NSSAI and DNN, as defined in clause 5.3.2.5.7 in TS 29.503 [xx].

NOTE:	that if there are more than one (DNN, S-NSSAI) combination to access this AF, the NEF will find the SMF(s) serving the PDU session(s) to any of these (DNN, S-NSSAI) combinations.

3. The UDM provides the SMF id(s) and the PDU Session id(s), S-NSSAI, DNN using Nudm_UECM_Get_Response to the NEF.

4. The NEF sends Nsmf_EvenExposure_Subscribe to the SMF(s) identified in step 3, including the Target for Event Reporting set to the PDU Session id(s) provided in step 3 and the Event ID set to IP address/prefix allocation/change.

5.	The SMF provides the allocated IPv4 address or IPv6 prefix or both to the NEF.

6.	The NEF provides the allocated IPv4 address or IPv6 prefix or both provided by  SMF in step 5 to the AF.

7.	The AF correlates the UE data that includes the UE IP address and the NWDAF request for the GPSI using the retrieved IPv4 address or IP v6 prefix.

-----------------------------End of Changes---------------------------------------