3GPP TSG-SA WG2 Meeting #145E (e-meeting)	S2-2103870

# May 17 – 28, 2021, Elbonia									   				          (revision of S2-2103309)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 2647 | rev | 3 | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | PMF extensions for sending UE-assistance data to UPF |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, Lenovo, Motorola Mobility, LG Electronics |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | ATSSS_Ph2 |  |  |  |  |  | Date: |  |  | 2021-05-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | It has been concluded in TR 23.700-93 that: For all steering modes that will be considered in the normative phase: a) The network may provide a UE-assistance indication, which indicates that (a) the UE can decide how to distribute the UL traffic based on its internal state (e.g. battery level), and (b) the UE can request from UPF to apply the same distribution for the DL traffic, and the UPF can take the UE's request into account when deciding the DL transmission traffic distribution. b) The UE requests from UPF to apply the same distribution for the DL traffic by using the PMF protocol, if available, or another mechanism, if the PMF protocol is not available. This other mechanism will be determined during the normative phase of the work. Therefore, it shall be possible to support the functionality in bullet b). Changes in rev3: Rev3 is a revision of approved CR from SA#144E. Additional reason for change in rev3: - Rev2 contains an EN on what information the UE provides to UPF to allow UPF to identify the impacted traffic. - Rev2 also contains an EN on how UE can send assistance data to UPF if PMF is not supported. However, so far no other options or protocols have been discussed, so it seems that this EN should be removed. In rel-17 it would be assumed that PMF has to be used for UE assistance. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The PMF protocol is extended so that it can enable the UE to send UE-assistance data to UPF, which specify the UL traffic distribution applied by the UE. Updates in rev3: - Introduction of ATSSS Rule ID that is provided to both UE and UPF. UPF uses it to identify impacted traffic when UE assistance operation is used. - Removal of EN on how UE notifies UPF in case PMF is not supported. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It is not possible for UPF to apply the same distribution for the DL traffic as the distribution for the UL traffic. Remaining Editor’s notes. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.8.2.11.8, 5.32.5.1, 5.32.5.X (new), 5.32.8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

##### 5.8.2.11.8	Multi-Access Rule

The following table describes the Multi-Access Rule (MAR) that includes the association to the two FARs for both 3GPP access and non-3GPP access in the case of supporting ATSSS.

Table 5.8.2.11.8-1: Attributes within Multi-Access Rule

| Attribute |  | Description | Comment |
| --- | --- | --- | --- |
| N4 Session ID |  | Identifies the N4 session associated to this MAR. |  |
| Rule ID |  | Unique identifier to identify this rule. |  |
| Steering functionality |  | Indicates the applicable traffic steering functionality: Values "MPTCP functionality", "ATSSS-LL functionality". |  |
| Steering mode |  | Values "Active-Standby", "Smallest Delay", "Load Balancing" or "Priority-based". |  |
| Steering Mode Indicator |  | Allows autonomous operation if steering mode is set to "Load Balancing". |  |
| ATSSS Rule ID |  | The ATSSS Rule ID used to match an PMF-UAD request as described in clause 5.32.5.X. | Only included in case Steering Mode Indicator indicates UE-assistance operation |
| Per-Access Forwarding | Forwarding Action Rule ID | The Forwarding Action Rule ID identifies a forwarding action that has to be applied. |  |
| Action information | Weight | Identifies the weight for the FAR if steering mode is "Load Balancing" | The weights for all FARs need to sum up to 100 |
| (NOTE 1) (NOTE 2) | Priority | Values "Active or Standby" or "High or Low" for the FAR | "Active or Standby" for "Active-Standby" steering mode and "High or Low" for "Priority-based" steering mode |
|  | List of Usage Reporting Rule ID(s) | Every Usage Reporting Rule ID identifies a measurement action that has to be applied. | This enables the SMF to request separate usage reports for different FARs (i.e. different accesses) |
| NOTE 1: The Per-Access Forwarding Action information is provided per access type (i.e. 3GPP access or Non-3GPP access). NOTE 2: The Weight is treated as the default percentages if the Autonomous operation is allowed for the "Load Balancing" steering mode. |  |  |  |

NEXT CHANGE

#### 5.32.5.1	General principles

When an MA PDU Session is established, the network may provide the UE with Measurement Assistance Information. This information assists the UE in determining which measurements shall be performed over both accesses, as well as whether measurement reports need to be sent to the network.

Measurement Assistance Information shall include the addressing information of a Performance Measurement Function (PMF) in the UPF, the UE can send PMF protocol messages to:

-	For a PDU Session of IP type, Measurement Assistance Information contains one IP address for the PMF, one UDP port associated with 3GPP access and another UDP port associated with non-3GPP access;

-	For a PDU Session of Ethernet type, Measurement Assistance Information contains one MAC address associated with 3GPP access and another MAC address associated with non-3GPP access.

NOTE 1:	To protect the PMF in the UPF (e.g. to block DDOS to the PMF), the IP addresses of the PMF are only accessible from the UE IP address via the N3/N9 interface.

NOTE 2:	After the MA PDU Session is released, the same UE IP address/prefix is not allocated to another UE for MA PDU Session in a short time.

The UE and the UPF may need to perform access performance measurements in order to estimate the Round-Trip Time (RTT) and/or the Packet Loss Rate (PLR) that an SDF is expected to experience when transmitted on a certain access type. Based on these measurements and the provisioned ATSSS rules in the UE and MAR rules in the UPF, the UE and the UPF decide how to distribute the traffic of an SDF across the two accesses.

If the UE and the UPF decide to initiate access performance measurements to estimate the RTT and/or the PLR for an SDF, the access performance measurements shall be performed either

(a)	using the default QoS Flow, or

(b)	using the target QoS Flow, which is the QoS Flow that the SDF traffic is transmitted on.

The UE shall perform access performance measurements based on the target QoS Flow only when this is explicitly indicated in the Measurement Assistance Information. Otherwise, the UE shall perform access performance measurements based on the default QoS Flow. The UPF shall perform access performance measurements based on the target QoS Flow only when this is explicitly indicated by SMF. Otherwise, the UPF shall perform access performance measurements based on the default QoS Flow.

Editor's note:	It is FFS how the Measurement Assistance Information indicates whether access performance measurements shall be performed based on (a) or (b). For example, (b) will be used when the Measurement Assistance Information contains different PMF addresses/ports for different QoS Flows, or when the Measurement Assistance Information contains the same PMF address/port for all QoS Flows and a new indicator.

The UE and the UPF may decide not to initiate access performance measurements using PMF over a certain target QoS Flow, when they already have access performance measurements for another target QoS Flow which they determine can be reused

NOTE 3:	How the UE and UPF determine that the performance measurements using a certain target QoS Flow apply to another target QoS Flow is based on implementation e.g. AN resource to QoS Flow mapping in the UE or getting similar access measurements results with other QoS Flow.

Editor's note:	It is FFS how to send PMF message over the target QoS Flow. For transmitting the PMF messages over the target QoS Flow, different alternatives have been identified, e.g. (a) using different PMF addresses/ports for each QoS Flow, or (b) using the same PMF address/port for all QoS Flows and include QFI in the PMF message header.

Editor's note:	It is FFS how the UE and UPF determines target QoS Flow.

The addressing information of the PMF in the UPF is retrieved by the SMF from the UPF during N4 session establishment.

The following PMF protocol messages can be exchanged between the UE and the PMF:

-	Messages to allow for Round Trip Time (RTT) measurements, i.e. when the "Smallest Delay" or "Load-Balancing" steering mode is used;

-	Messages to allow for Packet Loss Rate (PLR) measurements, i.e. when the "Load Balancing" steering mode is used;

-	Messages for reporting Access availability/unavailability by the UE to the UPF;

-	Messages for sending UE-assistance data to UPF. Such messages may be sent from the UE to UPF only when the UE receives the UE-assistance indicator in an ATSSS rule, as specified in clause 5.32.8. Further details are provided in clause 5.32.5.X.

The PMF protocol is specified in TS 24.193 [109].

The PMF protocol messages used for access availability/unavailability reports shall be sent on the default QoS Flow, i.e. the QoS Flow associated with default QoS rule. The PMF protocol messages used for access performance measurements shall be sent either on the default QoS Flow, or on the target QoS Flow, as specified above.

The QoS Flow associated with default QoS rule for MA PDU Session is Non-GBR QoS Flow.

The UE shall not apply the ATSSS rules and the UPF shall not apply the MAR rules for the PMF protocol messages.

When the UE requests a MA PDU session and indicates it is capable to support the MPTCP functionality with any steering mode and the ATSSS-LL functionality with only the Active-Standby steering mode (as specified in clause 5.32.6.1), the network may send Measurement Assistance Information for the UE to send Access availability/unavailability reports to the UPF. In this case, the UE and UPF shall not perform RTT and PLR measurements using PMF as the UE and UPF can use measurements available at the MPTCP layer.

NEXT CHANGE

#### 5.32.5.X	UE Assistance Operation

When UE-assistance operation is authorized by the PCF in the PCC Rule, the SMF provides an indication for UE-assistance in the ATSSS Rule to the UE, as described in clause 5.32.8, and in the MAR to the UPF, as described in clause 5.8.2.11.8. The SMF also generates and provides a ATSSS Rule ID in the ATSSS Rule together with the UE-assistance indicator and provides the same ATSSS Rule ID in the MAR.

If the UE receives the UE-assistance indicator in an ATSSS rule (as specified in clause 5.32.8) and the UE decides to apply a different UL traffic distribution for an SDF than the default UL traffic distribution indicated in the Steering Mode component of the ATSSS rule (e.g. because the UE is running out of battery), then the following applies.

-	The UE may send a PMF-UAD (UE Assistance Data) message to UPF that contains information specifying the UL traffic distribution applied by the UE for a particular SDF.

-	A PMF-UAD message contains: (a) ATSSS Rule ID that allows the UPF to identify the associated DL SDF, and (b) UL Distribution Info, indentifying how the UL traffic of the SDF is distributed by the UE.

Editor’s note: It is FFS whether the UE, when it decides to apply a different UL distribution, can change to an arbitrary traffic split percentage or whether the UE would change to using a single access for the SDF. It is further FFS whether the UL Distribution Info needs to contain the traffic split percentage if changed, or whether it can contain a preferred access for DL.

-	The UPF may apply the information in a received PMF-UAD message to align the DL traffic distribution for an SDF with the UL traffic distribution applied by the UE for the same SDF. The UPF identifies the impacted MAR by comparing the ATSSS Rule ID received from the UE with the ATSSS Rule ID included in the MARs provided by SMF.

NEXT CHANGE

### 5.32.8	ATSSS Rules

As specified in clause 5.32.3, after the establishment of a MA PDU Session, the UE receives a prioritized list of ATSSS rules from the SMF. The structure of an ATSSS rule is specified in Table 5.32.8-1.

Table 5.32.8-1: Structure of ATSSS Rule

| Information name | Description | Category | SMF permitted to modify in a PDU context | Scope |
| --- | --- | --- | --- | --- |
| Rule ID (NOTE X) | Unique identifier to identify this ATSSS Rule | Optional | No | PDU context |
| Rule Precedence | Determines the order in which the ATSSS rule is evaluated in the UE. | Mandatory (NOTE 1) | Yes | PDU context |
| Traffic Descriptor | This part defines the Traffic descriptor components for the ATSSS rule. | Mandatory (NOTE 2) |  |  |
| Application descriptors | One or more application identities that identify the application(s) generating the traffic (NOTE 3). | Optional | Yes | PDU context |
| IP descriptors (NOTE 4) | One or more 5-tuples that identify the destination of IP traffic. | Optional | Yes | PDU context |
| Non-IP descriptors (NOTE 4) | One or more descriptors that identify the destination of non-IP traffic, i.e. of Ethernet traffic. | Optional | Yes | PDU context |
| Access Selection Descriptor | This part defines the Access Selection Descriptor components for the ATSSS rule. | Mandatory |  |  |
| Steering Mode | Identifies the steering mode that should be applied for the matching traffic and associated parameters. | Mandatory | Yes | PDU context |
| Steering Mode Indicator | Indicates autonomous operation. | Optional | Yes | PDU context |
| Threshold Values | A Maximum RTT and/or a Maximum Packet Loss Rate | Optional | Yes | PDU context |
| Steering Functionality | Identifies whether the MPTCP functionality or the ATSSS-LL functionality should be applied for the matching traffic. | Optional (NOTE 5) | Yes | PDU context |
| NOTE 1: Each ATSSS rule has a different precedence value from the other ATSSS rules. NOTE 2: At least one of the Traffic Descriptor components is present. NOTE 3: An application identity consists of an OSId and an OSAppId. NOTE 4: An ATSSS rule cannot contain both IP descriptors and Non-IP descriptors. NOTE 5: If the UE supports only one Steering Functionality, this component is omitted. NOTE X: ATSSS Rule ID is only included if the Steering Mode Indicator is set to UE-assistance operation. |  |  |  |  |

The UE evaluates the ATSSS rules in priority order.

Each ATSSS rule contains a Traffic Descriptor (containing one or more components described in Table 5.32.8-1) that determines when the rule is applicable. An ATSSS rule is determined to be applicable when every component in the Traffic Descriptor matches the considered service data flow (SDF).

Depending on the type of the MA PDU Session, the Traffic Descriptor may contain the following components (the details of the Traffic Descriptor generation are described in clause 5.32.3):

-	For IPv4, or IPv6, or IPv4v6 type: Application descriptors and/or IP descriptors.

-	For Ethernet type: Application descriptors and/or Non-IP descriptors.

One ATSSS rule with a "match all" Traffic Descriptor may be provided, which matches all SDFs. When provided, it shall have the least Rule Precedence value, so it shall be the last one evaluated by the UE.

NOTE 1:	The format of the "match all" Traffic descriptor of an ATSSS rule is defined in stage-3.

Each ATSSS rule contains an Access Selection Descriptor that contains the following components:

-	A Steering Mode, which determines how the traffic of the matching SDF should be distributed across 3GPP and non-3GPP accesses. The following Steering Modes are supported:

-	Active-Standby: It is used to steer a SDF on one access (the Active access), when this access is available, and to switch the SDF to the available other access (the Standby access), when Active access becomes unavailable. When the Active access becomes available again, the SDF is switched back to this access. If the Standby access is not defined, then the SDF is only allowed on the Active access and cannot be transferred on another access.

-	Smallest Delay: It is used to steer a SDF to the access that is determined to have the smallest Round-Trip Time (RTT). As defined in clause 5.32.5, measurements may be obtained by the UE and UPF to determine the RTT over 3GPP access and over non-3GPP access. In addition, if one access becomes unavailable, all SDF traffic is switched to the other available access. It can only be used for the Non-GBR SDF.

-	Load-Balancing: It is used to split a SDF across both accesses if both accesses are available. It contains the percentage of the SDF traffic that should be sent over 3GPP access and over non-3GPP access. Load-Balancing is only applicable to Non-GBR SDF. In addition, if one access becomes unavailable, all SDF traffic is switched to the other available access, as if the percentage of the SDF traffic transported via the available access was 100%.

-	Priority-based: It is used to steer all the traffic of an SDF to the high priority access, until this access is determined to be congested. In this case, the traffic of the SDF is sent also to the low priority access, i.e. the SDF traffic is split over the two accesses. In addition, when the high priority access becomes unavailable, all SDF traffic is switched to the low priority access. How UE and UPF determine when a congestion occurs on an access is implementation dependent. It can only be used for the Non-GBR SDF.

-	A Steering Mode Indicator, which indicates that the UE may change the default steering parameters provided in the Steering Mode component and may adjust the traffic steering based on its own decisions. Only one of the following Steering Mode Indicators may be provided:

-	Autonomous load-balance indicator: This indicator may be provided only when the Steering Mode is Load-Balancing. When provided, the UE may ignore the percentages in the Steering Mode component (i.e. the default percentages provided by the network) and may autonomously determine its own percentages for traffic splitting, in a way that maximizes the aggregated bandwidth in the uplink direction. The UE is expected to determine its own percentages for traffic splitting by performing measurements across the two accesses. The UPF may apply a similar behaviour when the autonomous load-balance indicator is included in an N4 rule.

Editor's note:	It is FFS how UE assisted operation is specified.

-	Threshold Values: One or more threshold values may be provided only when the Steering Mode is Load-Balancing. A threshold value may be either a Maximum RTT or a Maximum Packet Loss Rate. A threshold value is applicable to both accesses. When at least one measured parameter (i.e. RTT or Packet Loss Rate) on one access exceeds the provided threshold value, the UE and UPF may continue to send traffic on this access but should reduce the traffic on this access by an implementation specific amount.

Editor's note:	Whether also "hard" thresholds (where all traffic is switched to the other access if threshold is not met) will be supported, and for what steering functionality(ies) and threshold type(s) is FFS.

-	A Steering Functionality, which identifies whether the MPTCP functionality or the ATSSS-LL functionality should be used to steer the traffic of the matching SDF. This is used when the UE supports multiple functionalities for ATSSS, as specified in clause 5.32.6 ("Support of Steering Functions").

NOTE 2:	There is no need to update the ATSSS rules when one access becomes unavailable or available.

As an example, the following ATSSS rules could be provided to UE:

a)	"Traffic Descriptor: UDP, DestAddr 1.2.3.4", "Steering Mode: Active-Standby, Active=3GPP, Standby=non-3GPP":

-	This rule means "steer UDP traffic with destination IP address 1.2.3.4 to the active access (3GPP), if available. If the active access is not available, use the standby access (non-3GPP)".

b)	"Traffic Descriptor: TCP, DestPort 8080", "Steering Mode: Smallest Delay":

-	This rule means "steer TCP traffic with destination port 8080 to the access with the smallest delay". The UE needs to measure the RTT over both accesses, in order to determine which access has the smallest delay.

c)	"Traffic Descriptor: Application-1", "Steering Mode: Load-Balancing, 3GPP=20%, non-3GPP=80%", "Steering Functionality: MPTCP":

-	This rule means "send 20% of the traffic of Application-1 to 3GPP access and 80% to non-3GPP access by using the MPTCP functionality".

d)	"Traffic Descriptor: Application-1", "Steering Mode: Load-Balancing, 3GPP=20%, non-3GPP=80%, "Threshold Condition: Packet Loss Rate < 1%", "Steering Functionality: MPTCP":

-	This rule means "send 20% of the traffic of Application-1 to 3GPP access and 80% to non-3GPP access if Packet Loss Rate < 1% on both accesses by using the MPTCP functionality. If the measured Packet Loss Rate of an access is above 1%, then the traffic of Application-1 may be reduced on the given access or may be sent via the other access".

END OF CHANGES