3GPP TSG-SA2 Meeting # 145E (e-meeting)	S2-2103903r01

# Elbonia, May 17 - 28, 2021											 			 	  (revision of S2-210xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 2869 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support for RedCap UE indication and eDRX |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, MediaTek Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_redcap-Core, TEI17 |  |  |  |  |  | Date: |  |  | 2021-05-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | RAN2 LS S2-2103769/R2-2104374 requests the support for RedCap UE for NR. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introducing the support for RedCap UE in NR with indication and eDRX. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Missing functionality for RedCap |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR …CR … |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

****************** START CHANGE ***************

## 5.x	Support for UE with reduced NR capability

### 5.x.1	General

This clause provides an overview for the support of UE with reduced NR capability. UE with reduced NR capability is specified in TS 38.300 [27]. The specific functionalities are described in features of this specification and affected procedures in TS 23.502 [3] and other specifications.

The functionalities supporting UE with reduced NR capabilities are only applicable over NR access.

### 5.x.2	Extended Discontinuous Reception (DRX) for CM-IDLE and CM-CONNECTED with RRC-INACTIVE

#### 5.x.2.1	Overview

The support of extended DRX for UE with reduced NR capability reuses extended DRX support described in clause 5.31.7.2.1 with the following differences:

- The extended DRX is applicable for NR access.

- The gNB broadcasts an indication for the support of extended DRX.

- The supported extended DRX value range applies for CM-IDLE up to 10485.76s as specified in TS 38.304[50].

Editor’s Note: It’s FFS how extended DRX value that is longer than 10.24s for CM-CONNECTED with RRC Inactive is supported and how MT signaling and data are managed .

#### 5.x.2.2	Paging for extended idle mode DRX in NR connected to 5GC

##### 5.x.2.2.1	General

For UE with reduced NR capability connected to 5GC via NR access, the extended idle mode DRX value range consists of values starting from 5.12s (i.e. 5.12s, 10.24s, 20.48s, etc.) up to a maximum of 10485.76s (i.e. almost 3 hours) (see TS 38.304 [50]). The extended idle mode DRX cycle length is negotiated via NAS signalling. The AMF includes the extended idle mode DRX cycle length for NR in paging message to assist the NG-RAN node in paging the UE.

For extended idle mode DRX cycle length of 5.12s or 10.24s, the network follows the regular DRX framework as defined in clause 5.4.5. PTW and Paging Hyperframe are not used.

For extended idle mode DRX cycle length that is longer than 10.24s, clauses 5.31.7.2.2.1, 5.31.7.2.2.2 and 5.31.7.2.2.3 apply with the following differences:

- TS 36.304 [52] is replaced by TS 38.304[50].

#### 5.x.2.3	Paging for extended DRX for RRC Inactive in NR connected to 5GC

For UE with reduced NR capability connected to 5GC via NR access, the extended DRX value for RRC Inactive range consists of values starting from 5.12s (i.e. 5.12s, 10.24s) (see TS 38.304 [50]). The extended DRX cycle length is configured via AS signalling as specified in TS 38.331[x] considering the extended idle mode DRX value provided by AMF in "RRC Inactive Assistance Information" as defined in clause 5.3.3.2.5.

For extended DRX cycle length of 5.12s or 10.24s, the gNB follows the regular DRX framework as defined in TS 38.304 [50].

Editor’s Note: It’s FFS how extended DRX value that is longer than 10.24s for CM-CONNECTED with RRC Inactive is supported and how MT signaling and data are managed .

### 5.x.3	Differentiation of UE with reduced NR capability

This functionality is used by the network to identify traffic to/from UE with reduced NR capability, e.g. for charging differentiation.

A UE with reduced NR capability using NR access provides a RedCap Indication to the NG-RAN during RRC Connection Establishment procedure as defined in TS 38.331 [51].

When the UE has provided a RedCap indication to the NG-RAN during RRC Connection Establishment, the NG-RAN shall provide a RedCap Indication to the AMF in the Initial UE Message (see TS 23.502 [3] clause 4.2.2.2.1 and TS 38.413 [34]).

When the AMF receives a RedCap Indication from NG-RAN in an Initial UE Message, the AMF shall store the RedCap Indication in the UE context, and signal it accordingly to the SMSF during registration procedure for SMS over NAS, to the SMF during PDU Session Establishment or PDU Session Modification procedure. The PCF will also receive the RedCap Indication, when applicable, from the SMF during SM Policy Association Establishment or SM Policy Association Modification procedure.

The NFs interactions with CHF shall include the RedCap Indication.

Upon AMF change, the source AMF shall provide the RedCap Indication to the target AMF as part of the UE context.

****************** END CHANGES ***************