3GPP TSG-SA2 # 144E (e-meeting)	S2-2104077r01

# April 12 – April 16, 2021                                  (Revision of S2-2103286)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0314 | rev | 1 | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Historical Data Handling |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Spirent Communications |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_Ph2 |  |  |  |  |  | Date: |  |  | 2021-4-5 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | There is no procedures for storing historical data of NWDAF, DCCF, MFAF into ADRF. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addition of procedures to store historical data of NWDAF, DCCF, MFAF into ADRF. If there is no matching data in ADRF, it will store the historical data, if there is matching data in ADRF, it will refuse to store the historical data. Spirent #145E: Step 2 in Figure 6.2.6.3.7-1 indicates consumer is requesting information while the procedure is dealing with data provided for storage and not information requested. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Procedures for historical data handling not specified correctly |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.6.3.x(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Changes * * *

##### 6.2.6.3.x	Historical Data and Analytics storage

The procedure depicted in figure 6.2.6.3.x-1 is used by consumers (e.g. NWDAF, DCCF or MFAF) to store historical data or analytics, i.e. data or analytics related to past time period that has been obtained by the consumer. After the consumer obtains data or analytics, consumer may store historical data or analytics in an ADRF. Whether the consumer directly contacts the ADRF or goes via the DCCF or via the Messaging Framework is based on configuration.

Figure 6.2.6.3.x-1: Historical Data and Analytics storage

1.	The consumer sends data or analytics to the ADRF by invoking the Nadrf_DataManagement_StorageRequest(collected data, analytics) service operation.

2.	ADRF determines whether the same data or analytics is already stored or being stored based on the information sent in step 1 by the consumer NF. If the data or analytics are not already stored in the ADRF, the ADRF stores the data or analytics sent by the consumer.

3. The ADRF sends Nadrf_DataManagement_StorageRequest Response message to the consumer indicating the result of the service operation. If the data or analytics are already stored or being stored in the ADRF, the ADRF sends a StorageRequestan Accept response message to the consumer NF.

Editor's note:	Additional step for ADRF to subscribe to data towards NF is FFS.

* * *End of Changes * * *