3GPP TSG-SA2 Meeting #145E	S2-2104119

# May 17th - May 28th , 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 2896 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Exposure of Time synchronization as a service - description |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shangai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | IIoT |  |  |  |  |  | Date: |  |  | 2021-04-12 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | SA2 FS_IIoT study has concluded the support for exposure of Time synchronization service and it need to be captured in TS 23.501. This CR updates the description of time synchronization service exposure as agreed in TR 23.700-20 and addresses the time synchronization open issues. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Solve open ENs in clause 5.27.1.8 and progress description of time synchronization service within 23.501. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete specifications |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.27.1.8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## ---Start of the 1st Change---

#### 5.27.1.8	Exposure of Time Synchronization

5G System supports time synchronization service that can be activated and deactivated by AF. Exposure of time synchronization comprises the following capabilities:

-	The AF may learn 5GS capabilities to support time synchronization.

-	The AF may request time synchronization service with specific requirements and supply information that can be used to configure time synchronization distribution for targeted UE(s). The AF controls activation and deactivation of the time synchronization service for the target UE(s).

The NEF exposes the 5GS capabilities to support time synchronization service to the AF. The exposed information may include the supported time synchronization distribution methods (i.e. supported IEEE Std 1588 [126] or IEEE Std 802.1AS [104] operation modes or 5G clock sync), (g)PTP grandmaster capable, 5G Clock quality.

The AF requests sent to the NEF may target a UE or multiple UEs. The request may include the following information:

-	Target UE(s) Identification. This may correspond to:

-	Individual UEs identified using GPSI, or an IP address/Prefix or a MAC address. Any UE accessing the combination of DNN, S-NSSAI and DNAI(s).

-	Requested time synchronization distribution method:

-	IEEE Std 1588 [126] or IEEE Std 802.1AS [104] operation (i.e. as a Boundary Clock, peer-to-peer Transparent Clock, or end-to-end Transparent Clock or as a PTP relay instance).

-	5G clock sync. In this case, the time source is provided by the 5GS. 5G-AN provides a 5GS reference time to the UE via 3GPP radio access; UE/DS-TT may provide 5G reference timing information to end stations using implementation specific means. This method is applicable for both IP and ETH PDU Sessions.

-	For IEEE Std 1588 [126] or IEEE Std 802.1AS [104] operation, information to be used by NEF to configure IEEE Std 1588 or IEEE Std 802.1AS functionality in DS-TT or NW-TT, if applicable.

-	Spatial Validity Condition on the UE(s) location as described in clause 5.6.7.1.

Editor's note:	The clock accuracy attribute for time synchronization configuration, NEF exposure of 5G time domain accuracy, AF input regarding requested accuracy and use of time sync accuracy within 5G System is FFS.

The NEF uses the Time Synchronization parameters received from AF. When IEEE Std 1588 [126] or IEEE Std 802.1AS [104] operation have been selected, the NEF determines the necessary (g)PTP parameters to activate and control the service in DS-TT(s) and NW-TTs. For this purpose, the NEF uses the PMIC or BMIC to manage the IEEE Std 1588 [126] or IEEE Std 802.1AS [104] operation in the DS-TT or NW-TT, respectively (see clause 5.27.1.4).

Editor's note:	FFS the time synchronization information provided by the AF that may be stored in the UDR and how the NEF may update or revoke information at the UDR for existing or future PDU Sessions.

For handling (g)PTP traffic, the PCF, according to PCC rule authorization, chooses a 5QI and dynamically set the PDB and/or MDBV according to requirements for (g)PTP protocol. The PCF provides the SMF with a PCC rule generated based on the AF request. The SMF may take the information in the PCC rule to may decide to modify or establish a PDU Session to create or modify or release a QoS flow for transmitting the (g)PTP messages. The PCF acknowledges the time synchronization service request to the NEF and the NEF replies to the AF with the result.

## ---End of Changes---