3GPP TSG-SA2 Meeting #145E	S2-2104122

# May 17th - May 28th , 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 2897 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Time synchronization accuracy - description |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shangai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | IIoT |  |  |  |  |  | Date: |  |  | 2021-05-06 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | SA2 FS_IIoT study has concluded the support for exposure of Time synchronization service and it need to be captured in TS 23.501. This CR progresses the description of accuracy requirement for time synchronization service configuration within 5GS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Solve open ENs in clause 5.27.1.8 about time synchronization accuracy. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete specifications |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.27.1.8.1 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## ---Start of the 1st Change---

## ---Start of the 2nd Change---

#### 5.27.1.8.1	Time Synchronization Assistance Information

Time synchronization assistance information describes time synchronization service characteristics for the distribution of 5G internal system clock. Time synchronization assistance information may be provided by AMF to the NG-RAN to assist the distribution of 5G internal system clock to the targeted UE(s) as described in TS 38.331 [28]. The knowledge of the time synchronization service characteristics is useful for NG-RAN to allow it to determine how to provide ReferenceTimeInfo IE (as described in TS 38.331 [28]), propagation delay compensation, time information transfer during mobility events to the targeted UE(s).

Time synchronization assistance information may include the attributes listed in Table 5.27.1.8.1-1 on a UE basis. UE basis means the attribute applies to the distribution of the 5G internal system clock that may map to one or multiple time synchronization processes configured at the UE(s) (e.g. multiple (g)PTP time domains).

Table 5.27.1.8.1-1: Time synchronization assistance information

| Assistance Information | Description |
| --- | --- |
| Time synchronization accuracy (optional, UE basis) | It refers to an upper bound for the E2E synchronicity accuracy budget supported by the 5G clock sync time distribution process. It may be provided by the AF when configuring the time synchronization error, if not provided, a value based on 5GS constrains is selected. |
| Uu synchronization error budget (optional, UE basis) | It refers to the Uu interface synchronicity accuracy budget for the 5G clock sync. |

If the AF provides a synchronization accuracy requirement, the value is used to determine the Uu synchronization error budget. Otherwise, a value based on 5GS constraints is used. For a given synchronization accuracy requirement, the NEF provides the accuracy budget to the AMF and to the targeted DS-TT(s) via PMIC. The end to end synchronicity accuracy budget can be divided into three parts: Network, Uu interface, and device. The device part of the time synchronization accuracy budget can be assumed within a preconfigured range in the 5GS. The network part depends on the time synchronization topology for the 5G clock to the serving gNB (i.e. where the GM clock is placed). AMF subtracts an inaccuracy value for the network and device part of the end to end time distribution chain from the synchronization accuracy budget to determine the remaining synchronization budget for Uu interface. This remaining synchronization accuracy budget is provided to the NG-RAN via AMF. If there is no network part in the time distribution chain, the received accuracy budget can be sent to the AMF without the correction. If there are multiple AFs providing synchronization accuracy requirement for a given UE, then the AMF picks the most stringent time synchronization accuracy information to be part of time synchronization assistance information.

## ---End of Changes---