SA WG2 Meeting #145E e-meeting	S2-2104202r07

May 17-28, 2021, Elbonia 	()

Source:	vivo

Title:	TS 23.304: Clause 5.9.3.3 EN remove

Document for:	Approval

Agenda Item:	8.8

Work Item / Release:	5G_ProSe / Rel-17

Abstract: This contribution is to remove ENs in clause 5.9.3.3 Identifiers for Layer-2 UE-to-Network Relay.

# 1.	Introduction

This paper focuses on removing the following ENs in 5.9.3.3 Identifiers for Layer-2 UE-to-Network Relay.

Editor's note:	It is FFS what identifiers are used in addition to the identifiers specified in 5.9.3.1 for Layer-2 UE-to-Network Relay use case.

Editor's note:	It is FFS with what information the RSC is associated for Layer-2 UE-to-Network Relay use case.

For the first EN, it is proposed to and the following identifier in the L2 relay discovery message:

-	Relay TAI: indicates the Tracking Area Identity corresponding to the serving cell of the Layer-2 UE-to-Network Relay.

For the second EN, Considering the PDU session for L2 relay belongs to remote UE, remote UE establishes the PDU session with network based on its own URSP, all the PDU session parameters are transparent for relay UE. Remote UE only needs to know the relay UE has the L2 relay capability and no need to know what PDU session parameters that the relay UE supports.

As for how the remote UE know whether the relay UE support L2 relay, since the RSC applied to L3 relay

is associated with a set of PDU session parameters (e.g. PDU Session type, DNN, SSC Mode, S-NSSAI, Access Type Preference) but the one of L2 relay has not. Thus the remote UE can, based on whether there is this association, be aware of the RSC whether is for L2 relay or L3 relay.

Thus there is no need to associate with additional info/parameters for RSC applied to L2 relay, the second EN can be removed by adding the following EN to explanation:

NOTE:	The Relay Service Code applied to L2 relay has no associated parameters.

# 2.	Proposal

It is proposed to capture the following changes into TS 23.304.

# * * * * First change * * * *

#### 5.9.3.1	Common identifiers for UE-to-Network Relay

The following parameters are used in the UE-to-Network Relay Discovery Announcement message (Model A):

-	ProSe Relay UE ID: link layer identifier that is used for direct communication and is associated with a Relay Service Code. A UE-to-Network Relay shall have a distinct ProSe Relay UE ID for each Relay Service Code.

Editor's Note:	the use of Relay UE ID needs to be revisited whether it should be made in line with V2X design approach (i.e. using source and destination layer-2 IDs).

-	Announcer Info: provides information about the announcing user.

-	Relay Service Code: parameter identifying a connectivity service the ProSe UE-to-Network Relay provides to a Remote UE. The Relay Service Codes are configured in a ProSe UE-to-Network Relay for advertisement. Additionally, the Relay Service Code may also identifies authorized users the ProSe UE-to-Network Relay would offer service to, and may be used to select the related security policies or information e.g. necessary for authentication and authorization between the Remote UE and the ProSe UE-to-Network Relay (e.g. a Relay Service Code for relays for police members only would be different than a Relay Service Code for relays for Fire Fighters only, even though potentially they provided connectivity to same DN e.g. to support Internet Access).

-	Relay TAI: indicates the Tracking Area Identity corresponding to the serving cell of the UE-to-Network Relay.

Editor’s Note: Other identifiers required to support UE configured for high priority access is FFS.

The following parameters are used in the UE-to-Network Relay Discovery Solicitation message (Model B):

-	Discoverer Info: provides information about the discoverer user.

-	Relay Service Code: information about connectivity that the discoverer UE is interested in. The Relay Service Codes are configured in the Remote UEs interested in related connectivity services.

-	ProSe Relay UE ID: link layer identifier of a UE-to-Network Relay that is used for direct communication and is associated with a Relay Service Code. A UE-to-Network Relay shall have a distinct ProSe Relay UE ID for each Relay Service Code. The ProSe Relay UE ID is optional.

Editor's Note:	the use of Relay UE ID needs to be revisited whether it should be made in line with V2X design approach (i.e. using source and destination layer-2 IDs).

The following parameters are used in the UE-to-Network Relay Discovery Response message (Model B):

-	ProSe Relay UE ID: link layer identifier that is used for direct communication and is associated with a Relay Service Code. A UE-to-Network Relay shall have a distinct ProSe Relay UE ID for each Relay Service Code.

Editor's Note:	the use of Relay UE ID needs to be revisited whether it should be made in line with V2X design approach (i.e. using source and destination layer-2 IDs).

-	Relay Service Code: identifies the connectivity service the ProSe UE-to-Network Relay provides to Remote UEs that matches the Relay Service Code from the corresponding Discovery Solicitation message.

-	Discoveree Info: provides information about the discoveree.

Editor's note:	Whether User Info ID (i.e. Announcer Info, Discoverer Info, Discoveree Info) can be used for both public safety and commercial purpose is FFS.

-	Relay TAI: indicates the Tracking Area Identity corresponding to the serving cell of the UE-to-Network Relay.

Editor’s Note: Other identifiers required to support UE configured for high priority access is FFS.

# * * * * Next change * * * *

#### 5.9.3.2	Identifiers for Layer-3 UE-to-Network Relay

For Layer-3 UE-to-Network relay, a Relay Service Code in the Announcement Message is associated with a set of PDU session parameters (e.g. PDU Session type, DNN, SSC Mode, S-NSSAI, Access Type Preference). The Relay Service Code may also represent if the relay UE can provide secure N3IWF connection.

For remote UE discovering Layer-3 UE-to-Network relay, the Relay Service Code in the Solicitation Message represents the PDU session parameters that a PDU session of the relay should be able to support. The Relay Service Code may also represent if the remote UE requires secure N3IWF connection.

# * * * * Next change * * * *

#### 5.9.3.3	Identifiers for Layer-2 UE-to-Network Relay

Editor's note:	It is FFS what identifiers are used in addition to the identifiers specified in 5.9.3.1 for Layer-2 UE-to-Network Relay use case.

# * * * * End of changes * * * *