3GPP TSG-WG SA2 Meeting #145E e-meeting	S2-2104218r072

Elbonia, May 17 – 28, 2021	(revision of S2-2103075)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 2815 | rev | 2 | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | KI#1 – T5, Enable mobility between networks |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | [ZTE, OPPO,] Huawei, HiSilicon, Nokia, Nokia Shanghai Bell[, Ericsson] |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA WG2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNPN |  |  |  |  |  | Date: |  |  | 2021-05-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | According to the conclusion of KI#1 of FS_eNPN, SNPN mobility can be supported from the source network to target network. - In the case that there are common AMF and/or N14 interface between the source network and target network, mechanism defined in TS 23.502 [6] clause 4.9.1 is re-used to address UE mobility. - In the case of idle mode mobility, the UE performs initial or mobility registration as specified in clause 4.2.2.2.2 of TS 23.502 [6]. This needs to modify the general registration procedure to implement this. Besides that, it needs to clarify how to recover the previously established PDU Session, for example, a PDU Session is established in the source PLMN or source SNPN, when the UE moves to the target SNPN network, UE may attempt to recover the previously established PDU Sessions in the following manner: - set the List Of PDU Sessions To Be Activated in registration request during mobility registration update procedure within the target network. In case that the target SNPN supports N14 interface with source SNPN and the target AMF can connect to the SMF serving the PDU Session, this may succeed. - initiate the Handover of a PDU Session between 3GPP and untrusted non-3GPP access procedure that follows a PDU Session Establishment procedure at the target network. In case that the target SNPN and source PLMN supports the Underlay/Overlay architecture as described in Annex D.3, then this may succeed. - initiate PDU Session Establishment procedure with Initial Request indication and same DNN, S-NSSAI as the previous PDU Session. In case that the PDU Session is not re-established in the above methods, then the UE can establish a new PDU session with the same DNN, S-NSSAI as the previsous PDU Session in order to access to the same serivces over the target PLMN. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding a new subclause on the mobility between SNPNs, and between SNPN and PLMN. Clarify how to recover the previously established PDU Sessions when UE moves to/from a SNPN. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The mobility between SNPNs, and between SNPN and PLMN is not supported. It is not clear how to recover the previously established PDU Sessions during UE mobility from/to SNPN |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.30.2.X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | The new clause is the same as added by CR2751 with this CR to be implemented first. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

5.30.2.X	UE Mobility support for SNPN

If the UE moves its 3GPP access between SNPN and PLMN the UE performs initial registration as specified in TS 23.502 [3] clause 4.2.2.2.2.

If the UE moves its 3GPP access between SNPNs, then the UE performs initial or mobility registration as specified in TS 23.502 [3] clause 4.2.2.2.2.

NOTE:	When the UE moves its 3GPP access between SNPNs, it is up to UE implementation whether and when to establish again the previously established PDU Sessions using existing mechanism.

END OF CHANGES