3GPP TSG-WG SA2 Meeting #145E e-meeting	S2-2104266r01

Elbonia, May 17 – 28, 2021	(revision of S2-200xxxx)

Source:	Huawei, HiSilicon

Title:	Identifiers for Layer-2 UE-to-Network Relay

Document for:	Approval

Agenda Item:	8.8

Work Item / Release:	5G_ProSe / Rel-17

Abstract: This contribution proposes to add identifiers for Layer-2 UE-to-Network Relay.

# 1. Introduction/Discussion

It is agreed in last meeting that the RSC can indicate if the UE-to-Network Relay is a Layer-3 or Layer-2 UE-to-Network Relay. For Layer-3 UE-to-Network Relay, RSC contained in the discovery message is associated with PDU session parameters. For Layer-2 UE-to-Network Relay, there is no need to use RSC to differentiate the services and a Remote UE can get the PLMN ID of the UE-to-Network Relay during the Relay discovery process based on clause 6.7.2.3 of TR 23.752 and RAN2 agreement, so there is also no need to associate different RSCs with different PLMNs.

Proposal 1: No information needs to be associated with the RSC for Layer-2 UE-to-Network Relay, and the RSC only indicates the Layer-2 UE-to-Network Relay service.

# 2. Text Proposal

It is proposed to capture the following changes vs. TS 23.304.

# * * * * First change * * * *

#### 5.9.3.3	Identifiers for Layer-2 UE-to-Network Relay

Editor’s note:	Any identifiers that are used in addition to the identifiers specified in 5.9.3.1 for Layer-2 UE-to-Network Relay requires cooperation with RAN2.

# * * * * End of changes * * * *