SA WG2 Meeting #S2-145E	S2-2104275r10

May 17 - 28, 2021, Electronic, Elbonia	(revision of S2-2102445)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0281 | rev | 01 | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Analytics Subscription Transfer when analytics consumer changes |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_Ph2 |  |  |  |  |  | Date: |  |  | 2021-05-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | 1. There are Editor's notes in procedures for analytics subscription transfer as follows: Editor's note: It is preferably to have one procedure to cover the re-selection of NWDAF covering both when consumer change and when not. But it is FFS how. Editor's note: It FFS how this aligns with the procedure when Analytics consumer changes. Resolvement of these ENs are in CR0339, but this contribution introduces a way towards first EN. When analytics consumer changes, the new analytics consumer may select a new NWDAF for the active analytics subscription(s) inherited from the old analytics consumer. Instead of notifying the source NWDAF of the target NWDAF ID, it is more sound that the new analytics consumer notifies the the target NWDAF of the source NWDAF ID when requesting analytics subscription to the target NWDAF, so that the Target NWDAF may request analytics subscription transfer from the source NWDAF (e.g. by requesting analytics context information transfer). This alternative also applies to the case of analytics consumer does not change, i.e. analytics consumer may decide to select a new NWDAF for an existing analytics subscription with another NWDAF instance, based on various reasons (e.g. change of NWDAF profile). 2. As specified currently, the analytics subscription transfer request optionally includes "analytics context identifier(s)" to indicate the availability of analytics context for particular Analytic ID(s), and the target NWDAF can then use the "analytics context identifier(s)" to request analytics context information from the source NWDAF. However, there are cases that the target NWDAF may request the analytics context for all the Analytics ID(s) (-then indicating all the analytics context identifiers may become cumbersome) or even without "analytics context identifier(s)" provided from the source NWDAF (e.g. if the analytics subscription transfer request is received from the analytics consumer, see 1). Therefore, it is proposed that the Subscription Correlation ID, which is provided in the analytics subscription transfer request to the target NWDAF, can also be used by the target NWDAF to request analytics context information from the source NWDAF; so that the source NWDAF can respond with the available analytics context information associated with the Subscription Correlation ID. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. Add the procedure of analytics subscription transfer initiated by target NWDAF which is selected by analytics consumer. 2. Remove the ENs on NWDAF re-selection when analytics consumer changes. 3. Add new parameters (e.g. source NWDAF ID) for analytics subscription transfer to Nnwdaf_AnalyticsSubscription_Subscribe service operation. 4. Add the descriptions that the target NWDAF can use Subscription Correlation ID to request analytics context information from the source NWDAF. 5. Add new parameter (Subscription Correlation ID) to Nnwdaf_AnalyticsInfo_Transfer service operation for analytics context information request. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It is unclear how analytics subscription transfer is performed when analytics consumer changes. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1B.1, 6.1B.2, 6.1B.2.1(new), 6.1B.2.x(new), 6.1B.4, 7.2.2, 7.3.3 |  |  |  |  |  |  |  |  |
| “ |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

******************** 1st Change ********************

******************** 2nd Change ********************

6.1B.2.x	Analytics Subscription Transfer initiated by target NWDAF which is selected by analytics consumer

The procedure in Figure 6.1B.2.x-1 is used by an NWDAF instance to request the transfer of analytics subscription(s) from another NWDAF instance, triggered by an Nnwdaf_AnalyticsSubscription_Subscribe service operation which is invoked by an analytics consumer and contains information of analytics subscription transfer as defined in clause 7.2.2.

In this procedure, the analytics consumer decides to select a new NWDAF instance for an existing analytics subscription with another NWDAF instance, based on various reasons (e.g. change of NWDAF profile or UE context transfer procedures, see TS 23.502 [3]).

Figure 6.1B.2.x-1: Analytics subscription transfer initiated by target NWDAF

0.	The analytics consumer is subscribed to analytics information from the source NWDAF or receives information about active analytics subscription(s) from another analytics consumer, e.g. during AMF UE context transfer procedure.

1.	The analytics consumer determines, e.g. triggered by UE mobility or by receiving a subscription status as "not able to provide Analytics" in an Nnwdaf_AnalyticsSubscription_Notify service operation from source NWDAF, to select a new NWDAF instance for the ongoing analytics subscription(s). The analytics consumer selects the target NWDAF to transfer the analytics subscription to, as specified in clause 5.2.

2.	The analytics consumer requests, using Nnwdaf_AnalyticsSubscription_Subscribe request service operation, a transfer of the analytics subscription(s) determined in step 1 to the target NWDAF. The request contains information on the analytics subscription(s) to be transferred, e.g. Analytics ID(s),  Analytics specific parameters, Source NWDAF ID and Subscription Correlation ID used with the source NWDAF.

3.	Target NWDAF accepts the subscription request and allocates a new Subscription Correlation ID to the received analytics subscription.

4.	Target NWDAF requests from the source NWDAF an analytics context transfer for the Subscription Correlation ID provided in step 2, using the analytics context information transfer procedure as specified in clause 6.1B.3.

5.	Source NWDAF responds to the target NWDAF with the requested analytics context information. And based on the analytics context transfer request, the Source NWDAF decides that the analytics subscription is transferred to the target NWDAF.

6.	Source NWDAF unsubscribes from the data source(s) that are no longer needed for the remaining analytics subscriptions.

7.	Target NWDAF subscribes to relevant data source(s), if it is not yet subscribed to the data source(s) for the data required for the Analytics.

8.	Target NWDAF starts to generate analytics information taking into account the analytics context information received from the source NWDAF.

9.	[Optional] Target NWDAF notifies the analytics consumer about the successful analytics subscription transfer using a Nnwdaf_AnalyticsSubscription_Notify message.

10.	Target NWDAF receives new input data from the data source(s) and derives new analytics for the analytics subscription.

11.	Target NWDAF notifies the analytics consumer about the new analytics using a Nnwdaf_AnalyticsSubscription_Notify message as specified in clause 6.1.1.

******************** 3rd Change ********************

### 6.1B.4	Contents of Analytics Context Information

The consumers of the Nnwdaf_AnalyticsInfo_Transfer service operation (as specified in clause 7.3.3) provide the following input parameters listed below:

-	[Optional] Subscription Correlation ID: identifies the analytics subscription to transfer and for which the related analytic context information is requested;

-	[Optional] A list of analytics context identifier(s): identify a set of analytics context information that are available at the NWDAF instance providing this service and that are requested to be transferred to the consumer NWDAF instance.

NOTE:	A list of "analytics context identifier(s)" is received during an analytics subscriptions transfer request as described in clause 6.1B.2.

Editor's note:	It is FFS whether an enhanced Analytics request is to be used instead of transfer service for Analytics. And it is FFS whether bulk data is to be used for Data instead of transfer service.

The NWDAF provides to the consumer of the Nnwdaf_AnalyticsInfo_Transfer service operation (as specified in clause 7.3.3), the output information listed below:

-	(Set of) Analytics context information for each of the "analytics context identifier(s)" provided as input parameter. "Analytics context information" includes the following information parts:

-	Analytics related:

-	[OPTIONAL] Pending output analytics (i.e. not yet notified to the consumer) or historical output analytics information, if available at the source NWDAF. The content of the output analytics is specified in clause 6.1.3 as output information of the Nnwdaf_AnalyticsSubscription_Notify or Nnwdaf_AnalyticsInfo_Request service operations.

-	Timestamp(s) of the last output analytics provided to the analytics consumer(s). Value is set to 0 if no output analytics had been sent yet.

-	Data collection related:

-	[OPTIONAL] Historical data that is available at the source NWDAF and that is related to the Analytics to be handed over to the target NWDAF. This includes not only the raw data collected from the data source(s), but also "analytics metadata" such as the time period of the collected data, information about the data source(s), etc. If historical data is available at the source NWDAF, this data shall be provided.

-	Information about subscriptions with data sources that are related to the analytics.

******************** 4th Change ********************

### 7.2.2	Nnwdaf_AnalyticsSubscription_Subscribe service operation

Service operation name: Nnwdaf_AnalyticsSubscription_Subscribe.

Description: Subscribes to NWDAF analytics with specific parameters.

Inputs, Required: (Set of) Analytics ID(s) defined in Table 7.1-2, Target of Analytics Reporting, Notification Target Address (+ Notification Correlation ID), Analytics Reporting Parameters, Analytics target period.

NOTE 1:	Target of Analytics Reporting can be provided per individual Analytics ID in a set of Analytics IDs.

Inputs, Optional: Analytics Filter Information, Source NWDAF ID (in the case of analytics subscription transfer), Subscription Correlation ID (in the case of modification of the analytics subscription or analytics subscription transfer), preferred level of accuracy of the analytics, accuracy level per analytics subset, Reporting Thresholds, Maximum number of objects requested (max), Preferred order of results, Maximum number of SUPIs requested (SUPImax), time when analytics information is needed, Analytics Metadata Request, Dataset Statistical Properties, Output strategy and Data time window.

NOTE 2:	Analytics Filter Information, Reporting Thresholds, Maximum number of objects requested (max), Maximum number of SUPIs requested (SUPImax), Analytics Metadata Request, Dataset Statistical Properties, Output strategy and Data time window can be provided per individual Analytics ID in a set of Analytics IDs.

Outputs Required: When the subscription is accepted: Subscription Correlation ID (required for management of this subscription).

Outputs, Optional: None.

***************** 6th Change ********************

7.3.3	Nnwdaf_AnalyticsInfo_Transfer service operation

Service operation name: Nnwdaf_AnalyticsInfo_Transfer.

Description: Requests to NWDAF to transfer information related to analytics subscriptions.

Inputs, Required: (Set of) Analytics context identifier(s), or Subscription Correlation ID.

Inputs, Optional: None.

Outputs Required: (Set of) Analytics context information, or analytics context information associated with the Subscription Correlation ID.

Outputs, Optional: None.

******************** 7th Change ********************

7.2.4	Nnwdaf_AnalyticsSubscription_Notify service operation

Service operation name: Nnwdaf_AnalyticsSubscription_Notify.

Description: NWDAF notifies the consumer instance of the analytics that has subscribed to the specific NWDAF service.

Inputs, Required: Notification Correlation Information: this parameter indicates the Subscription Correlation Id that has been assigned to the consumer during event subscription. If this event is for informing the creation of a new subscription Id at the NWDAF, this IE shall indicate the old Subscription Correlation Id.

Inputs, Optional:

-	Set of the tuple (Analytics ID, Analytics specific parameters): this parameter shall be present if output analytic(s) is reported.

-	Subscription Change Notification Correlation Id: this parameter shall be present if the notification is for informing the creation of a new Subscription Correlation Id at the NWDAF.

-	Timestamp of analytics generation.

-	Validity period.

-	Probability assertion.

-	Revised waiting time.

-	Analytics Metadata Information.

-	Level of accuracy reached.

-	Subscription status (e.g. not able to provide Analytics)

NOTE 1:	Some NWDAF output analytics already include confidence of predictions, which provides the same information as probability assertion.

NOTE 2:	Validity period can also be provided as part of Analytics specific parameters for some NWDAF output analytics.

******************** End of Change ********************