SA WG2 Meeting #145E (e-meeting)	S2-2104281r03

May 17 – 28, 2021, Elbonia

Source:	CATT

Title:	MBS Session state model

Document for:	Approval

Agenda Item:	8.9

Work Item / Release:	5MBS / Rel-17

Abstract of the contribution: This contribution proposes the update to the description of MBS Session state model.

- Introduction

This contribution proposes some updates to the description of MBS Session state model in clause 4.3 of TS 23.247, to remove the EN, etc.

# 2. Proposal

It is proposed to include the following changes in TS 23.247.

******************** 1st Change ********************

## 4.3	Multicast session state model

The following states are defined for the multicast session:

-	Configured state: Information about the multicast session (e.g. QoS information) is configured in 5GC NFs (e.g. MB-SMF) serving the multicast session, but no User Plane resources towards NG-RAN are reserved and no MBS data can be transmitted. A TMGI can be allocated for the multicast session.UEs may be allowed to join (subject to authorization check and configuration), but the first accepted UE join request will trigger the multicast session establishment towards the NG-RAN and the UE.

NOTE 1:	The SMF is not involved in the multicast session while the multicast session is in configured state, but this state affects the MB-SMF.

NOTE 2:	There may be several sub-states in the configured state, e.g. TMGI requested, or information about the multicast session provided.

-	Active state: Multicast session is established and MBS data can be transmitted to the UEs that have joined the multicast session. UEs are allowed to join the multicast session (subject to authorization check). 5GC resources and radio resources for the multicast session are reserved for UEs that joined the multicast session.

-	Inactive state: Multicast session is established but no MBS data are transmitted to the UEs that have joined the multicast session. UEs that joined the multicast session may be in CM CONNECTED or CM IDLE state. UEs are allowed to join the multicast session (subject to authorization check).

The following procedures are defined which result in transition of the multicast session state:

-	Multicast Session Configuration: The AF provides information about the multicast session and optionally request the allocation of a TMGI. Alternatively, there is network-internal configuration of the multicast session. Only resources at MB-SMF, NEF and MB-UPF are reserved and no multicast data are transmitted. The configuration may indicate whether the multicast session may be established in active or inactive state and when an multicast session can become active. The AF may provide configuration in several steps, e.g. to first request TMGI and then provide full information about the multicast session and allow it to be established. Multicast session state transitions from NULL to Configured state.

NOTE 3:	A multicast session can also be configured by the operator via OAM or be established without prior configuration..

-	Multicast Session Establishment: When the join request of the first UE for the multicast session is accepted, the multicast session is established towards the NG-RAN node and the UE. . Multicast session state transitions from NULL or Configured state to either Inactive or Active state,.

-	Multicast Session Activation: Triggered by the 5GC, the radio resources for the multicast session are established and multicast session data starts to be transmitted to the UE. CM IDLE UEs that joined the multicast session are notified. Activation can be triggered by AF request, or data notification from the MB-UPF. Multicast session state transitions from Inactive state to Active state..

-	Multicast Session Deactivation: Triggered by the 5GC, the radio resources for the multicast session are released and multicast session data stops to be transmitted to the UE.. Deactivation can be triggered by AF request, or no reception of multicast data by the MB-UPF. Multicast session state transitions from Active to Inactive state.

-	Multicast Session Release: Triggered by an AF request or the last UE leaving the multicast session, the resources for the multicast session are released in both 5GC nodes and RAN nodes. UEs that are still participating in the multicast session are notified.  Multicast session state transitions from Active or Inactive state to Configured .

-	Multicast Session Deconfiguration: All information about the multicast session is removed from the 5GC, and the TMGI for the multicast session (if allocated during Multicast Session Configuration) is deallocated. The deconfiguration may be triggered by an AF request. Multicast session state transitions from Configured, Active or Inactive state to NULL.

Figure 4.3-1: Multicast session states and state transitions

Figure 4.3-2: Multicast session states and state transitions in MB-SMF (with Configured state)

Figure 4.3-3: Multicast session states and state transitions in MB-SMF (without Configured state)

Figure 4.3-4: Multicast session states and state transitions in NG-RAN

Figure 4.3-5: Multicast session states and state transitions in SMF

******************** End of Change ********************