SA WG2 Meeting #145E (e-meeting)	S2-2104282r01

May 17 – 28, 2021, Elbonia

Source:	CATT

Title:	MB-SMF services

Document for:	Approval

Agenda Item:	8.9

Work Item / Release:	5MBS / Rel-17

Abstract of the contribution: This contribution proposes definitions of services provided by the MB-SMF to support MBS.

- Introduction

The MB-SMF provides services related to multicast and broadcast session management to support MBS. This contribution proposes definitions of those services.

# 2. Proposal

It is proposed to include the following changes in TS 23.247.

******************** 1st Change (all new text)********************

9.X	MB-SMF services

9.x.1	General

Table 9.x.1-1: NF services provided by the MB-SMF

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Nmbsmf_MBSSession | Create | Request/Response | MBSF, NEF, AMF, SMF |
|  | Update | Request/Response | MBSF, NEF, AMF, SMF |
|  | Activate | Request/Response | AMF, SMF |
|  | Deactivate | Request/Response | AMF, SMF |
|  | Release | Request/Response | MBSF, NEF, AMF, SMF |

9.x.2	Nmbsmf_MBSSession Service

9.x.2.1	General

Service description: This service operates on the multicast and broadcast sessions. The following are the key functionalities of this NF service:

-	(between MBSF/NEF and MB-SMF) Configuration/Modification/Activation/Deactivation/Deconfiguration of multicast sessions;

-	(between MBSF/NEF and MB-SMF) Configuration/Modification/Start/Stop/Deconfiguration of broadcast sessions;

-	(between AMF, SMF and MB-SMF) Establishment of multicast session towards the NG-RAN and UE during multicast session join or mobility procedures.

-	(between AMF, SMF and MB-SMF) Release of multicast session towards the NG-RAN and UE during multicast session leave or mobility procedures.

9.x.2.2	Nmbsmf_MBSSession_Create service operation

Service operation name: Nmbsmf_MBSession_Create.

Description: Create a new multicast session or broadcast session during MBS session configuration, or establish the multicast session towards the NG-RAN and UE during multicast session join or mobility procedures.

Input, Required: DNN, Serving Network (PLMN ID, or PLMN ID and NID, see clause 5.18 of TS 23.501 [5]).

Input, Optional: MBSF ID or NEF ID, AMF ID, SMF ID, S-NSSAI, MBS Session ID (source specific multicast address or TMGI), MBS service area, multicast session activation time, Subscription for MBS Session Status Notification, multicast/broadcast QoS flow information, N2 SM information.

Output, Required: Result Indication, and if successful MBS SM Context ID.

Output, Optional: MBS Session ID, Cause, S-NSSAI, MB-UPF tunnel info, multicast/broadcast QoS flow information, N2 SM information.

9.x.2.3	Nmbsmf_MBSSession_Update service operation

Service operation name: Nmbsmf_MBSession_Update.

Description: Update the established multicast session or broadcast session, e.g. QoS update.

Input, Required: MBS Session ID.

Input, Optional: Multicast/broadcast QoS flow information, MBS service area, N2 SM information.

Output, Required: Result Indication.

Output, Optional: Cause.

9.x.2.3	Nmbsmf_MBSSession_Activate service operation

Service operation name: Nmbsmf_MBSession_Activate.

Description: Activate the multicast session.

Input, Required: MBS Session ID.

Input, Optional: Multicast/broadcast QoS flow information, MBS service area, N2 SM information.

Output, Required: Result Indication.

Output, Optional: Cause.

9.x.2.4	Nmbsmf_MBSSession_Deactivate service operation

Service operation name: Nmbsmf_MBSession_Deactivate.

Description: Deactivate the multicast session.

Input, Required: MBS Session ID.

Input, Optional: None.

Output, Required: Result Indication.

Output, Optional: Cause.

9.x.2.5	Nmbsmf_MBSSession_Release service operation

Service operation name: Nmbsmf_MBSession_Release.

Description: Release the multicast session or broadcast session during MBS session deconfiguration, or release of multicast session towards the NG-RAN and UE during multicast session leave or mobility procedures.

Input, Required: MBS Session ID.

Input, Optional: None.

Output, Required: Result Indication.

Output, Optional: Cause.

******************** End of Change ********************