SA WG2 Meeting #145E (e-meeting)	S2-2104282r03

May 17 – 28, 2021, Elbonia

Source:	CATT, Nokia, Nokia Shanghai-Bell

Title:	MB-SMF services

Document for:	Approval

Agenda Item:	8.9

Work Item / Release:	5MBS / Rel-17

Abstract of the contribution: This contribution proposes definitions of services provided by the MB-SMF to support MBS.

- Introduction

The MB-SMF provides services related to multicast and broadcast session management to support MBS. This contribution proposes definitions of those services.

# 2. Proposal

It is proposed to include the following changes in TS 23.247.

******************** 1st Change (all new text)********************

## 9.X	MB-SMF services

### 9.x.1	General

Table 9.x.1-1: NF services provided by the MB-SMF

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Nmbsmf_MBSSession | Create | Request/Response | MBSF, NEF, AF |
|  | Update | Request/Response | MBSF, NEF, AF |
|  | Release | Request/Response | MBSF, NEF, AF |

### 9.x.2	Nmbsmf_MBSSession Service

#### 9.x.2.1	General

Service description: This service operates on the multicast and broadcast sessions. The following are the key functionalities of this NF service:

-	(between MBSF/NEF and MB-SMF) Create/Modification/Activation/Deactivation/Release of multicast sessions;

-	(between MBSF/NEF and MB-SMF) Create/Modification/Start/Stop/Release of broadcast sessions;

Editor's note:		Parameters of the service operations are FFS.

Editor´s note: Subscription for MBS Session Status and related notifications are ffs (and depend on the definition of related events).

#### 9.x.2.2	Nmbsmf_MBSSession_Create service operation

Service operation name: Nmbsmf_MBSession_Create.

Description: Create a new multicast session or broadcast session during MBS session configuration.

Input, Required: MBS Session ID (source specific multicast address or TMGI) or TMGI request

Input, Optional: DNN S-NSSAI, , MBS service area, MBS activation time, MBS termination time, service description, , QoS flow information,, Input Transport Address Request, session activity status (active/inactive).

Output, Required: Result Indication, and if successful MBS Context ID.

Output, Optional: TMGI, Cause, MB-UPF tunnel info.

#### 9.x.2.3	Nmbsmf_MBSSession_Update service operation

Service operation name: Nmbsmf_MBSession_Update.

Description: Update the established multicast session or broadcast session, e.g. QoS update.

Input, Required: MBS control context ID.

Input, Optional: QoS flow information, MBS service area, session activity status (active/inactive).

Output, Required: Result Indication.

Output, Optional: Cause.

#### 9.x.2.5	Nmbsmf_MBSSession_Release service operation

Service operation name: Nmbsmf_MBSession_Release.

Description: Release the multicast session or broadcast session,.

Input, Required: MBS Session ID.

Input, Optional: None.

Output, Required: Result Indication.

Output, Optional: Cause.

******************** End of Change ********************