SA WG2 Meeting #145E (e-meeting) 	S2-2104284r01

May 17 – 28, 2021, Elbonia	(revision of S2-2103074)

Source:	CATT

Title:	Updates to MBS Session Join procedure

Document for:	Approval

Agenda Item:	8.9

Work Item / Release:	5MBS / Rel-17

Abstract: This contribution updates the multicast session join procedure for MBS.

# 1. Introduction/Discussion

In SA2 #144e meeting, S2-2103074 for MBS Session Join and Establishment procedures was technically endorsed. Based on S2-2103074, further clarifications and modifications are proposed to resolve the Editor's notes and other unsettled issues. The updates compared to S2-2103074 are highlighted in yellow color.

2. Text Proposal

It is proposed to include the following changes in TS 23.247.

# * * * * First change * * * *

## 7.2	MBS procedures for multicast Session

### 7.2.1	MBS join and Session establishment procedure

#### 7.2.1.1	General

Session Join procedure is used by UEs to inform the 5GC of the UE interest in an MBS Session. The user plane management is described in clause 6.6.

#### 7.2.1.2	Establishment of a PDU Session that can be associated with multicast session(s)

A PDU Session associated with multicast session(s) is established using the procedures as specified in TS 23.502 [6] clause 4.3.2.2 with the following differences:

NOTE:	The DNN and S-NSSAI are used to establish the PDU session which can carry the Multicast operation, i.e. join/leave, and can be associated with multicast MBS session(s).

-	In step 2, based on the DNN and S-NSSAI use for establishing a PDU Session associated with multicast session(s), the AMF selects an SMF capable of handling multicast sessions based on DNN and S-NSSAI, or locally configured data or a corresponding SMF capability stored in the NRF. For indirect discovery, the AMF requests the SCP to select an SMF capable of handling multicast sessions.

#### 7.2.1.3	Multicast session join procedure

The following steps are executed before the UE requests to join the MBS session:

- The MBS Session has been configured. Details see clause 7.1.1. MBS Session configuration for multicast session is optional.

- The UE registers in the PLMN and establishes a PDU session.

- The UE has known at least the MBS Session ID of a multicast group that the UE can join, e.g. via service announcement or NAS signaling.

UE

NG-RAN

AMF

SMF

UPF

NRF

MB-SMF

MB-UPF

AF

1a. UL NAS message (N1 SM container (PDU Session Modification Request))

1b. Nsmf_PDUSession_UpdateSMContext request

5. Nsmf_PDUSession_UpdateSMContext response

3. Nnrf_NFDiscovery request/response

4. Nmbsmf_MBSSession_Create request/response

6. N2 message request

13. Multicast data

14. Multicast data

15. Bear selection

16. Multicast data via PTP or PTM

Transmission via 5GC shared MBS traffic delivery

17. Multicast data

18. Multicast data via PDU Session

19. Multicast data via PDU Session

Transmission via 5GC Individual MBS traffic delivery

2. Authorization check, see clause 6.1.1

Establishment of resources for 5GC shared MBS traffic delivery

7a. N2 message

7b. Nmbsmf_MBSSession_Create request

7c. Configuration for Shared delivery

7d. Nmbsmf_MBSSession_Create response

7e. N2 message

9. N2 message response

10. Nsmf_PDUSession_UpdateSMContext request

Establishment of resources for 5GC Individual MBS traffic delivery

11a. Configuration for Individual delivery

11b. Nmbsmf_MBSSession_Update request

11c. Configuration for Individual delivery

11d. Nmbsmf_MBSSession_Update response

11e. Configuration for Individual delivery

8. RRC message (PDU Session Modification command)

12. Nsmf_PDUSession_UpdateSMContext response

7f. N2 message

7h. Configuration for Shared delivery

7g. Nmbsmf_MBSSession_Update

Figure 7.2.1.3-1: PDU Session modification for UE joining multicast session

1.	To join the multicast group, the UE sends a PDU Session Modification Request which contains an MBS Session ID). The MBS Session ID indicates the multicast group that UE wants to join.

NOTE 1:	The UE may request to join multiple multicast groups by including multiple MBS Session ID(s), each indicates one MBS Session for the multicast group, in the PDU Session Modification Request. In this case, the following steps are performed for each MBS Session.

NOTE 2:	Before sending the PDU Session Modification Request, UE may already know 5GC supports MBS. It is up to RAN specification whether the UE knows if the NG-RAN supports MBS or not.

2.	Based on the received MBS Session ID, the SMF determines this is MBS Session join request. The SMF authorizes MBS Session join request, see clause 6.1.1. If the Multicast MBS session is configured but the configuration indicates that the data transmission has not started yet, the SMF may reject the MBS session join with appropriate cause value. If authorization check fails, the SMF indicates cause value in the PDU Session Modification Reject sent to the UE and goes to step 5.

Editor's note:	It is ffs if database interactions are required for the authorization check.

3.	If SMF has no MBS Session context information for the indicated MBS Session ID, SMF discovers and selects an MB-SMF for the MBS Session via the NRF as described in clause 7.1.2. If SMF has the MBS Session context information for the indicated MBS Session ID, the SMF selects the MB-SMF in the MBS Session context information, which can be the SMF itself.

NOTE x1:		SMF and MB-SMF can be identical, i.e. MB-SMF can be an SMF enhanced with MBS functionality.

Editor's note:	Whether SMF acting as the MB-SMF is needed and how it works if needed is FFS.

Editor's note:	More consideration on how to prevent denial of service attack type situation when first UE joining the multicast group, triggers the MB-UPF to join the multicast tree towards the content provider is FFS.

4.	By using Nmbsmf_MBSSession_Create or Nmbsmf_MBSSession_Update request (MBS Session ID), SMF interacts with the MB-SMF to retrieve multicast QoS flow information of the indicated MBS session. For multicast transport between MB-UPF and content provider, if it is the first UE joining the multicast group and MB-UPF did not join the multicast tree in the MBS configuration procedure, the MB-SMF requests the MB-UPF to join the multicast tree towards the content provider, otherwise MB-SMF will not send the request to the MB-UPF.

5.	SMF responds to AMF through Nsmf_PDUSession_UpdateSMContext response(MBS Session ID, MB-SMF ID, N2 SM information (PDU Session ID, MBS Session ID, [updated PDU Session information], mapping information between unicast QoS flow(s) and multicast QoS flow(s)), N1 SM container (PDU Session Modification Command)) to:

-	create an MBS session context for the indicated MBS session in the RAN, if it does not exist already; and

-	inform about the relation including the mapping information between the multicast context and the UE's PDU session context to RAN.

Based on operator policy, the SMF may prepare for 5GC individual MBS traffic delivery fall-back. The SMF maps the received QoS information of the multicast QoS Flow into PDU Session's QoS Flow information, and includes the information of the QoS Flows and the mapping information about the QoS Flows in the SM information sent to RAN.

Editor's note:	Details information included in N2 SM information will be aligned with RAN WG3.

Editor's note:	Whether it needs to inform about the relation including the mapping information between the multicast context and the UE's PDU session to UE is FFS.

6.	The N2 message, which includes the multicast session information and PDU session modification information is sent to the NG-RAN.

If the MBS is not supported by NG-RAN, 5GC individual MBS traffic delivery may be used. Otherwise, 5GC shared MBS traffic delivery is adopted.

If the NG-RAN supports MBS, the NG-RAN uses the MBS Session ID to determine that the PDU Session Modification procedures corresponds to the indicated multicast session.

NOTE x2:	It is NG-RAN that decides whether radio resource is allocated or not.

[Conditional] If shared tunnel has not been established for the MBS session, step 7 is used for establishing 5GC shared MBS traffic delivery:

7a.	The NG-RAN sends an N2 MBS Session request message (MBS Session ID, N2 SM information (MBS Session ID, [AN Tunnel Info])) towards AMF. MBS Session ID is included.

If the NG-RAN node uses a unicast transport for shared delivery, it allocates a downlink tunnel ID for the reception of MBS data and includes the downlink tunnel information as AN Tunnel Info in the request.

7b.	AMF invokes Nmbsmf_MBSSession_Create request (MBS Session ID, [AN Tunnel Info]) towards the MB-SMF indicated in step 5.

7c.	If a downlink tunnel information is included in AN Tunnel Info of the Nmbsmf_MBSSession_Create request, MB-SMF configures MB-UPF to transmit the MBS data for multicast session towards NG-RAN using the downlink tunnel information.

If AN Tunnel Info is not included in the request, the MB-SMF or the MB-UPF allocates a common DL tunnel ID and optionally an Low Layer source specific multicast address (LL SSM) for multicast transport between the MB-UPF and NG-RAN.

7d.	MB-SMF responds to AMF through Nmbsmf_MBSSession_Create response (multicast QoS flow information, [common DL tunnel ID], [LL SSM]).

7e.	AMF sends an N2 MBS Session response (MBS Session ID, N2 SM information (MBS Session ID, multicast QoS flow information, [common DL tunnel ID], [LL SSM])) to the NG-RAN node.

7f.	The NG-RAN sends an N2 MBS Session acknowledgement (MBS Session ID, N2 SM information(MBS Session ID, accepted/rejected multicast QoS flow information)) to the AMF.

7g.	AMF sends Nmbsmf_MBSSession_Update request (MBS Session ID, accepted/rejected multicast QoS flow information) towards the MB-SMF.

7h.	MB-SMF provides the accepted/rejected multicast QoS flow information by the NG-RAN to the MB-UPF.

8.	The NG-RAN performs AN specific signalling exchange with the UE to establish radio resource for the MBS session if not established yet. If the NG-RAN does not support MBS, radio resource are reconfigured for unicast transmission of the MBS data over the associated PDU session. As part of the AN specific signalling exchange, the N1 SM container (PDU Session Modification Command) is provided to the UE.

9.	The NG-RAN sends the PDU session modification response.

If the MBS is not supported by NG-RAN, the accepted unicast QoS flow is included in the N2 SM response container.

10.	The AMF invokes Nsmf_PDUSession_UpdateSMContext request to the SMF.

Per the accepted unicast QoS flow information, the SMF determines that 5GC individual MBS traffic delivery is used for multicast packet transferring.

NOTE x3:	If the shared tunnel is used, the interaction with UPF is not needed for the indicated MBS session

[Conditional] Step 11 is used for 5GC Individual MBS traffic delivery, e.g. the related NG-RAN does not support multicast.

If the shared tunnel between the UPF(PSA) and MB-UPF for individual delivery have not been established, step 11a to 11e are executed.

11a.	If unicast transport for the multicast data between UPF and MB-UPF is to be used, SMF allocates a downlink tunnel endpoint and configures UPF. Or, SMF requests UPF to allocate a downlink tunnel ID.

11b.	SMF invokes Nmbsmf_MBSSession_Update request (MBS Session ID, DL tunnel info) towards MB-SMF that includes MBS Session ID and downlink tunnel info of UPF, for establishing the multicast session distribution between MB-UPF and UPF.

11c.	MB-SMF configures MB-UPF to transmit the multicast distribution session towards UPF using the received downlink tunnel ID.

11d.	MB-SMF responds to SMF through Nmbsmf_MBSSession_Update response. For multicast transport between MB-UPF and UPF, it also indicates in the downlink tunnel information the transport multicast address for the multicast session.

11e.	For multicast transport between MB-UPF and UPF, SMF configures UPF to receive the multicast distribution session and forwards the data within unicast transport.

12. The SMF invokes Nsmf_PDUSession_UpdateSMContext response to the AMF.

13. MB-UPF receives multicast PDUs, either directly from the content provider or via the MBSTF that can manipulate the data.

Step 14 to 16 are for 5GC shared MBS traffic delivery:

14. MB-UPF sends multicast PDUs in the N3mb tunnel associated to the multicast distribution session to the RAN. There is only one tunnel per multicast distribution session and NG-RAN node, i.e., all associated PDU sessions share this tunnel.

15. The NG-RAN selects PTM or PTP radio bearers to deliver the multicast PDUs to UEs that joined the multicast group.

16. The NG-RAN performs the transmission using the selected radio bearer.

Step 17 to 19 are for 5GC individual MBS traffic delivery:

17. MB-UPF sends multicast PDUs in the N19mb tunnel associated to the multicast distribution session to UPF. There is only one tunnel per multicast distribution session and destination UPF, i.e., all associated PDU sessions share this tunnel.

18. UPF forwards the multicast data via unicast.

19. The NG-RAN forwards the multicast data via unicast.

NOTE x4:	Details of the DL MBS data transmission could refer to clause 6.7.

NOTE x5:	When the MBSF is involved in the multicast MBS session, the tunnel between MBSTF and MB-UPF has been established in the configuration procedure.

# * * * * End of changes * * * *