SA WG2 Meeting #S2-145E  	S2-2104292

# 17 – 28 May, 2021, Elbonia

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 2915 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates initialization of PTP instance |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | IIoT |  |  |  |  |  | Date: |  |  | 2021-05-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Event Initialize is part of PTP management, which is performed via user plane (i.e., included in PTP header using a TLV), which is used rarely. The management used in 5GS as per Release 17 is based on management based on YANG or NETCONF (i.e., provided via control plane, AF). delete related text not aligned with CP approach. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify the initialization of PTP instance in DS-TT/NW-TT. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | . Incorrect specification with regards to initialization. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | K.2.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * 1st change * * * *

### K.2.2.1	General

Based on input received from external applications (CNC in case of TSN AF or any AF in case of NEF), TSN AF and NEF may configure PTP instances (identified by PTP Instance ID) in a DS-TT or NW-TT by sending PTP port management information (see Table 5.28.3.1-1) to DS-TT and NW-TT, respectively. TSN AF or NEF may also configure PTP instances for DS-TT ports in NW-TT by sending Bridge Management information (see Table 5.28.3.1-2) to NW-TT to enable NW-TT to operate as a grandmaster on behalf of DS-TT (see clause K.2.2.4 for more details).

For each PTP instance TSN AF or NEF may provide individual PTP configuration parameters or may provide a PTP profile ID to DS-TT or NW-TT.

NOTE:	Even if PTP profiles are used to configure DS-TT or NW-TT, individual PTP parameters can still be configured in addition, e.g. domain numbers, transport to use, etc.

To configure DS-TT and NW-TT to operate as a PTP relay instance, TSN AF or NEF shall set the PTP profile (see Table 5.28.3.1-1) to IEEE  802.1AS [104].

To initialize a PTP instance in 5GS, TSN AF or NEF retrieves the "defaultDS.clockIdentity" of the PTP instance in NW-TT via BMIC. Upon detection of a DS-TT port, the TSN AF or NEF initializes the PTP instance in the DS-TT by setting the "defaultDS.clockIdentity" via PMIC to the same value as retrieved from the NW-TT. The TSN AF or NEF can initialize any number of PTP instances up to the maximum number of supported PTP instances by the NW-TT and DS-TT(s), whichever is the lowest.

# * * * * End of 1st change * * * *