3GPP TSG-WG SA2 Meeting #145E e-meeting 	S2-2104300r01

Elbonia, May 17 – 28, 2021

Source:	Nokia, Nokia Shanghai-Bell

Title:	5MBS Information in UDM

Document for:	Approval

Agenda Item:	8.9

Work Item / Release:	5MBS / Rel-17

Abstract: This contribution defines 5MBS information stored in the UDM, following proposals in S2-2103840

- Introduction

See proposals in S2-2103840.

2. Text Proposal

It is proposed to agree the following changes for TR 23.757.

## 6.4	Subscription to multicast services

6.4.1.	General

The user's profile in the UDM contains the subscription information to give the user permission to use multicast services.

At any time, the operator may change the subscription for multicast services in user's profile in the UDM.

The user may have following subscription for multicast services:

-	Allowed to join any multicast service with authorization for the user in the MBS session subscription data

The MBS session subscription data contain information whether any UE that is entitled to use multicast may join multicast session, or indicate groups of UEs or UE lists entitled to join the multicast service.

The MBS session subscription data is provided by the UDM to the SMF during PDU session establishment as defined in clause 4.3.2.2.1 of TS 23.502 [6] using Nudm_SDM service for Subscription data type "UE context in SMF data". The SMF can also use the Nudm_SDM service to retrieve MBS session subscription data information.

The SMF selection subscription data is provided by the UDM to the AMF as described in clause 4.2.2.2 of TS 23.502 [6].

## 6.4.2	MBS information in UDM

The information stored in the UDM described in clause 4.3.2.2.1 of TS 23.502 [6] is extended as follows:

- The UE Subscription data types are extended with MBS Subscription data, as defined in Table 6.4.2-1

- MBS session subscription data as defined in Tables 6.4.2-2 and 6.4.2-3 with keys according to Table 6.4.2-4 are introduced

Table 6.4.2-1: MBS subscription data type

| Subscription data type | Field | Description |
| --- | --- | --- |
| MBS subscription data | Multicast allowed | Allowed to join any multicast service with authorization for the user in the MBS information. |

Table 6.4.2-2: MBS session subscription data type

| data type | Field | Description |
| --- | --- | --- |
| MBS session subscription data | MBS Session ID | MBS Session ID as defined in Clause 6.5.1 |
|  | Internal Group Identifier (NOTE 1) | Identifies internal group of UEs that are entitled to join a multicast session |
|  | SUPI list (NOTE 1) | Identifies UEs that are entitled to join a multicast session |
|  | Any UE indication (NOTE 1) | Indicates that any UE that is entitled to use multicast may join multicast session. |
|  | TMGI (conditional) | Shall be included once allocated if Source Specific IP Multicast Address is used as MBS Session ID |
|  | Location area (NOTE 2) |  |
|  | List of Location dependent service information (NOTE 2) | Location dependent service information entries are defined in Table 6.X-3 |
| NOTE 1: For multicast groups, one of "Internal Group Identifier", "SUPI list", or "Any UE indication" shall be present. NOTE 2: For multicast groups, either location area or location dependent service information may be provided or both parameters may be omitted. For broadcast groups, either location area or location dependent service information shall be provided. |  |  |

Table 6.4.2-4: MBS session subscription data type keys

| Subscription Data Types | Data Key | Data Sub Key |
| --- | --- | --- |
| MBS session subscription data | MBS session ID | - |