3GPP TSG-SA WG2 Meeting #145E	S2-2104331r01

# Elbonia, May 17 - May 28 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 2918 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | KI#2 T1: Informative guideline for QoS Notification between overlay network and underlay network |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNPN |  |  |  |  |  | Date: |  |  | 2021-05-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | TR 23.700-07 has concluded when using N3IWF based solution, overlay network and underlay network can use existing NEF notification procedures for QoS related information: When N3IWF based solution is used, the overlay network and its service AF can use existing NEF notification procedures, such as of subscribing the "QoS monitoring" or "QoS sustainability" via the interface between NEF and AF, to subscribe and receive the notification from underlay network regarding the connectivity QoS status or QoS mapping changes that are associated with the IPsec of the overlay network. With the QoS update information from the underlay network, the overlay network can adjust its connectivity QoS accordingly. The opposite way is also applicable that the underlay network and its service AF can use existing NEF notification procedures, such as of subscribing the "QoS monitoring" or "QoS sustainability" via the interface between NEF and AF, to subscribe and receive the notification from overlay network regarding the connectivity QoS status or QoS mapping changes that are associated with the IPsec of the overlay network. With the QoS update information from the underlay network, the underlay network can adjust its connectivity QoS accordingly. No need of new information for QoS notification between PLMN and SNPN has been identified in this study. Informative guideline on how to use existing QoS notification mechanism between SNPN and PLMN can be provided during the normative phase as informative annex. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Informative guideline on how to use existing NEF notification procedures to support QoS notification between overlay network and underlay network. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Lacking information regarding how to support QoS notification between overlay network and underlay network. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Annex D.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

# D.3	Support for access to PLMN services via Stand-alone Non-Public Network and access to Stand-alone Non Public Network services via PLMN

Figure D.3-1: Access to PLMN services via Stand-alone Non-Public Network

NOTE 1:	The reference architecture in Figure D.3-1 and Figure D.3-2 only shows the network functions directly connected to the UPF or N3IWF and other parts of the architecture are same as defined in clause 4.2.

In order to obtain access to PLMN services when the UE is camping in NG-RAN of Stand-alone Non-Public Network, the UE obtains IP connectivity, discovers and establishes connectivity to an N3IWF in the PLMN.

In the Figure D.3-1, the N1 (for NPN) represents the reference point between UE and the AMF in Stand-alone Non-Public Network. The NWu (for PLMN) represents the reference point between the UE and the N3IWF in the PLMN for establishing secure tunnel between UE and the N3IWF over the Stand-alone Non-Public Network. N1 (for PLMN) represents the reference point between UE and the AMF in PLMN.

Figure D.3-2: Access to Stand-alone Non-Public Network services via PLMN

In order to obtain access to Non-Public Network services when the UE is camping in NG-RAN of a PLMN, the UE obtains IP connectivity, discovers and establishes connectivity to an N3IWF in the Stand-alone Non-Public Network.

In Figure D.3-2, the N1 (for NPN) represents the reference point between UE and the AMF in the Stand-alone Non-Public Network. The NWu (for NPN) represents the reference point between the UE and the N3IWF in the stand-alone Non-Public Network for establishing a secure tunnel between UE and the N3IWF over the PLMN. The N1 (for PLMN) represents the reference point between UE and the AMF in PLMN.

When using the mechanism described above to access overlay network via underlay network, AF in overlay network can subscribe QoS Sustainability Analytics with NEF in underlay network, as described in TS 23.288[86] clause 6.1.1.2. Then, NEF in underlay network can subscribe QoS Sustainability Analytics with NWDAF in underlay network as described in TS 23.288[86] clause 6.9.4.

NOTE:	The QoS Sustainability Analytics can be based on 5QI which belongs to QoS Flow in underlay network in order to support the corresponding User Plane IPsec Child SA of the overlay network, according to the SLA as described in Annex D.Y. Location information required by the QoS Sustainability Analytics is based on implementation specific solutions.

Once the threshold is reached, the AF in overlay network receives the notification from the NEF in underlay network. Then AF in overlay network can interact with overlay network PCF to lower the service requirement.

Also subject to the agreement, the opposite way which AF in the underlay network interacts with NEF in the overlay network to subscribe QoS Sustainability Analytics is also applicable.

END CHANGE