3GPP TSG-WG SA2 Meeting #145E e-meeting  	S2-2104382

Elbonia, May 17 – 28, 2021	(revision of S2-21xxxxx)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 2812 | rev | - | Current version: | 16.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | L-PSA release during mobiity procedure with I-SMF removal |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon, Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17, ETSUN |  |  |  |  |  | Date: |  |  | 2021-05-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The L-PSA can be controlled by the I-SMF. Due the the mobility the I-SMF may be removed or changed. However it has not been specified how to remove those L-PSA. It is clarify that if the I-SMF is removed, the I-SMF needs wait the N4 information from SMF for release of L-PSA. |  |  |  |  |  |  |  |  |
| Summary of change: |  | When the L-PSA controlled by I-SMF is inserted before and the I-SMF is to be removed, the SMF provides N4 information to indicate L-PSA removal. Per the received N4 information the I-SMF release the L-PSA. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unclear on whether the L-PSA is removed or kept when the I-SMF is to be removed. Also whether the SMF need send the N4 information to I-SMF is unclear. These will cause the IOT problem. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.23.9.0 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * 1st change * * * *

### 4.23.9	Branching Point or UL CL controlled by I-SMF

#### 4.23.9.0	Overview

The procedures in this clause describe the Addition, Removal and Change of PDU Session Anchor (PSA2), Branching Point or UL CL controlled by I-SMF. They all rely on following principles:

1.	When a (new) I-SMF is inserted (e.g. as described in clause 4.23.7 or clause 4.23.11), the I-SMF provides the DNAI list it supports to the SMF. This list is assumed to remain constant during the N16a association between the I-SMF and the SMF for a PDU Session.

2.	Based on the DNAI list information received from I-SMF, the SMF may then at any time provide or update the list of DNAI(s) of interest for this PDU Session to I-SMF. This may take place e.g. when the I-SMF provides the DNAI list it supports or when new or updated or removed PCC rule(s) is/are received by the SMF as defined in clause 4.23.6. This list of DNAI(s) of interest for this PDU Session indicates to the I-SMF the list of DNAI(s) candidate for local traffic steering within the PDU Session.

An indication of whether Multi-homing is possible is also provided to the I-SMF, and the I-SMF uses this information to decide whether multi-homing can be used for the PDU Session.

3.	Whenever the I-SMF has inserted or removed or changed a local offload capability the I-SMF invokes a Nsmf_PDUSession_Update Request to indicate to the SMF the list of corresponding DNAI(s). Based on this indication the SMF invokes a Nsmf_PDUSession_Update Request to send the corresponding N4 information to the I-SMF.

4.	Then SMF may then at any time invoke a Nsmf_PDUSession_Update Request to send N4 information to the I-SMF.

5.	The I-SMF may at any time send a Nsmf_PDUSession_Update Request to forward to the SMF N4 events received from a local UPF; this may e.g. correspond to traffic reporting.

6.   Even when an I-SMF is to be removed from a PDU Session (e.g. at I-SMF change or removal) the SMF is responsible of issuing the corresponding N4 information  targeting the UL CL (s) and L-PSA(s) controlled by this I-SMF, including requests to release the corresponding N4 Sessions. The SMF remains responsible of any N9 session between UL CL(s) controlled by the source I-SMF and a target I-SMF  or the SMF itself (I-SMF removal).

When it receives a Nsmf_PDUSession_ReleaseSMContext Request from AMF, the Source I-SMF initiates a timer before releasing the resources of the PDU Session. it releases such resources either when it has received N4 release from SMF for all local UPF(s) or at the expiry of this timer

NOTE: 	As explained in Figure 4.3.5.7-1, even though due to UE mobility a target UL CL / L-PSA and a target I-SMF are used for a PDU Session,  the SMF may be configured (via Nnef_Traffic_Influence) to keep a N9 data forwarding tunnel between source and target UL CL  as long as the end of active traffic over this N9 forwarding tunnel has not been detected. In this case, the source I-SMF needs to remain involved for a PDU Session even though it has received Nsmf_PDUSession_ReleaseSMContext Request from AMF.

When an I-SMF is actually removed from a PDU Session (receiving a N16a release from SMF) this I-SMF ensures proper release of the N4 sessions associated with the PDU Session  if these N4 sessions had not been already released (by the SMF ).

When IPv6 multi-homing is used and an I-SMF is removed, the SMF re-configure the UE for the original IP prefix @L-PSA(s).

# * * * * End of change * * * *